3GPP TSG-RAN WG4 Meeting # 112bis	R4-2415806

Hefei, Anhui, China, 14th – 18th October, 2024

Source: 	vivo

Title: 	Co-existence evaluation of NTN HPUE

Agenda Item:	6.8.2.1

Document for:	Approval

- Introduction

In the last meeting, the co-existence simulation assumptions for NTN HPUE were approved in [1], and in this contribution we provide our preliminary evaluation results and analysis.

- Discussion

  - General

Agreement:

- Consider isolation distance in scenario 4 and 5 for both NR-NTN HPUE coex and IoT-NTN HPUE coex.

  - Use the isolation distance 1.5km as the starting point

  - Other values for isolation distance are not precluded

- Consider both Scenario 4 and 5 for co-existence study

  - Prioritize scenario 4 since it is the worst case

- Do not consider Scenario 1, 2, 3 and 6

In [1], The following agreements are captured:

In previous PC3 NTN co-existence study, the isolation distance is not assumed for scenario 4. For comparison purpose, the results of PC3 are also provided in following part.

  - NR NTN

For NR NTN co-exist with NR TN, the simulation results are shown in Table I

Table I Scenario 4 co-existence evaluation results for NR GEO NTN and TN Uma with AAS antenna

|  | Average Throughput Loss (%) |  |  |  |  | 5% cell-edge Throughput Loss (%) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ACLR | 20 | 25 | 30 | 35 | 40 | 20 | 25 | 30 | 35 | 40 |
| PC3 | 0.35 | 0.18 | 0.10 | 0.06 | 0.05 | 1.49 | 0.62 | 0.16 | 0.11 | 0.09 |
| PC2 | 0.52 | 0.28 | 0.18 | 0.10 | 0.08 | 2.07 | 1.25 | 0.59 | 0.22 | 0.16 |
| PC1.5 | 0.76 | 0.42 | 0.24 | 0.16 | 0.13 | 3.62 | 1.76 | 1.04 | 0.57 | 0.46 |
| PC1 | 0.97 | 0.62 | 0.32 | 0.22 | 0.18 | 5.21 | 2.92 | 1.41 | 0.94 | 0.62 |

With the assumption of 1.5km isolation distance, the interference from TN is significantly reduced. In previous study, around 28 dB ACLR is required but the results above shows that even for PC1, 22 dB ACLR is already enough to reach 5% throughput loss.

Observation 1: With the isolation distance assumption, the required ACLR is less than 20 dB for PC2/PC1.5, and about 22 dB for PC1.

For PC3, 30 dB ACLR is defined which actually reuse the same value of PC3 TN. Based on the results above, it is possible to reuse same ACLR for PC2/PC1.5/PC1.

Proposal 1: For PC2/PC1.5/PC1 NR NTN HPUE, it is suggested to reuse the same ACLR of PC3, i.e., 30 dB.

  - IoT NTN

For NR-NTN:

ACLR modelling for TN and NTN co-existence study referring to clause 5.1.1.4.1 and 5.1.1.4.2 in TR 36.942[8] is used as baseline. The number of RBs refers to Table 2.2.2.1-4 and Table 2.2.2.4-1 respectively.

For IoT-NTN:

For Case 1: Flat model with adjusting the TN ACLR according to TR 36.942, i.e. adjusting the TN ACLR with FACLR = 10 × LOG10(Bvictim/BAggressor), and assuming flat ACS. FACLR = ~20.5 dB with 1.5km isolation distance.

Case 4: Symmetric bandwidth NTN ACLR model according to TR 36.942 section 5.1.1.4.2 with Flat NTN ACLR model and FACLR = 0 dB, assuming flat ACS.

For IoT NTN, a clearly different assumption compared to NR NTN is the ACIR model:

In our understanding, the symmetric bandwidth model means that only the UE who adjacent with NR will bring interference on the NR, and the interference of all other UEs are ignored. Taking 15KHz case as an example:

Figure 1 Symmetric bandwidth ACIR model

Since only one UE provides the interference, the results for different activated UE number should be similar. For cases with smaller bandwidth, e.g., 3.75KHz, they are easier reach 15dB SINR at BS side with lower transmit power, so it is possible for such case the interference may be reduced. The corresponding simulation results are shown in Table II and Table III

Table II Scenario 4 co-existence evaluation results for 3.75KHz IoT GEO NTN

and TN Uma with AAS antenna

| 36x 3.75kHZ | Average Throughput Loss (%) |  |  |  |  | Cell-edge Throughput Loss (%) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ACLR | 20 | 25 | 30 | 35 | 40 | 20 | 25 | 30 | 35 | 40 |
| PC3 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| PC2 | 0.01 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |
| PC1 | 0.02 | 0.01 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 | 0.00 |

Table III Scenario 4 co-existence evaluation results for 15KHz IoT GEO NTN

and TN Uma with AAS antenna

| 9x 15kHZ | Average Throughput Loss (%) |  |  |  |  | Cell-edge Throughput Loss (%) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ACLR | 20 | 25 | 30 | 35 | 40 | 20 | 25 | 30 | 35 | 40 |
| PC3 | 0.01 | 0.01 | 0.00 | 0.00 | 0.00 | 0.01 | 0.0 | 0.0 | 0.0 | 0.0 |
| PC2 | 0.03 | 0.12 | 0.00 | 0.00 | 0.00 | 0.03 | 0.01 | 0.0 | 0.0 | 0.0 |
| PC1 | 0.07 | 0.23 | 0.01 | 0.00 | 0.00 | 0.11 | 0.03 | 0.0 | 0.0 | 0.0 |

The results shows that the interference from IoT NTN is quite low and PC2/PC1 can reuse same ACLR as PC3.

Observation 2: With the isolation distance assumption, the interference from IoT NTN for PC2/PC1 is at a quite low level.

Proposal 2: For PC2/PC1 IoT NTN HPUE, it is suggested to reuse the same ACLR of PC3, i.e., 37 dB.

- Conclusion

In this contribution, we provide our view on NTN HPUE.

Observation 1: With the isolation distance assumption, the required ACLR is less than 20 dB for PC2/PC1.5, and about 22 dB for PC1.

Proposal 1: For PC2/PC1.5/PC1 NR NTN HPUE, it is suggested to reuse the same ACLR of PC3, i.e., 30 dB.

Observation 2: With the isolation distance assumption, the interference from IoT NTN for PC2/PC1 is at a quite low level.

Proposal 2: For PC2/PC1 IoT NTN HPUE, it is suggested to reuse the same ACLR of PC3, i.e., 37 dB.

- References

- R4-2414452	WF on UE RF requirements for NTN HPUE, RAN4#112bis, Samsung