# 3GPP TSG- RAN WG4 Meeting # 112bis	R4-2415817

# Hefei, China, 14th–18th Oct, 2024

# Source: 	ZTE Corporation, Sanechips

# Title: 	Discussion on MPR applicability for FR2

# Agenda item:	6.1.1.2.3

# Document for:	Approval

# 1 Introduction

In RAN#102 meeting, a new Work Item [1] was approved on UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, one of the objectives is about power boosting and/or MPR reduction as follows. In this contribution, we want to share some further views on MPR applicability for FR2.

- Specify power domain enhancement, e.g., MPR reduction for NR single carrier and NR intra-band UL CA - Study the scenarios, and if feasible, specify the power domain enhancement, e.g., MPR reduction, for PC2 and PC3 with applicable ACLR/SEM/spurious emission modification with BS indication for NR FR1 on a single UL carrier - Include the following scenarios: - when there is no adjacent in-band/out-of-band co-existence issue - when a UE uses a narrower channel bandwidth within a wider BS bandwidth - Include both (e)RedCap UE (only PC3) and non-RedCap UE - Limited to QSPK and 16QAM - Specify MPR applicability based on the UL CCs with activated cells for NR intra-band UL CA configuration - Include both intra-band UL contiguous CA and intra-band non-contiguous UL CA for FR1 - Include intra-band UL contiguous CA and intra-band DL contiguous CA with single UL for FR2 - MPR requirement is not applicable until the SCell is activated - Necessary signaling to support the above objectives

# 2 Discussion

- Issue 3-2-1: Applicable MPR for FR2 single carrier UL with DL intra band CA - Proposals - Proposal 1: For the case of DL intra band CA with single UL CC (activated or configured), the reference MPR (originally being Table 6.2A.2.4-1) is changed as the MPR requirement for single carrier. (Huawei) - WF - FFS in next meeting.

For FR2 single carrier UL with DL intra band CA, there is a remaining issue needs further discussion whether to take the minimum values of MPR between the intra-band CA case and the single carrier. For example, assuming a FR2-2 band combination with 400MHz single UL CC and 800MHz intra-band DL contiguous CA, MPR of 400MHz single UL CC are larger than that of 1200MHz CABW for Pi/2 BPSK, QSPK and 16QAM. According to the definition of OOBE requirements for CA configuration consisting of a single UL CC, the OOBE requirement for single CC can be applied. Hence, with the intention of MPR reduction, it is feasible to take the minimum values of MPR between the intra-band CA case and the single carrier for FR2-2 with emissions compliance. We propose that for FR2 single carrier UL with DL intra band CA, applicable MPR shall be derived as MIN(MPR1,MPR2), where: MPR1 is based on single UL CC, and MPR2 is based on CABW.

Table 6.2.2.1-4 MPRWT for power class 1, BWchannel >= 400 MHz in FR2-2

| Modulation |  | MPRWT (dB), BWchannel = 400, 800, 1600, 2000 MHz |  |  |
| --- | --- | --- | --- | --- |
|  |  | Outer RB allocations | Inner RB allocations |  |
|  |  |  | Region 1 | Region 2 |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 6.0 | ≤ 1.0 | ≤ 3.5 |
|  | QPSK | ≤ 6.0 | ≤ 1.0 | ≤ 4.0 |
|  | 16 QAM | ≤ 4.5 | ≤ 3.0 | ≤ 3.0 |
|  | 64 QAM | ≤ 8.0 | ≤ 8.0 | ≤ 8.0 |
| CP-OFDM | QPSK | ≤ 6.0 | ≤ 1.5 | ≤ 3.5 |
|  | 16 QAM | ≤ 6.0 | ≤ 4.0 | ≤ 5.5 |
|  | 64 QAM | ≤ 10.0 | ≤ 10.0 | ≤ 10.0 |

Table 6.2A.2.2-2: Maximum power reduction (MPRWT_C_CA) for UE power class 1 in FR2-2

| Waveform Type | Cumulative aggregated channel bandwidth |  |  |  |
| --- | --- | --- | --- | --- |
|  | < 400 MHz | ≥ 400 MHz and < 800 MHz | ≥ 800 MHz and ≤ 1400 MHz | > 1400 MHz and ≤ 2000 MHz |
| Pi/2 BPSK | ≤ 7.0 | ≤ 5.0 | ≤ 2.0 | ≤ 2.0 |
| QPSK | ≤ 8.0 | ≤ 6.0 | ≤ 3.0 | ≤ 3.0 |
| 16 QAM | ≤ 8.0 | ≤ 6.0 | ≤ 4.0 | ≤ 4.0 |
| 64 QAM | ≤ 10.0 | ≤ 10.0 | ≤ 10.0 | ≤ 10.0 |

In case the CA configuration consists of a single UL CC, spectrum emission mask defined in subclause 6.5.2.1 applies.

In case the CA configuration consists of a single UL CC, the adjacent channel leakage ratio defined in subclause 6.5.2.3 applies.

For intra-band CA, in case the CA configuration consists of a single UL CC, spurious emissions requirements defined in subclause 6.5.3 apply.

Proposal 1: For FR2 single carrier UL with DL intra band CA, applicable MPR shall be derived as MIN(MPR1,MPR2), where: MPR1 is based on single UL CC, and MPR2 is based on CABW.

- Issue 3-2-2: Applicable MPR for FR2 UL CA with DL intra band CA - WF - In general, MPR based on UL BWchannel_CA applies instead that based on cumulative aggregated channel BW (CABW) - If only 1 UL CC is activated, the MPR requirements of single carrier could be reused - No need to specify new column for <400MHz CABW in the MPR table. FFS specific spec change to accommodate 200MHz BW granularity - FFS whether to take the minimum values of MPR between the intra-band CA case and the single carrier for FR2-2

For FR2 UL CA with DL intra-band CA, it is feasible to apply MPR based on UL BWchannel_CA instead that based on CABW in general. In the case of only 1 UL CC is activated, there also exists MPR of single carrier are larger than that of intra-band UL contiguous CA. For intra-band UL contiguous CA, SEM/ACLR applies from frequencies starting from the ±edge of the UL aggregated channel bandwidth. If we use the MPR of single carrier when only 1 UL CC is activated, we will face the same problem as the discussion in FR1 UL CA whether spurious emissions/ACLR/SEM are kept for UL aggregated CBW. If the applying range of OOBE requirement keeps unchanged, SEM requirement is relaxed in some extent when use single carrier MPR for FR2 UL CA with 1 activated UL CC. Hence, with the intention of MPR reduction, it is feasible to take the minimum values of MPR between the intra-band CA case and the single carrier for FR2-2 with emissions compliance. We also propose that for FR2 UL CA with 1 activated UL CC, applicable MPR shall be derived as MIN(MPR1,MPR2), where: MPR1 is based on activated UL CC, and MPR2 is based on UL aggregated CBW.

Proposal 2: FR2 UL CA with 1 activated UL CC, applicable MPR shall be derived as MIN(MPR1,MPR2), where: MPR1 is based on activated UL CC, and MPR2 is based on UL aggregated CBW.

- Issue 3-3-1: Whether an optional UE capability is considered - Proposals - Proposal 1: no need to introduce a new UE capability for such MPR improvement as Network does not need to enable the MPR improvement originating from the application of MPR requirements. (CATT) - Proposal 2: The new capability shall be defined as optional and per UE, and only applicable to FR2. In addition, it could be release independent. (ZTE) - Proposal 3: RAN4 to consider as an optional capability. (Qualcomm) - Proposal 4: Subject to UE capability indication on the support of Rel-19 MPR applicability for FR2 CA. (Huawei) - WF - FFS in next meeting. - Issue 3-3-2: Whether UE capability, if needed, is agnostic to specific LO implementations - Proposals - Proposal 1: Consider all possible UE implementations, including UE with fast LO switching, or with dedicated LOs for Tx and Rx paths for the new UE capability. (ZTE) - Proposal 2: Capability should not be dependent on a specific implementation. (NTT DOCOMO) - WF - FFS in next meeting

For the new UE capability, we need to consider all possible UE implementations, including UE with fast LO switching, or with dedicated LOs for Tx and Rx paths. Considering the new UE capability is related to UE implementation, this new capability shall be defined as optional and per UE, and only applicable to FR2.

Proposal 3: We propose to consider all possible UE implementations, including UE with fast LO switching, or with dedicated LOs for Tx and Rx paths for the new UE capability.

Proposal 4: The new capability shall be defined as optional and per UE, and only applicable to FR2.

# 3 Conclusion

In this contribution, we shared some views on power boosting and/or MPR reduction and the proposals are made as follows:

Proposal 1: For FR2 single carrier UL with DL intra band CA, applicable MPR shall be derived as MIN(MPR1,MPR2), where: MPR1 is based on single UL CC, and MPR2 is based on CABW.

Proposal 2: FR2 UL CA with 1 activated UL CC, applicable MPR shall be derived as MIN(MPR1,MPR2), where: MPR1 is based on activated UL CC, and MPR2 is based on UL aggregated CBW.

Proposal 3: We propose to consider all possible UE implementations, including UE with fast LO switching, or with dedicated LOs for Tx and Rx paths for the new UE capability.

Proposal 4: The new capability shall be defined as optional and per UE, and only applicable to FR2.

# 4 Reference

- RP-240828, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, Huawei