# 3GPP TSG- RAN WG4 Meeting # 112bis	R4-2415825

# Hefei, China, 14th–18th Oct, 2024

# Source: 	ZTE Corporation, Sanechips

# Title: 	Discussion on SAN RF requirements for NR NTN Ku band

# Agenda item:	6.9.5

# Document for:	Approval

# 1 Introduction

In RAN#104 meeting, a new Work Item [1] was approved on introduction of Ku Band for NR NTN for NR NTN with the following RAN4 objective. In this contribution, we want to share some initial views about NR NTN Ku band from SAN RF requirements perspective.

The following assumptions are taken as the baseline for this work: - GSO and NGSO - NTN capable UE type 1 to 5 (38.101-5 Table 9.2.1.0-1) for above 10 GHz (no handheld devices) - FDD Mode - GNSS Receiver capability The objectives are: - Use the regulation requirements in ITU Regions 1, 2 and 3 to identify relevant adjacent band co-existence scenarios for NTN Ku Band covering the following frequency ranges, considering targeted deployment scenarios: - For ITU regions 1 and 3: - Downlink 10.70 – 12.75 GHz - Uplink 13.75-14.5 GHz - For ITU regions 2 excluding US: - Downlink 10.70 – 12.70 GHz - Uplink 13.75-14.5 GHz - For ITU regions 1 and 3 - Downlink 10.70 – 12.75 GHz - Uplink 12.75 - 13.25 GHz - For ITU regions 2 excluding US - Downlink 10.70 – 12.70 GHz - Uplink 12.70 - 13.25 GHz - Note: Appropriate TX/Rx separation for uplink downlink pairing will be required. - Conduct coexistence study. Reuse Ka band coexistence study assumptions as much as possible, with modifications where necessary, e.g. for frequency range and subcarrier spacing. Conduct new co-existence studies based on adjacent services . - Specify RF requirements for satellite access node and NTN VSAT types in 38.101-5 also considering existing regulations on antenna sizes for certain parts of the Ku band. - Priority 1: for all ITU regions, define Ku band #1a and #1b for the downlink block and uplink block B to enable the future possibility of supporting satellite services in 125 MHz (UL) and 250 MHz (DL) aggregated bandwidths based on existing channel bandwidths. Potential approaches for achieving this shall be studied, from which one shall be selected including but not limited to: - Ku band #1a and #1b as FR1-NTN supporting 20 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing - Ku band #1a and #1b as FR2-NTN supporting 50 MHz, 100 MHz, 200 MHz, 400 MHz (120kHz SCS only) channel bandwidths with 120 kHz subcarrier spacing - RAN4 will select which one of the above approaches for DL and UL will be pursued by RAN #106. - Priority 2: for all ITU regions excluding the US in region 2, using the approach in Priority 1, define Ku band #2a and #2b as appropriate for the downlink block and uplink block A to support satellite services in 125 MHz (UL) and 250 MHz (DL) aggregated bandwidths. - Specify RRM requirements Note: Support for NTN CA is not in the scope of this WID

# 2 Discussion

- Issue 2-1-1: Supported SAN classes for NR NTN Ku band - The supported SAN classes for NR NTN Ku band include GEO, LEO-600, and LEO-1200.

The above is way forward in last meeting, we reached an agreement on NTN VSAT types, and no agreement on SAN RF requirements.

  - 2.1 SAN types

There are three SAN types defined in TS 38.108: SAN type 1-H, SAN type 1-O and SAN type 2-O, and only SAN type 2-O is specified for FR2-NTN. Based on our analysis in [2][3], FR1 numerology is feasible for Ku band, and ECC regulations specify some radiated requirements such as off-axis EIRP emission density, on-axis/off-axis spurious requirement, antenna pointing accuracy and performance, and receiver antenna off-axis performance. Hence, it is proposed to support SAN type 1-O and SAN type 1-H for Ku band.

Proposal 1: It is proposed to support SAN type 1-O and SAN type 1-H for Ku band.

  - 2.2 SAN RF requirements

We make the following proposals for SAN RF requirements for NTN Ku band.

Table 2.2 SAN RF requirements for NTN Ku band

| NR BS RF Tx/Rx requirements | Proposal for NTN Ku band |
| --- | --- |
| Conducted Tx Requirements |  |
| 6.2 SAN output power | No specification impact. |
| 6.3 Output power dynamics | Need to define requirements for new channel bandwidths. |
| 6.4 Transmit ON/OFF power | Not applicable. |
| 6.5 Transmitted signal quality | Not band specific --> No specification impact. |
| 6.6.2 Occupied bandwidth | Not band specific --> No specification impact. |
| 6.6.3 Adjacent Channel Leakage Power Ratio (ACLR) | It depends on the outcome of coexistence study. In addition, it needs to define requirements for new channel bandwidths. |
| 6.6.4 Out-of-band emissions | Not band specific --> No specification impact. |
| 6.6.5 Transmitter spurious emissions | Not band specific --> No specification impact. |
| 6.7 Transmitter intermodulation | Not applicable. |
| Conducted Rx Requirements |  |
| 7.2 Reference sensitivity level | Need to define requirements for new channel bandwidths. |
| 7.3 Dynamic range | Need to define requirements for new channel bandwidths. |
| 7.4.1 Adjacent channel selectivity | It depends on the outcome of coexistence study. In addition, it needs to define requirements for new channel bandwidths. |
| 7.4.2 In-band blocking | Not applicable. |
| 7.5 Out-of-band blocking | Not band specific --> No specification impact. |
| 7.6 Receiver spurious emissions | Not applicable. |
| 7.7 Receiver intermodulation | Not applicable. |
| 7.8 In-channel selectivity | Need to define requirements for new channel bandwidths. |
| Radiated Tx Requirements |  |
| 9.2 Radiated transmit power | No specification impact. |
| 9.3 OTA SAN output power | No specification impact. |
| 9.4 OTA output power dynamics | Use SAN type 1-H requirements. --> No specification impact. |
| 9.5 OTA transmit ON/OFF power | Not applicable. |
| 9.6 OTA transmitted signal quality | No specification impact. |
| 9.7.2 OTA occupied bandwidth | No specification impact. |
| 9.7.3 OTA Adjacent Channel Leakage Power Ratio | Use ACLR requirements in tables 6.6.3.2-1 and 6.6.3.2-2. --> No specification impact. |
| 9.7.4 OTA out-of-band emissions | No specification impact. |
| 9.7.5 OTA transmitter spurious emissions | No specification impact. |
| 9.8 OTA transmitter intermodulation | Not applicable. |
| Radiated Rx Requirements |  |
| 10.2 OTA sensitivity | No specification impact. |
| 10.3 OTA reference sensitivity level | Need to define requirements for new channel bandwidths. |
| 10.4 OTA dynamic range | Need to define requirements for new channel bandwidths. |
| 10.5.1 OTA adjacent channel selectivity | It depends on the outcome of coexistence study. In addition, it needs to define requirements for new channel bandwidths. |
| 10.5.2 OTA in-band blocking | Not applicable. |
| 10.6 OTA out-of-band blocking | No specification impact. |
| 10.7 OTA receiver spurious emissions | Not applicable. |
| 10.8 OTA receiver intermodulation | Not applicable. |
| 10.9 OTA in-channel selectivity | Need to define requirements for new channel bandwidths. |

Proposal 2: We make the proposals as Table 2.2 for SAN RF requirements for NTN Ku band.

# 3 Conclusion

In this contribution, we shared some views on SAN RF requirements for NTN Ku band and the proposals are made as follows:

Proposal 1: It is proposed to support SAN type 1-O and SAN type 1-H for Ku band.

Proposal 2: We make the proposals as Table 2.2 for SAN RF requirements for NTN Ku band.

# 4 Reference

- RP-241690, New WID on Introduction of Ku Band for NR NTN, Intelsat et al.