3GPP TSG-RAN WG4 Meeting # 112bis	R4-2415829

# Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:	6.27.3

Source: 	Huawei, HiSilicon

Title: 	Discussion on RRM impacts for R19 IoT NTN

Document for:	Discussion

# Introduction

The new WID of NTN IoT was approved in [1] and revised [2][3]. In this contribution, we provide our initial analysis of RRM impacts of the WI.

# Discussion

In last meeting, RAN4 initiate the discussion about the scope of RRM impact of the Work item, with following agreement:

### Sub-Topic 1-1: RRM impact The work scope of RRM for Rel-19 IoT NTN is to discuss whether and how to define timing requirements for Msg3 transmission without msg1/ Random Access Response (RAR). - Note: It can be revisited if the RRM impact from other objectives has been identified.

In additional, RAN4 discuss the LS from RAN2 about the timing aspect for Msg3 without msg1/2 [4]. Based on the discussion in last RAN4 meeting, RAN4 rely the LS in [5]. In Aug meeting, RAN1 also discussed the LS and send the LS reply in [6], in which RAN4 is required to confirm the assumptions made by RAN1. The content of the LS is shown as follows:

1. Overall Description: RAN1 would like to thank RAN2 for the LS in R1-2405786/R2-2405769 on UL synchronization for contention based Msg3 transmission without Msg1/Msg2. Regarding RAN2’s questions, following is the RAN1’s response: Q1: Whether an RRC Idle UE with a pre-compensated TA (i.e., the one used for Msg1 transmission during random access for IoT NTN) can satisfy the required timing accuracy for Msg3 transmission without Msg1/Msg2? RAN1’s response: From RAN1 perspective, assuming Note 1 and Note 2 are satisfied, an RRC Idle UE with a pre-compensated TA (i.e., the one used for Msg1 transmission during random access for IoT NTN) can satisfy requirement for Msg3 transmission/reception without Msg1/Msg2 at least in the following cases: - NB-IoT with 3.75kHz SCS. - eMTC CE mode A. Note 1: RAN1 assumes NTA=0 for Msg3 transmission without Msg1/Msg2 Note 2: RAN1 assumes that the RAN4 UL synchronization requirements specified in Table 7.20A.2-1 and Table 7.24A.2-1 of TS 36.133 apply to the Msg3 transmission without Msg1/Msg2 in NB-IoT over NTN and eMTC over NTN, respectively. Note 3: RAN1 may further discuss the cases of NB-IoT with 15kHz SCS and eMTC CE mode B 2. Actions: To RAN2: ACTION: RAN1 respectfully asks RAN2 to take the response into account. To RAN4: ACTION: RAN1 respectfully asks RAN4 to confirm whether RAN1’s assumptions in Note 1 and Note 2 are valid.

Apparently, Note 2 is valid as LS reply from RAN4 in last meeting.

Observation 1: Note 2 in RAN1 LS [6] is valid as replied by RAN4 in last meeting.

For Note 1, there was discussion in last RAN4 meeting. And RAN4 applied that RAN4 assumed that NTA_Ref is not configured by the network for MSG3 transmission without Msg1/Msg2. RAN4 will follow the conclusion of the other working groups in the future, if any. From our understanding, compared with PUR procedure, for which UE will enter connected mode first, UE has no configured NTA for Msg3 transmission. Thus, it seems UE has no other choice but to assume NTA is 0.

Proposal 1: From RAN4 perspective, it is valid to assume NTA is equal to zero if there is no other procedure for NTA determination introduced by other WG

# Conclusions

Observation 1: Note 2 in RAN1 LS [6] is valid as replied by RAN4 in last meeting.

Proposal 1: From RAN4 perspective, it is valid to assume NTA is equal to zero if there is no other procedure for NTA determination introduced by other WG

# References

[1] RP-234077 New WID: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3

[2] RP-241624 Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3

[3] RP-242397 Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3

[4] R2-2405769 LS on UL synchronization for contention based Msg3 transmission without Msg1/Msg2

[5] R4-2414114 Reply LS to RAN2 on UL synchronization for contention based Msg3 transmission without Msg1/Msg2

[6] Reply LS to RAN2 on UL synchronization for contention based Msg3 transmission without Msg1/Msg2