3GPP TSG-RAN WG4 Meeting # 112bis	R4-2415832

# Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:	6.22.2.1

Source: 	Huawei, HiSilicon

Title: 	Discussion on RRM impact for Rel-19 NES

Document for:	Discussion

# Introduction

For Rel-19 NES, RAN 4 initiate the discussion in last meeting with agreements and issues captured in [1]. In this paper, we provide our views on remaining issues with further progress in other WGs considered.

# Discussion

## On-demand SSB

Regarding the on-demand SSB, RAN4 achieved following agreements in last meeting:

Issue 1-1-1: OD-SSB work plan - Agreement: - RAN4 to first focus on the MAC CE based single SCell activation case as starting point. - After RAN4 concludes on the above case, RAN4 may discuss other Scell activation scenarios if needed. Issue 1-1-2: OD-SSB scenarios - Agreement: - RAN4 to discuss OD-SSB based SCell activation requirements based on following RAN1 agreed scenarios. - Case #1 and Scenario #2 - Case #1 and Scenario #2A - Case #2 and Scenario #2 - Case #2 and Scenario #2A - Note 1: Follow RAN1 definition on the scenarios and cases. - Note 2: This list can be updated pending on RAN1 further agreement. Issue 1-1-3: Deactivated SCell measurement requirement - Agreement: - RAN4 to discuss OD-SSB based deactivated Scell measurement and Scell activation requirement. - Includes both FR1 and FR2-1 - Sub-topic 1-2: OD-SSB requirements(Case 1) Deactivated SCell L3 measurement part Issue 1-2-1: Case 1- OD-SSB based deactivated SCell measurement - Proposals - Option 1: For deactivated SCell measurement requirements, follow the periodicity of configured OD-SSB (not the measurement cycle). - Option 2: For deactivated SCell measurement requirements, follow the measCycleSCell. - Other option is not precluded Issue 2-2-2: Case 1 - OD-SSB based deactivated SCell measurement before OD-SSB indication - Agreement: - In (Case #1 Scenario #2) and (Case #1, Scenario #2A), UE is not expected to perform the SSB based deactivated SCell L3 measurement before OD-SSB is triggered. - Note: How to differentiate OD-SSB SCell and normal SCell depends on other WGs’ discussion. Issue 2-2-10: Case #1 and Scenario #2A - unknown SCell activation - Wayforward: - Option 1: RAN4 to only define unknown SCell activation provided that the OD-SSB is the first OD-SSB triggered by NW for (Case #1, Scenario #2A). - Option 2: RAN4 only specifies requirements assuming the target SCell is unknown in (Case #1, Scenario #2A). - Option 3: RAN4 needs to further consider the scenario (Case #1, Scenario #2A) together with (Case #1, Scenario #2) for deactivated SCell OD-SSB measurement. - Other option is not precluded

In last RAN1 meeting, RAN1 further discussed the activation/deactivation for OD-SSB, with following agreements:

RAN1#118 Agreement For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell at least for the case where this RRC also configures the SCell, activates the SCell, and provides on-demand SSB configuration. - FFS: Whether to support RRC based signaling for other cases. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell for Scenarios #2 and #2A. Note: Deactivation and adaptation of on-demand SSB transmission can be separately discussed. Agreement For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s), multiple candidate values can be configured by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell. - Periodicity of the on-demand SSB - FFS: Any other relevant parameters

It could be observed that RAN1 agreed to support MAC CE based OD-SSB activation, and RRC based OD-SSB activation at least for direct SCell activation. And RAN1 is also discuss the possible approach of deactivation of OD-SSB.

Observation 1: RAN1 support MAC CE based OD-SSB activation, and RRC based OD-SSB activation at least for direct SCell activation.

Observation 2: RAN1 is also discuss the possible approach of deactivation of OD-SSB.

In details, RAN1 also discussed the time offset between the OD-SSB and MAC CE which triggers the OD-SSB, and RAN1 send a LS to RAN4 to ask about the minimum value of the time offset [3]. RAN1 asks RAN4 to confirm the minimum value of T_min. The relevant agreements are shown as follows:

It could be observed that the working assumption in RAN1 is aligned with most MAC CE triggered procedures, which could be regarded as MAC CE processing time. E.g. For SCell activation requirements, when UE receives SCell activation command in slot n, there is n+Tharq+3ms in the total SCell activation delay.

Observation 3: For MAC CE processing time for OD-SSB triggering, the RAN1’s working assumption is valid which is same for most MAC CE triggered procedures.

However, for OD-SSB triggering, we identified following cases which may need further investigation for OD-SSB. For case#2 when there is always-on SSB, it is possible that the first OD-SSB is very close to the previous AO-SSB, which is shown in Fig.1. UE may have problem to process the OD-SSB, if UE is required to measure both of them. Another aspect is related to the F/T location of OD-SSB and AO-SSB. RAN1 is still under discussion on how to allocate the AO-SSB and OD-SSB. One case is that the AO-SSB and OD-SSB is in different frequency, which is shown in Fig.2. For this case, UE may need to switch to another frequency for OD-SSB measurement, and RAN4 shall investigate whether there is additional time.

Fig.1 Case#2 when the OD-SSB is very close to AO-SSB

Fig.2 Case#2 when the OD-SSB and AO-SSB is in different frequency

Proposal 1: For the value T_min for OD-SSB triggering, RAN4 confirm the MAC CE processing time as RAN1’s working assumption. RAN4 to further investigate whether to consider additional processing time for following cases, which depends on further RAN1 agreements on whether these cases are valid.

- when the OD-SSB is very close to the last AO-SSB in time domain

- when the OD-SSB and AO-SSB is in different frequency (frequency location w.r.t active BWP).

Based on the agreed 4 cases + scenarios in last RAN4 meeting, RAN4 will discuss the RRM impacts for following two procedures:

- SCell activation based on OD-SSB

- Deactivated SCell activation based on OD-SSB

For SCell activation based on OD-SSB, following valuable scenarios shall be considered:

Pure Unknown SCell activation (unknown and without L3 report after SCell activation command): For this case, per RAN1/RAN2 agreements, which refers to scenario 2A in RAN1, a new MAC CE for OD-SSB transmission will be transmitted together with SCell activation command. Then, for case#1, it is straight forward that UE shall perform all L3/L1 procedures based on OD-SSB. However, for case#2, whether UE shall use OD-SSB only all both OD-SSB and AO-SSB shall wait for further RAN1 design on the allocation of OD-SSB and AO-SSB.

Proposal 2: For pure Unknown SCell activation (unknown and without L3 report after SCell activation command):

- Case#1: UE shall perform all L3/L1 procedures (e.g. AGC/Search/L1-RSRP/timing) based on OD-SSB, and RAN4 to define requirements accordingly.

- Case#2: RAN4 to discuss whether UE shall use OD-SSB only or both OD-SSB and AO-SSB pending on further RAN1 conclusion

For unknown SCell activation with L3 report after SCell activation, there is no AGC/Search/L1-RSRP during the SCell activation procedure. However, one possible enhancement is that UE report L3 report based on OD-SSB in deactivated state. It is similar to extend the to known case when the L3 report based on OD-SSB before SCell activation, but the spec impacts could be minor.

Proposal 3: For known SCell activation and unknown SCell activation with L3 report after SCell activation, RAN4 to define requirements when the L3 measurement report is based on OD-SSB (Scenario#2).

Another aspect with significant RAN4 impacts is to define requirements based on OD-SSB in deactivated state, as agreed in last RAN4 meeting. One issue raised up in last meeting is about the whether UE shall follow periodicity of OD-SSB or the measCycleSCell. The motivation is about the UE power consumption. In legacy requirements, when an SCell is configured on the frequency in deactivated state, UE perform measurement on the frequency layer using measCycleSCell, for which the minimum value is 160ms.

Observation 4: In legacy requirements, UE is allowed relaxed measurement on a frequency with deactivated SCell using measCycleSCell than a frequency without SCell.

However, the essence of the feature is to transmit SSB “on-demand”. If UE performs measurement based on measCycleSCell, which means UE can choose any SSB within the measCycleSCell to perform measurement. It means NW has to transmit all OD-SSBs within measCycleSCell to guarantee the UE can obtain SSB no matter which SSB is chosen by the UE for measurement. But it somehow means many SSBs are transmitted but unused, which goes against with the purpose of the feature (i.e. transmit SSB “on-demand” for network energy saving).

Observation 5: If UE performs deactivated SCell measurement based on measCycleSCell, NW may have to transmit all SSBs to guarantee that UE can obtain SSB no matter which SSB is chosen by the UE for measurement, which means larger portion of SSBs are transmitted but unused.

Observation 6: The behavior described in observation 5 is against the purpose of the feature, which is to transmit SSB “on-demand” for network energy saving.

Proposal 4: UE shall perform deactivated SCell measurement based on OD-SSB periodicity to enable that NW only transmit OD-SSB when necessary (i.e. UE will perform measurement on) for network energy saving.

However, we believe the same principle in observation 4 shall be considered for UE power saving. Though there is no intention for NW to transmit OD-SSB very frequently (no energy saving gains), RAN4 could discuss possible conditions to avoid too frequency measurement. For instance, UE can perform “one round” deactivated SCell measurement based on OD-SSB and obtain results of the deactivated SCell, then UE is not required to perform the “2nd round” measurement right after first round, which is shown in Fig.3. In this way, UE can achieve similar or even better UE power saving gain. Meanwhile, the validity of measurement result shall be considered. If the interval between two rounds of OD-SSB measurement is too large (e.g. 100 seconds), UE may not be able to measure the Cell since UE may already loss the tracking of the Cell and has to keep searching it all the time. At the same time, it is not possible that results are reliable for known SCell activation or L3 measurement report after SCell activation.

Fig.3 deactivated SCell activation based on OD-SSB

Proposal 5: Based on proposal 4, RAN4 to discuss the interval between OD-SSBs for measurement considering following aspects:

- To avoid too frequency measurement which may cause UE power consumption issue

- To maintain the timing/AGC for subsequent measurement/SCell activation

For L3 measurement, apart from deactivated SCell measurement, since the scenario 3B is not agreed yet in RAN1, there is no need to consider L3 measurement based on OD-SSB after SCell is activated. But one may argue that it is still possible that a neighbor cell is sending OD-SSB, then whether and how-to performance L3 measurement on this neighbor cell. For a neighbour cell, if there is always on periodic SSB, we believe current L3 measurement requirements and UE behaviour still apply since UE can perform L3 measurement in legacy manner base on these always on SSB, no matter whether there is OD SSB or not. However, if there is a Cell with only OD-SSB, it is not clear whether the Cell is expected to be detected by other UEs base on neighbour measurement. Since the OD-SSB could be quite dynamic, it is not possible to dynamically cooperate among many Cell about the on-off state of OD-SSB, and transmit the information to all UE under these Cells. And from UE perspective, it is also questionable whether UE can tell if the OD-SSB is transmitted of not.

Proposal 6: RAN4 to deprioritize the discussion for L3 measurement based on OD SSB for neighbour cell, as feasibility and reliability are questionable and gain is not clear.

# Conclusions

Observation 1: RAN1 support MAC CE based OD-SSB activation, and RRC based OD-SSB activation at least for direct SCell activation.

Observation 2: RAN1 is also discuss the possible approach of deactivation of OD-SSB.

Observation 3: For MAC CE processing time for OD-SSB triggering, the RAN1’s working assumption is valid which is same for most MAC CE triggered procedures.

Proposal 1: For the value T_min for OD-SSB triggering, RAN4 confirm the MAC CE processing time as RAN1’s working assumption. RAN4 to further investigate whether to consider additional processing time for following cases, which depends on further RAN1 agreements on whether these cases are valid.

- when the OD-SSB is very close to the last AO-SSB in time domain

- when the OD-SSB and AO-SSB is in different frequency (frequency location w.r.t active BWP).

Proposal 2: For pure Unknown SCell activation (unknown and without L3 report after SCell activation command):

- Case#1: UE shall perform all L3/L1 procedures (e.g. AGC/Search/L1-RSRP/timing) based on OD-SSB, and RAN4 to define requirements accordingly.

- Case#2: RAN4 to discuss whether UE shall use OD-SSB only or both OD-SSB and AO-SSB pending on further RAN1 conclusion

Proposal 3: For known SCell activation and unknown SCell activation with L3 report after SCell activation, RAN4 to define requirements when the L3 measurement report is based on OD-SSB (Scenario#2).

Observation 4: In legacy requirements, UE is allowed relaxed measurement on a frequency with deactivated SCell using measCycleSCell than a frequency without SCell.

Observation 5: If UE performs deactivated SCell measurement based on measCycleSCell, NW may have to transmit all SSBs to guarantee that UE can obtain SSB no matter which SSB is chosen by the UE for measurement, which means larger portion of SSBs are transmitted but unused.

Observation 6: The behavior described in observation 5 is against the purpose of the feature, which is to transmit SSB “on-demand” for network energy saving.

Proposal 4: UE shall perform deactivated SCell measurement based on OD-SSB periodicity to enable that NW only transmit OD-SSB when necessary (i.e. UE will perform measurement on) for network energy saving.

Proposal 5: Based on proposal 4, RAN4 to discuss the interval between OD-SSBs for measurement considering following aspects:

- To avoid too frequency measurement which may cause UE power consumption issue

- To maintain the timing/AGC for subsequent measurement/SCell activation

Proposal 6: RAN4 to deprioritize the discussion for L3 measurement based on OD SSB for neighbour cell, as feasibility and reliability are questionable and gain is not clear.

# References

[1] R4-2413885 WF on Netw_Energy_NR_enh

[2] RP-242354 Revised WID: Enhancements of network energy savings for NR

[3] R1-2407565 LS on timeline for On-demand SSB operation on SCell