3GPP TSG-RAN WG4 Meeting # 112bis	R4-2415835

# Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:	6.15.3

Source: 	Huawei, HiSilicon

Title: 	Discussion on Fast SCell activation for UE supporting Rel-18 EMR

Document for:	Discussion

# Introduction

The new Work Item about RRM phase 5 was approved in [1]. One of the objectives which is Fast SCell activation for UE supporting Rel-18 EMR, which was discussed in last RAN4 meeting. In this contribution, we further provide our views regarding the RAN4 impacts for this objective.

# Discussion

Based on the discussion in last meeting, RAN4 achieved following agreement [2]:

- Issue 1-1-1: Clarification on Rel-18 eEMR Agreements: - Do not change the Rel-18 eEMR definition in this fast SCell activation discussion. - Issue 1-1-2: Applicability of fast SCell activation delay requirements Agreement: - The fast SCell activation delay requirements are defined for the case when - the UE supports Rel-18 eEMR: - Including both EMR and cell reselection measurement, - Including the case when measIdleValidityDuration-r18 and / or measReselectionValidityDuration-r18 are configured - FFS Including the case when measIdleValidityDuration-r18 and / or measReselectionValidityDuration-r18 are not configured - and the UE has reported valid results on the SCell to be activated before SCell activation command. - Issue 1-1-3: Scope of fast SCell activation for UE supporting Rel-18 eEMR Agreement: - Start from the following scope of fast SCell activation for UE supporting Rel-18 EMR: - Direct SCell Activation at SCell addition - FFS: Single MAC CE based DL SCell Activation - FFS: Single MAC CE based PUCCH SCell Activation - RAN4 to discuss whether and how to apply the solutions to other SCell activation procedures after RAN4 has conclusions for the above cases. - Issue 1-2-1: How to define the fast SCell activation delay requirements with valid eEMR reporting Agreement: - RAN4 to update the known condition for SCell activation to include the case when UE has valid EMR reporting before SCell activation command. - FFS how to update the known condition (issue 1-2-1a). - FFS the applicability of valid EMR reporting for the Rel-19 fast SCell activation

The description of the objective is shown as follows:

- Fast SCell activation for UE supporting Rel-18 EMR - Study and, if feasible, to reduce the SCell activation delay with valid EMR reporting - Apply fast scell activation in FR1 and FR2-1 - Note: RAN4 to start this work from Q3’2024 and aim for completion in Dec’2024. Workplan for this bullet can be discussed in May’2024

As emphasized in last meeting, this is a three-meetings objective, which means there is no chance for complicated or over enhanced solutions. The motivation of the objective is that: the known conditions in existing spec relies on L3 report which is shown as follows.

SCell in FR1 is known if it has been meeting the following conditions: - During the period equal to max(5*measCycleSCell, 5*DRX cycles) for FR1 before the reception of the SCell activation command: - the UE has sent a valid measurement report for the SCell being activated and - the SSB measured remains detectable according to the cell identification conditions specified in clause 9.2 and 9.3. - the SSB measured during the period equal to max(5*measCycleSCell, 5*DRX cycles) also remains detectable during the SCell activation delay according to the cell identification conditions specified in clause 9.2 and 9.3. Otherwise SCell in FR1 is unknown. For the first SCell activation in FR2 bands, the SCell is known if it has been meeting the following conditions: - During the period equal to 4s for UE supporting power class 1/5 and 3s for UE supporting power class 2/3/4 before UE receives the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and semi-persistent CSI-RS for CQI reporting (when applicable): - the UE has sent a valid L3-RSRP measurement report with SSB index, and - SCell activation command is received after L3-RSRP reporting and no later than the time when UE receives MAC-CE command for TCI activation - During the period from L3-RSRP reporting to the valid CQI reporting, the reported SSBs with indexes remain detectable according to the cell identification conditions specified in clauses 9.2 and 9.3, and the TCI state is selected based on one of the latest reported SSB indexes.

In Rel-18 EMR, RAN4 introduce validation procedure before EMR reporting. If configured by NW, UE shall verify the EMR results and only the valid one can be report. The corresponding conditions are shown as follows:

A UE shall perform validity check and report valid measurement results: - if the UE is supporting [Measurement validation based on EMR measurement] and measIdleValidityDuration-r18 is configured for carriers in measIdleCarrierListNR-r16 or measIdleCarrierListEUTRA-r16, - if the UE is supporting [Measurement validation based on non-EMR measurement] and measReselectionValidityDuration-r18 is configured for carriers in measReselectionCarrierListNR-r18 The measurement results are considered valid if the following conditions are met for the validity check: - the measurements are performed before msg1 transmission for RRC resume/setup request within the last: - measIdleValidityDuration-r18 seconds for carriers configured in measIdleCarrierListNR-r16 or measIdleCarrierListEUTRA-r16, and/or - measReselectionValidityDuration-r18 seconds for carriers configured in measReselectionCarrierListNR-r18, - the measurement results satisfy measurement accuracy requirement at the measurement instance.

Observation 1: Whether EMR report can enable known SCell activation is not explicitly specified in specifications. The baseline solution (i.e with minimum spec impact/workload) is to extend the known condition for SCell activation considering the valid L3-RSRP measurement report via EMR report.

Regarding whether and how to define conditions/definition of “valid L3-RSRP measurement report via EMR”. The intention is to tell NW that these reports are reliable for known SCell activation. And UE knows whether these results can be reported since UE shall meet known SCell activation based on them.

Observation 2: The intention to define conditions/definition of “valid L3-RSRP measurement report via EMR” is to:

- tell NW that these EMR reports are reliable for known SCell activation

- let UE know whether these results can be reported as UE shall meet known SCell activation based on them.

Taking existing validity conditions for Rel-18, the question is whether it is already sufficient for known SCell activation. The answer is NO. For instance, if the validity duration is configured as 100 seconds, based on existing Rel-18 EMR validity check conditions, UE can report it as long as it is performed within 100 seconds before msg1. But it does not necessarily mean the result can be used for known SCell activation.

Observation 3: The validity check condition (the time span between the measurement instance and msg1) for Rel-18 EMR may not fully guarantee that the EMR report can enable known SCell activation.

Thus, to enable known SCell activation based on EMR, following options could be considered:

- Option 1: UE support Rel-19 EMR based known SCell activation, it means all EMR report shall also guarantee known SCell activation.

For instance, even measReselectionValidityDuration-r18 is configured as 100 seconds, since UE support Rel-19 EMR based known SCell activation, the EMR report shall guarantee known SCell activation. It means UE shall be able to maintain the timing of the SCell for quite long time. From NW point of views, 100 seconds validity duration may be too long, and it is not reliable enough for known SCell activation, then NW can configure a shorter validation duration (e.g. 5s /10s).

- Option 2: RAN4 to define a condition/limit that measurement performed X seconds before SCell activation is considered as valid for known SCell activation conditions.

Compared with option 1, companies may have concerns that UE may not be able to maintain the timing of the deactivated SCell for known SCell activation. For instance, the measurement period of is defined in terms of DRX cycle, which could be very long. If the measurement period is several tens of seconds, it may be challenging to UE to maintain the timing of the Cell for known SCell activation (i.e. without L3 procedure). Thus, RAN4 could discuss a limit on the value. For instance, RAN4 could agree that 20s is a reasonable number. Thus, for UE support Rel-19 EMR based known SCell activation, when the validityDuration is configured less than or equal to 20s, UE shall guarantee that the EMR report can be used for fast SCell activation. The value of X could also be up to UE capability, if it is hard to converge on one value.

- Option 3: Introduce new dedicated EMR based SCell activation indication

Different from option 1 and option 2, it is also feasible to have dedicated EMR based SCell activation indication. Then, UE knowns that the expected EMR results is for known SCell activation, and UE shall guarantee that it can meet the known SCell activation requirements. Since this indication is dedicated for this purpose, it may not need to additionally define other conditions. As long as UE support this feature, and NW configure this EMR based SCell activation dedicated indication, before UE reports the EMR results upon entering connected mode, UE shall guarantee that the EMR result could be used for known SCell activation if the Cell is activated as SCell within 3 or 4 seconds (take FR2 as example) after EMR report.

Based on above analysis, the Text proposal for potential spec updating could be as follows (take 8.3.2 FR2 as example)

For the first SCell activation in FR2 bands, the SCell is known if it has been meeting the following conditions: - During the period equal to 4s for UE supporting power class 1/5 and 3s for UE supporting power class 2/3/4 before UE receives the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and semi-persistent CSI-RS for CQI reporting (when applicable): - the UE has sent a valid L3-RSRP measurement report with SSB index. - if the L3-RSRP measurement report with SSB index is from EMR report, the report is considered as valid when following conditions are met: - TBD - SCell activation command is received after L3-RSRP reporting and no later than the time when UE receives MAC-CE command for TCI activation - During the period from L3-RSRP reporting to the valid CQI reporting, the reported SSBs with indexes remain detectable according to the cell identification conditions specified in clauses 9.2 and 9.3, and the TCI state is selected based on one of the latest reported SSB indexes.

As mentioned above, it is a three-meeting objective and also argued by many companies in last meeting that we should try to reuse Rel-18 EMR as much as possible, we think option 2 is more reasonable. The value of measIdleValidityDuration-r18/ measReselectionValidityDuration-r18 is shown as follows. One approach is that RAN4 define the principle in the spec that when measIdleValidityDuration-r18/ measReselectionValidityDuration-r18 is configured and the value is less than X (e.g. 5 or 10 seconds), which means the valid EMR report means the measurement is perform X seconds before Msg1. Then UE shall guarantee that the EMR report can guarantee known SCell activation. One step further is to allow flexibility of the value of X. Different UE may have different capability to maintain the timing to guarantee known SCell activation. For instance, UE1 could guarantee known SCell activation as long as the measurement is performed within 10 seconds, while UE2 can maintain the timing for 20 seconds. Thus, UE can indicate the value by UE capabilities. Thus, for different UE, NW can configure different measIdleValidityDuration-r18/ measReselectionValidityDuration-r18 values.

Proposal 1: The known conditions for SCell activation with valid EMR reporting before SCell activation is defined as:

- the measurements are performed before msg1 transmission for RRC resume/setup request within the last measIdleValidityDuration-r18/ measReselectionValidityDuration-r18, when measIdleValidityDuration-r18/ measReselectionValidityDuration-r18 is less than X seconds.

Proposal 2: Based on proposal 1, RAN4 to discuss whether X is a constant value defined in spec or up to UE capability (i.e. Rel-19 EMR-based SCell activation capability).

# Conclusions

Observation 1: Whether EMR report can enable known SCell activation is not explicitly specified in specifications. The baseline solution (i.e with minimum spec impact/workload) is to extend the known condition for SCell activation considering the valid L3-RSRP measurement report via EMR report.

Observation 2: The intention to define conditions/definition of “valid L3-RSRP measurement report via EMR” is to:

- tell NW that these EMR reports are reliable for known SCell activation

- let UE know whether these results can be reported since UE shall meet known SCell activation based on them.

Observation 3: The validity check condition (the time span between the measurement instance and msg1) for Rel-18 EMR may not fully guarantee that the EMR report can enable known SCell activation.

Proposal 1: The known conditions for SCell activation with valid EMR reporting before SCell activation is defined as:

- the measurements are performed before msg1 transmission for RRC resume/setup request within the last measIdleValidityDuration-r18/ measReselectionValidityDuration-r18, when measIdleValidityDuration-r18/ measReselectionValidityDuration-r18 is less than X seconds.

Proposal 2: Based on proposal 1, RAN4 to discuss whether X is a constant value defined in spec or up to UE capability (i.e. Rel-19 EMR-based SCell activation capability).

# References

[1] RP-240830 New WID: WI resulting from discussion on RRM enhancements

[2] R4-2414041 WF on NR_RRM_Ph5_Part2 - fast SCell activation