3GPP TSG-RAN WG4 Meeting # 112bis	R4-2415836

# Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:	6.25.2

Source: 	Huawei, HiSilicon

Title: 	Discussion on RRM requirements for Rel-19 XR

Document for:	Discussion

# Introduction

The new WI about XR was approved in [1] and further revised in [2]. In last RAN4 meeting, RAN4 initially discussed the RRM impacts of the WI with agreements and issues captured in the WF [3]. In this contribution, we further provide our views on remaining issues.

# Discussion

## 2.1 Scenario

It should be noted that the WID was further updated in RAN#105 [4]. Based on our understanding, there is no significant change on RRM work for the WID updating.

Observation 1: No significant change on RRM impact based on the WID updating in RAN#105.

Based on latest version of the WID, RAN4 RRM work is still mainly about enable transmission/reception in gap/restriction caused by RRM measurement. The relevant description is shown as follows:

- Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc). [RAN1, RAN2, RAN4] - Specify the corresponding measurement gap and scheduling restriction to enable the identified enhancements with RRM performance impact taken into consideration, work being triggered by LS. [RAN4]

Based on progress in last RAN4 meeting as captured in WF [3], RAN4 spend quite a lot time to clarify the scenario gap and scheduling restrictions. One issue is about whether the to consider the scheduling restriction due to L1 operation. From our understanding, the discussion about the feature is only about L3 measurement (in MG or with measurement restriction). It could be also found in the TR 38.835 which his shown as follows that only intra-frequency and inter-frequency are studied. Another technical reason is that L1 measurement is highly correlated to data scheduling. We can try to find additional scheduling opportunity in L3 measurement occasions since there could be the case that the mobility demand is not that urgent when there is data XR data. But for L1 measurement, which is to guarantee the link between the UE and the serving cell, we see less room for cancel L1 measurement for additional transmission opportunity. Based on analysis above, we suggest to only consider the Tx/Rx in occasions of L3 measurement.

Proposal 1: RAN4 to only consider Tx/Rx in occasions of L3 measurement.

Another issue RAN4 discussed in last meeting is about the type of MGs, and RAN4 indeed spend quite a lot time on it. From our understanding, we see no significant difference of the solutions/RRM impacts for different MG types. Companies spend a lot time discussing which kind(s) MGs to be considered in the feature, but without very good justifications. This deliberate down selection on type of MGs does not help to figure out how this solution could work. Thus, we suggest RAN4 to deprioritize the discussion on applicable type of MGs at early stage of the WI, and focus on the solutions/impacts on RRM requirements. Unless, it is found that a particular type of MG cannot be supported or need special treatment, RAN4 can discuss whether to the feature can apply to this MG type.

Proposal 2: RAN4 to deprioritize the discussion on applicable type of MGs at early stage of the WI, and focus on the solutions/impacts on RRM requirements. Unless, it is found that a particular type of MG cannot be supported or need special treatment, RAN4 can discuss whether the feature can apply to this MG type.

## 2.2 LS on UAI

RAN4 receive the LS [5] from RAN1 in last meeting, but without much time for discussion. The LS from RAN1 are as follows:

RAN1 discussed UE assistance information related to measurements occasions and related to channel conditions, traffic, UE mobility. The following agreement and conclusions were agreed: Agreement RAN1 continues to discuss and decide whether or not to introduce new UE assistance information for solution(s) to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements. At least the following UE assistance information is considered for further study: - FFS: UE assistance information related to measurement occasions: - FFS: The number of needed measurement gaps/SMTC with restrictions within a time period; - FFS: The maximum number or ratio of MGs/SMTC with restrictions that can be skipped within a time period; - FFS: The number of required SSBs within a time period; - FFS: The number of consecutive RRM measurements that can be skipped; - FFS: The maximum interval between two consecutively reserved gap/restriction occasions for RRM measurements; - FFS: The patterns of gap(s)/restriction(s) where skipping is feasible or acceptable; - FFS: UE assistance information related to channel conditions: - FFS: RSRP is below/above search threshold (s-MeasureConfig); - FFS: UE assistance information related to traffic: - FFS: PSI (PDU set importance); - FFS: UE assistance information related to UE mobility: - FFS: L3 parameters related to mobility, e.g., static or not Companies are encouraged to provide additional details (e.g. how often the UE assistance info is provided, timing, applicable scenarios, performance gains, etc) on their preferred scheme. Note: From specification point of view, there is no mandated gNB behavior in response to any of the UE assistance information. RAN1 to make decision, from RAN1 perspective, in RAN1#117 on the support of UE assistance information. Conclusion RAN1 does not further discuss new UE assistance information related to channel conditions, traffic, UE mobility. Conclusion There is no consensus in RAN1 to support UE assistance information related to measurements occasions. The reason for this situation is lack of consensus on the need/feasibility for UAI and lack of technical understanding on issues outside of RAN1 expertise (e.g. impact of RRM measurement performance). It is up to other working groups to trigger further work in RAN1 on UE assistance information.. Particularly, it was discussed whether impact on RRM performance from skipping measurement occasions may be reduced if UE sends additional information: - Information about the maximum number of MGs/SMTC with restrictions that can be skipped within a time period. - Information about the patterns of gap(s)/restriction(s) where skipping is feasible or acceptable.

It could be observed that RAN1 has discuss whether and how to introduce UAI for several meetings but without consensus. RAN1 can not converge on feasibility and need of such UAI, which is more related to RAN4 RRM discussion. In the following part of the contribution, we would like to present our views on how the overall procedure could work to improve the experience of XR service by enabling transmission in MG/scheduling restriction, and whether UAI is needed.

As could be observed from the TR 38.835 B.1.7, performance of XR serving could be improved by canceling some measurement occasions. This maybe cause by the characteristic of XR service that XR service have typical frame rate (e.g. 60fps) and require rather long latency. The periodic MG /SMTC may lead to sustainable impact when colliding with XR data. However, to guarantee the mobility performance is also important, or even more important under certain scenario. For instance, if the UE is in the cell edge and about to handover to another cell, it is not reasonable to sacrifice the performance of RRM measurement in order to provide more scheduling opportunity for XR service, on the contrary, this may lead to server interruption to XR service.

Observation 2: In some scenarios, the RRM measurement performance (e.g. latency, accuracy) may have higher priority than XR service to guarantee the mobility performance.

Thus, from NW perspective, what is the expected UE behavior/performance is not clear. For instance, if one Gap is skipped for data transmission, what is the impacts to gap-based RRM measurement? If the consequence is that the measurement delay will be greatly extended, then NW is less likely to use this feature.

Observation 3: NW shall have clear expectation on RRM behavior/performance when the measurement occasion is skipped/canceled for data transmission/reception.

Proposal 3: RAN4 shall define corresponding RRM behavior/performance when the measurement occasion is skipped/canceled for data transmission/reception.

Based on Observation 3, network can determine whether to cancel a measurement occasion for data transmission considering the mobility status and urgency of the service. For RRM requirement perspective, one interpretation could be that, with limited number of MG/SMTC skipped, UE can fulfill the same requirements. It is because that the requirements is the minimum requirements considering the worst case, with current MG/MO configurations, it is likely that not all gap are actually used for measurement. For this particular enhancement, these occasions are wasted ones that could be used for data transmission/reception. Another interpretation could be that as long as one of the gap/SMTC is taken, the delay has to be extended accordingly (e.g. like NR-U).

Observation 4: Regarding the RRM behavior/performance when the measurement occasion is skipped/canceled, following options can be taken as starting point:

- Alt1: Same requirement still applies

- Alt2: The measurement delay to be extended

From our understanding, we think Alt1 is more reasonable. Take MG as example, based on reasonable NW implementation, MG are only configured when necessary since it will impact the throughput significantly. Then, we assume the RRM measurement is important at that stage. Especially, when UE is under XR service, it is not reasonable to assume that NW configured MG but the RRM measurement is not that important. Thus, from NW point views, it is safe to support this feature when it can provide additional scheduling opportunity but will not decrease the mobility performance.

Observation 5: From NW point views, it is safe to support this feature when it can provide additional scheduling opportunity but will not impacting the mobility performance.

Regarding the LS from RAN1 about UAI, some candidate options are mentioned but without agreements, which are shown as follows:

- FFS: UE assistance information related to measurement occasions: - FFS: The number of needed measurement gaps/SMTC with restrictions within a time period; - FFS: The maximum number or ratio of MGs/SMTC with restrictions that can be skipped within a time period; - FFS: The number of required SSBs within a time period; - FFS: The number of consecutive RRM measurements that can be skipped; - FFS: The maximum interval between two consecutively reserved gap/restriction occasions for RRM measurements; - FFS: The patterns of gap(s)/restriction(s) where skipping is feasible or acceptable; - FFS: UE assistance information related to channel conditions: - FFS: RSRP is below/above search threshold (s-MeasureConfig); - FFS: UE assistance information related to traffic: - FFS: PSI (PDU set importance); - FFS: UE assistance information related to UE mobility: - FFS: L3 parameters related to mobility, e.g., static or not

Regarding the mobility status (e.g. RSRP vs s-MeasureConfig), it can already be supported by current design. NW configure Event A1/A2, thus NW can know whether s-MeasureConfig is met or not. There is no need to report it via UAI which will add another layer of complexity and more latency. Regarding the UAI about measurement occasion, it may help NW scheduling as long as the expected UE measurement behaviour and requirements are clear. From scheduling perspective, the maximum number of MGs/SMTC with restriction can be skipped within a time period is more flexible. NW has the idea about the scheduling restriction, if the UAI information is only a certain pattern, the MGs/SMTC suggested by UE may not the ones that is essential for XR service.

Observation 6: UAI for measurement occasion may be helpful for enabling transmission/reception in MGs/SMTC with restriction. And the flexibility of scheduling shall be guaranteed as NW can properly scheduling the data transmission/reception.

Based on analysis above, we believe that the UAI indicate the numbers and/or position of MGs/SMTC with restriction can be skipped should be introduced, when there is no impact on L3 measurement performance. Furthermore, companies also mentioned that the measurement delay could be extended when the MGs are skipped. We think it is also reasonable scenario that there is no urgent/significant demand for mobility measurement, and gNB can take the liberty to decide whether to skip more MGs/SMTC with restriction to have more scheduling opportunities.

Proposal 4: RAN4 to introduce UAI at least for following case:

- UE reports UAI about the numbers and/or position of MGs/SMTC with restriction can be skipped, when there is no impact on L3 measurement performance

Proposal 5: RAN4 to discuss whether to introduce UAI for following case:

- UE reports UAI about the numbers and/or position of MGs/SMTC with restriction can be skipped, when the measurement delay could be extended. FFS on the impact on delay extension.

## 2.3 LS on indication

RAN4 receive another LS [6] from RAN1 about indication to enable Tx/Rx in particular gap/restriction that are caused by RRM. It could be observed that there were two alternatives under discussion in RAN1 in previous meeting. One is the dynamic indication via DCI, and the other one is the Semi-static solution via RRC. And RAN1 also ask the question about the minimum time offset between the indication and the skipped measurement occasion if the dynamic approach is supported.

Agreement For solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements select one among the following options: Option 1: Support Alt. 1-1: - Alt. 1: Dynamic indication to enable Tx/Rx in particular gap(s)/restriction(s) that are caused by RRM measurements. - Alt 1-1: Explicit indication by DCI to skip a particular gap(s)/restriction(s); - Indication is included as part of scheduling DCI: - FFS: Bit-field size is one bit; - FFS: Bit-field size is >1 bit; - Note: Minimum time offset(s) between the end of [the first] received dynamic indication and start of corresponding gap(s)/restriction(s) occasion that is going to be skipped shall be introduced. - FFS: DCI format, DCI content, DCI bit-field size; - FFS: Whether indication is for one or more occasions; - FFS: How to consider time offset between the end of received dynamic indication and start of gap(s)/restriction(s) occasion that is going to be skipped. Option 2: Support Alt. 3-1: - Alt. 3: Semi-static solution to enable TX/RX in gaps/restrictions that are caused by RRM measurements. - Alt 3-1: Configure a pattern(s) via RRC to indicate occasions where to skip gaps/restrictions; - FFS: Details of pattern: - FFS: Pattern is based on periodicity, offset and duration; - FFS: Pattern is based on a bitmap; - FFS: whether a pattern is applied to all or subset of configured MG configurations/scheduling restrictions. Agreement If Alt. 1 from RAN1#117 agreement is supported, minimum time offset(s) X between indication to skip and skipped measurement occasion is up to RAN4 to discuss and decide on particular value(s). R1-2407289 Moderator summary #2 - Enabling TX/RX for XR during RRM measurements Moderator (Nokia) Working Assumption For solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements select the following option: - Alt. 1: Dynamic indication to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements. - Alt 1-1: Explicit indication by DCI to skip a particular gap/restriction; - Indication is included as part of scheduling DCI: - Bit-field size is one bit; - The bit in the DCI is used to indicate whether to skip the first gap/restriction occasion after a minimum time offset required between the last symbol of the PDCCH carrying the DCI format and the start of corresponding skipped gap/restriction occasion indicated by the DCI. Send an LS to RAN4 to inform them of the above working assumption and ask them if there is any issue with it. Final LS in R1-2407561.

From our understanding, we first agree that dynamic indication (RAN1 current working assumption) shall be supported. It is because that it can give more flexibility to NW scheduling, since NW can dynamically skip a MG for an emergent data transmission/reception. And it is hard for NW to predict which occasion could be actually used in the further.

Observation 7: The dynamic indication (Alt.1) can give NW more flexibility on scheduling.

Regarding the time offset value, it is really a UE implementation issue. Before a certain measurement occasion, there could be complicated processing procedures, e.g. RF tuning for measure a particular frequency. Thus, the value can not be too short. One example RAN4 can refer to is the 5ms for pre-MG.

However, from UE implementation perspective, we also found some issues for dynamic indications. UE may have a calculated measurement pattern/period based on existing configurations (e.g. MOs, MG). Then UE will perform measurement on different layers accordingly. However, based on the dynamic indication, the number/position which will be skipped is unknown to UE, thus it is hard for UE to determine the measurement pattern/period. Another point is that, as mentioned by many companies, there is unused MGs currently, and NW can use these occasions for scheduling. However, if the skipping is very dynamic, UE has no idea which occasion will be taken, and UE can not guarantee what will be the impact on RRM performance when different occasions are taken.

Based on the analysis above, to make full use of the feature, we suggest to consider both alternatives. RAN4 confirm the dynamic approach which can give more flexibility on NW scheduling. And RAN4 suggest RAN1/RAN2 to consider the Semi-static solution approach, that NW indicate the indicate occasions where to skip gaps/restrictions, and UE can have a stable measurement pattern/period. And whether UE support both approaches can be up to UE capability discussion.

Proposal 6: RAN4 reply LS that:

The dynamic approach (Alt.1) should be supported from RAN4 perspective, which can give more flexibility on NW scheduling.

RAN4 also suggest RAN1/2 to introduce the semi-static approach (Alt.2) that NW indicate the indicate occasions where to skip gaps/restrictions, and UE can have a stable measurement pattern/period.

Whether UE supports both approaches can be up to UE capability discussion.

# Conclusions

Observation 1: No significant on RRM impact based on the WID updating in RAN#105.

Proposal 1: RAN4 to only consider Tx/Rx in occasions of L3 measurement.

Proposal 2: RAN4 to deprioritize the discussion on applicable type of MGs at early stage of the WI, and focus on the solutions/impacts on RRM requirements. Unless, it is found that a particular type of MG cannot be supported or need special treatment, RAN4 can discuss whether the feature can apply to this MG type.

Observation 2: In some scenarios, the RRM measurement performance (e.g. latency, accuracy) may have higher priority than XR service to guarantee the mobility performance.

Observation 3: NW shall have clear expectation on RRM behavior/performance when the measurement occasion is skipped/canceled for data transmission/reception.

Proposal 3: RAN4 shall define corresponding RRM behavior/performance when the measurement occasion is skipped/canceled for data transmission/reception.

Observation 4: Regarding the RRM behavior/performance when the measurement occasion is skipped/canceled, following options can be taken as starting point:

- Alt1: Same requirement still applies

- Alt2: The measurement delay to be extended

Observation 5: From NW point views, it is safe to support this feature when it can provide additional scheduling opportunity but will not impacting the mobility performance.

Observation 6: UAI for measurement occasion may be helpful for enabling transmission/reception in MGs/SMTC with restriction. And the flexibility of scheduling shall be guaranteed as NW can properly scheduling the data transmission/reception.

Proposal 4: RAN4 to introduce UAI at least for following case:

- UE reports UAI about the numbers and/or position of MGs/SMTC with restriction can be skipped, when there is no impact on L3 measurement performance

Proposal 5: RAN4 to discuss whether to introduce UAI for following case:

- UE reports UAI about the numbers and/or position of MGs/SMTC with restriction can be skipped, when the measurement delay could be extended. FFS on the impact on delay extension.

Observation 7: The dynamic indication (Alt.1) can give NW more flexibility on scheduling.

Proposal 6: RAN4 reply LS that:

The dynamic approach (Alt.1) should be supported from RAN4 perspective, which can give more flexibility on NW scheduling.

RAN4 also suggest RAN1/2 to introduce the semi-static approach (Alt.2) that NW indicate the indicate occasions where to skip gaps/restrictions, and UE can have a stable measurement pattern/period.

Whether UE supports both approaches can be up to UE capability discussion.

# References

[1] RP‑234080 New WID: XR (eXtended Reality) for NR Phase 3

[2] RP-240791 Revised WID on XR (eXtended Reality) for NR Phase 3

[3] R4-2414043 WF on RRM requirements for XR_Ph3

[4] RP-241771 Revised WID on XR (eXtended Reality) for NR Phase 3

[5] R1-2405736 LS on UE assistance information

[6] R1-2407561 LS on gaps/restrictions that are caused by RRM measurements