3GPP TSG-RAN WG4 Meeting #112bis	R4-2415840

Hefei, Anhui, China, 14th – 18th October, 2024

Title: 	Discussion on RRM requirements for supporting intra-band non-collocated EN-DC/NR-CA deployment Phase2: new receiver type(s)

Source: 	Huawei, HiSilicon

Agenda item:	6.5.3

Document for:	Discussion

Introduction

Supporting of intra-band non-collocated EN-DC/NR-CA deployment for Type 2 UE has been completed in Rel-18. The requirements for FR1 inter-band non-contiguous EN-DC with overlapping or partially overlapping bands and FR1 intra-band non-contiguous NR-CA for Type 2 UE were specified.

The new R19 WI is to further support intra-band non-collocated EN-DC/NR-CA deployment scenarios for Type 4 FWA UE. The objectives of the WID are duplicated as below [1].

- 4.1 Objective of SI or Core part WI or Testing part WI The core part of the work item includes: - Specify the core requirements for type 4a/4b capable FWA UE that supports non-co-located scenarios for FR1 inter-band non-contiguous EN-DC with overlapping or partially overlapping bands and/or FR1 intra-band non-contiguous NR-CA for up to 4-layer DL MIMO per CC, where MRTD>CP is assumed. - RF power imbalance requirements of up to 25dB - RRM requirements including MTTD and MRTD requirements and other identified requirement such as scheduling restriction/availability, SCell operation related delay and interruption. - Work is limited to EN-DC/NR-CA for bands 42, n77/n78 - Discuss and if needed, specify the UE behavior when the UE supports type 4a/b but operates in co-located scenarios depending on whether network signaling is provided. R18 handling of UE behavior can be used as a reference.

The contribution provides some analysis for supporting intra-band non-collocated EN-DC/NR-CA deployment scenarios from RRM requirements perspective.

Discussion

Impact on RRM requirements

MRTD and MTTD

UE types for non-collocated non-contiguous intra-band NR-CA and inter-band EN-DC are defined in R4-2217734 [2]. The assumed UE architectures for Type 2 and Type 4a/4b are listed in Table 1.

Table 1: UE Types for 4-layer MIMO case [2]

| UE Type | CC# | antenna / LNA |  | Mixer | Analog BB | #Rx | Frequency Separation between 2cc | NRCA/ENDC | power imbalance | comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 4 shared |  | 4 shared | 4 shared | 4Rx | ≤ X MHz | NRCA,ENDC | 6dB full range | Baseline architecture (i.e. legacy architecture) |
|  | 2 |  |  |  |  | 4Rx |  |  |  |  |
| 2 | 1 | 2 | 4 total | 2 | 2 | 2Rx | No limitation or ≤ X MHz | NRCA,ENDC | 25dB full range | Reuse of baseline architecture restricted to 2Rx/band but need 2LO frequencies |
|  | 2 | 2 |  | 2 | 2 | 2Rx |  |  |  |  |
| 3a | 1 | 4 shared |  | 4 | 4 | 4Rx | No limitation or ≤ X MHz | ENDC | 6<P≤25dB partial range | Reuse of baseline RFFE architecture adding RF split after 2 LNAs + 1BB/Rx => common AGC on LNA => 25dB only for some range |
|  | 2 |  |  | 2 | 2 | 2Rx |  |  |  |  |
| 3b | 1 | 4 shared |  | 4 | 4 | 4Rx | No limitation or ≤ X MHz | NRCA,ENDC | 6<P≤25dB partial range | Reuse of baseline RFFE architecture adding RF split after 2 LNAs + 1BB/Rx => common AGC on LNA => 25dB only for some range |
|  | 2 |  |  | 4 | 4 | 4Rx |  |  |  |  |
| 4a | 1 | 4 | 6 total | 4 | 4 | 4Rx | No limitation or ≤ X MHz | ENDC | 25dB full range | Requires 6 antennas and LNA => is it compatible with smartphone? (for which frequency range), FWA only |
|  | 2 | 2 |  | 2 | 2 | 2Rx |  |  |  |  |
| 4b | 1 | 4 | 8 total | 4 | 4 | 4Rx | No limitation or ≤ X MHz | NRCA,ENDC | 25dB full range | Requires 8 antennas and LNA => is it compatible with smartphone? (for which frequency range), FWA only |
|  | 2 | 4 |  | 4 | 4 | 4Rx |  |  |  |  |

Type 4a/4b UE has separated RF chain for each DL CC. The difference between the type 4a and type 4b lies in fact that the former one supports up to 2 DL MIMO layers in one of the CCs and the latter one supports to 4 DL MIMO layers on both CCs. The RF chains of Type 4 UE are not shared on different CCs. In RAN4#112 meeting, the MRTD and MTTD requirements for Type 4 capable UE in both non-collocated condition and collocated condition have drawn conclusions. One minor open issue for type 4 UE in non-collocated conditions is listed as below.

<Way Forward> Issue 1-3: Whether to introduce MRTD/MTTD requirements for non-collocated FR1 intra-band asynchronous NR-CA for Type 4 UE in Rel-19 - Proposals - Option 1: Not to introduce MRTD/MTTD requirements for non-collocated FR1 intra-band asynchronous NR-CA for Type 4 UE in Rel-19.

In our understanding, the same slot format is supposed to be indicated by UL-DL-configuration for the intra-band CA and the slot boundary shall be synchronized. So asynchronous NR-CA should not be considered.

Proposal 1: Not to introduce MRTD/MTTD requirements for non-collocated FR1 intra-band asynchronous NR-CA for Type 4 UE in Rel-19.

SCell BFD and CBD

Regarding the requirements of SCell BFD/CBD in non-collocated scenario, the channel quality on one serving cell may be different with that of the other non-collocated serving cell in the same band. So the number of serving cells per band for Type 4 UE operating in non-collocated scenario is supposed to more than 1 and up to 2 which is the same as Type 2 UE.

For UE supporting intraBandNR-CA-non-collocated-r18 and nonCollocatedTypeNR-CA-r18 is not provided for the configured FR1 intra-band non-contiguous CA, the requirements in this clause apply when UE is required to perform beam failure detection on no more than 2 serving cells per band if these 2 serving cells are in non-contiguous carriers, and no more than 1 serving cell per band otherwise.

Proposal 2: Type 4 UE when operating in non-collocated condition can be configured with BFD/CBD measurements on up to two serving cells in the same band.

In R18 intra-band non-collocated WI, besides MRTD and MTTD, the requirements including scheduling restriction, interruption, and SCell activation delay are impacted by Type 2 UE as well. Taking scheduling restriction during BFD as an example, two cases for Type 2 UE are considered. One case (Case#1) is when nonCollocatedTypeNR-CA-r18 is not provided, and the other case (Case#2) is when nonCollocatedTypeNR-CA-r18 is provided. In Case #1, UE operates as a Type 2 UE in non-collocated scenario. Case #2 is a collocated scenario, and UE falls back to Type 1 UE capability. The scheduling restriction are different in two cases (see below).

TS 38.133 When intra-band non-contiguous carrier aggregation is configured for a UE indicating intraBandNR-CA-non-collocated-r18 and if nonCollocatedTypeNR-CA-r18 is not provided, there are no scheduling restrictions on FR1 serving cell(s) configured on the non-contiguous CC(s) in the same band. Otherwise, the scheduling restrictions on one serving cell apply to all other serving cells in the same band on the symbols that fully or partially overlap with the restricted symbols if nonCollocatedTypeNR-CA-r18 is provided.

For R19 intra-band non-collocated WI, in RF main session at RAN4#112 meeting, There is a tentative agreement in RF session that RAN4 will introduce capability for type 4 UE for EN-DC and NR CA separately. However whether/how to differentiate type 4a and 4b is FFS.

In RRM session at last RAN4#112 meeting, when Type 4 capable UE operates in non-collocated condition, most of conclusions including interruption requirements, SCell activation delay and scheduling restriction have been already achieved. So the open issues are mainly for the case when Type 4 capable UE operates in collocated condition.

According to the latest RF progress at RAN4#112 meeting, it is agreed that no new network indication from Type-4 to Type-2 switch is needed [refer to main session chairman notes]:

Agreement:

- No new NW signaling needed from Type-4 to Type-2 switch and maxMIMO-layer can be used.

It is common understanding in RF session that the new BS signaling switching between type 4 and type 1 with 6/8 layer per CC would be introduced, and “ Type 4 fall back to Type 1 with 4Rx(max 4 MIMO layers/CC)” is FFS.

From RRM perspective, either Type 4 UE falling back to Type1 UE with 4Rx or falling back to Type 1 UE with 6/8RX operates with shared RF chain. Therefore when Type 4 capable UE operates in collocated condition, the UE behaviour is similar as Type 2 capable UE in collocated condition.

Interruption requirements

Interruption requirements care about the interruption length. When type 4 capable UE operates in collocated condition, the interruption length would be the same as type2 capable UE in collocated scenario in R18. It shall be noted that the R19 UE capability and network indication(s) would be accordingly updated.

Proposal 3: When type 4 capable UE operates in collocated condition, interruption requirements (below listed) defined for type 2 UE in collocated scenario in R18 can be applied

- Interruption at SCell addition/release

- Interruptions at SCell activation/deactivation

- Interruptions during measurements on deactivated SCC

Note: Some clarifications are needed with regard to the UE capability signalling and network indication(s).

SCell activation delay

Whether UE operates on being activated SCel with shared RF chain or sperate RF chain would impact the element TSMTC_MAX and TFirstSSB_MAX in Tactivation_time. So the below proposal is presented:

Proposal 4: When type 4 capable UE operates in collocated condition, SCell activation delay defined for type 2 UE in collocated condition in R18 can be applied.

Note: Some clarifications are needed with regard to the UE capability signalling and network indication(s).

Scheduling availability

Similar as above analysis, no further details are provided for scheduling availability.

Proposal 5: When type 4 capable UE operates in collocated condition, scheduling availability (below listed) defined for type 2 UE in collocated condition in R18 can be applied

- scheduling availability in RLM

- scheduling availability in BFD

- scheduling availability in CBD

- scheduling availability in measurement requirement in clause 9

Note: Some clarifications are needed with regard to the UE capability signalling and network indication(s).

Conclusions

In this paper we provided our analysis on RRM requirements for supporting intra-band non-collocated EN-DC/NR-CA deployment for Type 4 UE. The following proposals are provided.

Proposal 1: Not to introduce MRTD/MTTD requirements for non-collocated FR1 intra-band asynchronous NR-CA for Type 4 UE in Rel-19.

Proposal 2: Type 4 UE when operating in non-collocated condition can be configured with BFD/CBD measurements on up to two serving cells in the same band.

Proposal 3: When type 4 capable UE operates in collocated condition, interruption requirements (below listed) defined for type 2 UE in collocated scenario in R18 can be applied

- Interruption at SCell addition/release

- Interruptions at SCell activation/deactivation

- Interruptions during measurements on deactivated SCC

Note: Some clarifications are needed with regard to the UE capability signalling and network indication(s).

Proposal 4: When type 4 capable UE operates in collocated condition, SCell activation delay defined for type 2 UE in collocated condition in R18 can be applied.

Note: Some clarifications are needed with regard to the UE capability signalling and network indication(s).

Proposal 5: When type 4 capable UE operates in collocated condition, scheduling availability (below listed) defined for type 2 UE in collocated condition in R18 can be applied

- scheduling availability in RLM

- scheduling availability in BFD

- scheduling availability in CBD

- scheduling availability in measurement requirement in clause 9

Note: Some clarifications are needed with regard to the UE capability signalling and network indication(s).

Reference

- RP-240838, New WID on Support of intra-band non-collocated EN-DC/NR-CA deployment Phase2: new receiver type(s)

- R4-2217734, WF on NonCol_intraB_ENDC_NR_CA for New Type UE, KDDI

- R4-2413973, WF on RRM requirements for supporting intra-band non-collocated EN-DC/NR-CA deployment Phase2: new receiver type(s), Huawei, HiSilicon