3GPP TSG-RAN WG4 Meeting # 112bis	R4-2415842

Hefei, Anhui, China, 14th – 18th October, 2024

Title: 	Discussion on conditional Intra-CU LTM

Source: 	Huawei, HiSilicon

Agenda item:	6.24.2.2

Document for:	Discussion

Introduction

The revised R19 mobility WI was agreed in [1]. The objectives are duplicated as below.

- 4.1 Objective of SI or Core part WI or Testing part WI - Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support - Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1] NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure. - Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint - Specify RRM requirements related to the above objectives as necessary [RAN4]

This contribution provides analysis on intra-CU LTM from RAN4 RRM requirement perspective.

Discussion

Regarding the third objective conditional intra-CU LTM, according to the latest revised WID, the conditional LTM and subsequent conditional LTM will be supported. The analysis on the RRM requirements for conditional LTM and subsequent conditional LTM are provided in the followings.

Conditional LTM

Legacy conditional handover delay is defined as DCHO = TRRC + TEvent_DU + Tmeasure + Tinterrupt + TCHO_execution, where the starting point is from UE receives a RRC message implying conditional handover. After UE successfully decodes the RRC signaling, UE wait for the conditional conditions are met and then perform handover to target cell. The ending point of CHO delay is when UE start the transmission of the new uplink PRACH channel.

For conditional LTM, LTM cell switch related conditions are also signalled by RRC. So it is straight forward that the starting point of conditional LTM is the instance when the UE receives a RRC message implying conditional LTM.

And then UE can perform early L1 RSRP measurement, TCI activation or early PDCCH order RACH on candidate cells before LTM cell switch. If UE has already got the TA information in advance, RACH-less cell switch can be executed. So the ending point of conditional LMT shall include both RACH-based and RACH-less cases.

Proposal 1: The starting point of conditional LTM delay is the instance when the UE receives a RRC message implying conditional LTM.

Proposal 2: The ending point of conditional LTM delay is the time the UE transmits the first UL message on the target cell (including RACH based and RACH based case).

In legacy CHO delay, the component TEvent_DU and Tmeasure are specified for the time duration when the condition event is really met at UE side.

TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional handover command until - a condition exists at the measurement reference point which will trigger the conditional handover, or - a condition exists at the measurement reference point which will trigger the NES-based conditional handover

The measurement time delay is defined from the end of TEvent_DU until UE executes a handover to a target cell and interruption time starts. For conditional intra-frequency handover, the measurement time delay measured without Time To Trigger (TTT) and L3 filtering shall be less than Tidentify intra with index or Tidentify_intra_without_index defined in clause 9.2.5.1 or clause 9.2.6.2.

For conditional LTM, new conditions for triggering LTM conditions will be discussed in RAN2. As we know, for event reporting evaluation topic, RAN2 agreed that beam level measurement rather than cell level measurement is sued in LTM event evaluation and FFS on conditional LTM case.

Proposal 3: TEvent_DU and Tmeasure for conditional LTM can wait for more progress in RAN2.

As we know besides L1 measurement on candidate neighbor cells, UE can also execute early TCI state activation or early PRACH procedure in R18 LTM. PDCCH ordered PRACH on candidate neighbor cells is triggered by network. There could be a case that when the conditions are met in UE side, UE starts to switch to the target cell, however at the same time, network is triggering PDCCH ordered RACH. So it shall be figured out how/whether to support early RACH during conditional LTM. Likewise TCI activation on neighbor cells during conditional LTM procedure shall also wait for more progress from other RAN group.

Proposal 4:  Figure out how/whether to support early RACH/TCI activation on candidate cells during conditional LTM and wait for more progress from other working group.

In legacy TCHO_execution is the UE execution preparation time for conditional handover. This time duration originally called TRRC_2 for RRC processing time for target cell. As LTM command is MAC CE, TCHO_execution may need update.

Proposal 5: TCHO_execution may need update in conditional LTM.

In legacy CHO delay, Tinterrupt comprise Tprocessing + TIU + T∆ + Tmargin. In legacy CHO procedure, interruptions happen from UE starts to execute CHO to target cell. However in LTM, interruptions may also happen at early PRACH procedure. Therefore from terminology perspective, we suggest the term Tinterrupt  is not used when define conditional LTM delay.

Proposal 6: The term Tinterrupt  may induce ambiguity when define conditional LTM delay, as interruptions may also happen in early PRACH procedure (if supported in conditional LTM).

Subsequent conditional LTM

The requirements of subsequent CPA and subsequent CPC have been specified in R18. The delay for subsequent CPA/CPC is alike CHO delay in some extent. The difference is the starting point. In subsequent CPA/CPAC the starting point is from the time UE transmits RRC Reconfigurationcomplete message for the previous PSCell addition or change. For subsequent conditional LTM, in our understanding either RACH based or RACH-less cell switch, the starting point of the subsequent conditional cell switch delay can also reuse the point that UE transmits RRC Reconfigurationcomplete message for the previous PCell.

Proposal 7: The starting point of the subsequent conditional LTM cell switch delay is the time UE transmits RRC Reconfigurationcomplete message for the previous PCell.

Apart from the starting point, the subsequent conditional LTM cell switch delay can be discussed based on the conclusion of conditional LTM delay.

Proposal 8: Apart from the starting point, the subsequent conditional LTM cell switch delay can be discussed based on the conclusion of conditional LTM delay.

Conclusion

The contribution provides analysis on intra-CU LTM from RAN4 RRM requirement perspective. The following proposals are proposed:

Proposal 1: The starting point of conditional LTM delay is the instance when the UE receives a RRC message implying conditional LTM.

Proposal 2: The ending point of conditional LTM delay is the time the UE transmits the first UL message on the target cell (including RACH based and RACH based case).

Proposal 3: TEvent_DU and Tmeasure for conditional LTM can wait for more progress in RAN2.

Proposal 4:  Figure out how/whether to support early RACH/TCI activation on candidate cells during conditional LTM and wait for more progress from other working group.

Proposal 5: TCHO_execution may need update in conditional LTM.

Proposal 6: The term Tinterrupt  may induce ambiguity when define conditional LTM delay, as interruptions may also happen in early PRACH procedure (if supported in conditional LTM).

Proposal 7: The starting point of the subsequent conditional LTM cell switch delay is the time UE transmits RRC Reconfigurationcomplete message for the previous PCell.

Proposal 8: Apart from the starting point, the subsequent conditional LTM cell switch delay can be discussed based on the conclusion of conditional LTM delay.

Reference

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4

- R4-2406502, WF on NR mobility enhancements (part 1), MediaTek

- R4-2406513, Draft Big CR to TS 38.133 on core requirement maintenance for Further NR mobility enhancements, MediaTek Inc, Apple