3GPP TSG-RAN WG4 Meeting # 112bis	R4-2415845

Hefei, Anhui, China, 14th – 18th October, 2024

Title: 	Discussion on impacts of RAN4 requirements in AI/ML mobility

Source: 	Huawei, HiSilicon

Agenda item:	6.18.2

Document for:	Discussion

Introduction

The revised SID on AI/ML for mobility in NR agreed in [1]. The objectives are duplicated as below.

### 4.1 Objective of SI or Core part WI or Testing part WI The study will focus on mobility enhancement in RRC_CONNECTED mode over air interface by following existing mobility framework, i.e., handover decision is always made in network side. Mobility use cases focus on standalone NR PCell change. UE-side and network-side AI/ML model can be both considered, respectively. Study and evaluate potential benefits and gains of AI/ML aided mobility for network triggered L3-based handover, considering the following aspects: - AI/ML based RRM measurement and event prediction, - Cell-level measurement prediction including intra and inter-frequency (UE sided and NW sided model) [RAN2] - Inter-cell Beam-level measurement prediction for L3 Mobility (UE sided and NW sided model) [RAN2] - HO failure/RLF prediction (UE sided model) [RAN2] - Measurement events prediction (UE sided model) [RAN2] - Study the need/benefits of any other UE assistance information for the network side model [RAN2] - The evaluation of the AI/ML aided mobility benefits should consider HO performance KPIs (e.g., Ping-pong HO, HOF/RLF, Time of stay, Handover interruption, prediction accuracy, and measurement reduction) etc.) and complexity tradeoffs [RAN2] - NOTE: Simulation assumption and methodology can leverage TR 38.901, 38.843 and 36.839. And leave the detail discussion to RAN2 - Potential AI mobility specific enhancement should be based on the Rel19 AI/ML-air interface WID general framework (e.g. LCM, performance monitoring etc) [RAN2] - NOTE: This would only be treated after sufficient progress is made in the Rel-19 AI/ML air interface WID - Potential specification impacts of AI/ML aided mobility [RAN2] - Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4] - Study the impacts on requirements based on RAN2 assumptions, and coordinate with RAN2 if needed - Study the testability and interoperability based on RAN2 framework (e.g., number of cells to measure, beams etc.) - NOTE 4: Leverage the work from “AI/ML for NR air interface” led by RAN1 and avoid the duplicate study for testability and interoperability. - NOTE 5: Avoid the overlaps with RAN2 work for evaluation - NOTE 1: RAN1/3 work can be triggered via LS NOTE 2: To avoid duplicate study with “AI/ML for NG-RAN” led by RAN3 NOTE 3: Two-sided model is not included

This contribution provides preliminary analysis on RAN4 RRM requirements in AI/ML mobility.

Discussion

Potential RRM impacts

In R18 study item phase on “AI/ML for NR air interface”, the general functional framework for AI/ML for NR air interface is illustrated in below figure (duplicated from TR38.843[3]). The functional architecture and high-level procedure can be regarded as a baseline for AI/ML aided mobility.

Figure 4.4-1: Functional framework for AI/ML for NR Air Interface [TR 38.843]

There is a parallel R19 WI on “AI/ML for NR air interface” which focus on use cases of CSI compression, beam management and positioning. As per the revised SID on AI/ML mobility [1], the work from “AI/ML for NR air interface” shall be leveraged as much as possible.  In RAN4 progress on R19 AI/ML for NR air interface, the following agreements are achieved in [2]. These agreements provide a good guidance on RRM core requirements definition.

Issue 1-4: Inference latency handling Agreement: - Use the following as the starting point - Inference latency can be implicitly captured in the delay of corresponding procedures (e.g. reporting delay) and/or performance requirements. - Inference latency is part of delay of the corresponding procedure and no separate requirement will be specified for inference latency. - Check the above bullets based on per use case, and if other WGs define the inference latency, RAN4 can revisit the above bullets. Issue 1-5: LCM Requirements Agreement: - RAN4 considers defining RRM requirements for LCM procedures based on the specific use cases (beam management and positioning), and candidates are: - identification - selection - activation - deactivation - switching - fallback to non-AI operation - performance monitoring - Others are not precluded Issue 1-6: Legacy RRM requirements handling Agreement: - UE shall meet all legacy RRM requirements (non-AI/ML) even during the AI/ML operation mode.

For R19 AI/ML based mobility, there are three case cases covering RRM measurement prediction, measurement event prediction and RLF/HOF prediction for PCell change procedure. There are two study goals. The 1st study goal is to reduce measurement efforts in temporal, spatial or frequency domain by using predicted measurements. The 2nd study goal is to improve the handover performance (e.g., Ping-pong HO, HOF/RLF, short time of stay, Handover interruption) [4]. In the following, we try to analysis the potential RRM impacts for each use case.

RRM measurement prediction

- Data collection requirements

Data collection function provides input data to model training, management and inference function. If new measurement quantity is introduced in AI/ML mobility, new requirements would be specified accordingly, otherwise no RRM requirements on data collection is observed.

Proposal 1: If new measurement quantity is introduced in AI/ML mobility, new requirements for data collection may be specified accordingly, otherwise no RRM requirement is observed.

- Inference requirements

In RAN2, the priority of evaluation scenarios for measurement prediction is listed in [TR38.744]. RAN4 can also focus on the high priority scenarios at first.

Table 5.2.1-1 prioritization of evaluation scenarios [TR38.744]

| Case number | Prioritization | Evaluation scenario combination | target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

where case A and case B in intra-frequency temporal domain are roughly defined as below in RAN2.

Agreements in RAN2#126 Chairman notes 7 To agree on methodology of Intra_F_C_T_Case A as following: Intra-frequency intra-cell temporal domain case A prediction is done by predicting measurement result(s) in prediction window based on measurement results in observation window of the same cell for both FR1_to_FR1 and FR2_to_FR2. FFS aligning the prediction window 8 To agree on methodology of Intra_F_C_T_Case B: Intra-frequency intra-cell temporal domain prediction is done by predicting sub set measurement (case B) instances in temporal domain of the same cell for both FR1_to_FR1 and FR2_to_FR2. Several measurement reduction rates should be aligned among companies. The detail values for measurement reduction rate are FFS.

We know case A and case B have different working mechanism.

Figure 1. Intra-frequency intra-cell temporal domain prediction case A

In Intra-cell temporal domain prediction case A, observation window and prediction window are introduced [5]. Observation window is a duration UE performs actual measurement. Within a observation window, the measurement period shall follow the legacy measurement period. The prediction window is a duration UE predicts measurement. Observation window and prediction window will slide. It means that the instances in previous prediction window would become part of the observation window when UE perform measurement in predicted instances. It shall be noted the observation window length depends on model performance, channel environment, observation length, etc. Recalling the evaluation period in RLM which is alike a sliding window, a similar evaluation window for predication may be needed.

It is straight forward that the prediction would result in measurement result difference between the actual measured RSRP and predicted RSRP. The prediction accuracy would be evaluated and defined in RAN4 RRM.

Proposal 2: For intra-frequency Intra-cell temporal domain prediction case A, prediction window length (or called as prediction evaluation period) may need to be discussed and specified.

Proposal 3: For intra-frequency temporal domain prediction case, the RSRP prediction accuracy would be specified.

Figure 2. Intra-frequency intra-cell temporal domain prediction case B

In Intra-cell temporal domain prediction case B, some measurements are predicted and these time instances are completely skipped by UE. There may be many skipping patterns where some instances are skipped as illustrated in Figure 2. The periodicity (T) of the required actual measurement for deducing the predicted measurements may be different. To avoid many skipping pattern combinations, Measurement Reduction Rate is used for case B in RAN2 evaluation work.

MRRT = skipped measurement time instances / total measurement time instances

In our understanding, to ensure network and UE have the same understanding on the measurement prediction, the skipping pattern may be indicated by network. Then the MRRT is known to both UE and network. Under different MRRT (e.g., 1:1, 2:3), the prediction accuracy may be different. RAN4 may need to specify prediction accuracy under the side condition of MRRT. It shall be noted, some limitation on the prediction length may also be needed. For example, even with the same MRRT, e.g. 1:1=1, 100:100=1, the prediction accuracy of the sample in the farthest instance in the latter case (100:100) may be degraded. So one MRRT is not able to cover all kinds of skipping patterns. The accuracy requirements are to be specified for certain MRRT with limited prediction length.

Proposal 4: For intra-frequency Intra-cell temporal domain prediction case B, RSRP prediction accuracy under the side condition of MRRT may need to be discussed and specified. Certain limitation on the prediction length may also need to be considered.

For inter-frequency inter-cell frequency domain measurement, the idea is to leverage the RSRP result on one cell to infer the RSRP on another cell in another frequency. The prediction accuracy in frequency domain would be also evaluated and defined in RAN4 RRM. Even with frequency prediction model, it is not possible that the measurement on any frequency layer can be inferred. Therefore side conditions, e.g., EPRE difference, need to be specified.

Proposal 5: For inter-frequency inter-cell frequency domain prediction, the side condition of frequency prediction (e.g., EPRE difference) and RSRP prediction accuracy would be specified.

The legacy measurement reporting delay is defined as a delay for UE transmitting the measurement report. In AI/ML mobility, the inference function used for prediction may need some times. Therefore the processing time of inference function may need to be considered in measurement reporting delay.

The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH. This measurement reporting delay excludes a delay which caused by no UL resources being available for UE to send the measurement report on.

Proposal 6: The processing time of inference function may need to be considered in measurement reporting delay.

- LCM requirements

In management function (LCM procedure), the functionality/model activation/deactivation/fallback to non-AI function are to be considered. If the functionality/model activation/deactivation/fallback to non-AI function are indicated by network, e.g., MAC CE/DCI triggered, the delay of activating/deactivating one function would be defined. The side conditions of activation/deactivation/fallback delay may also be discussed. Otherwise if these functionality/model changes are autonomously performed by UE, UE monitors the function performance, then the requirements would be related with monitoring requirements.

Proposal 7: If the functionality/model activation, deactivation, or fallback to non-AI function are indicated by network, the activation/deactivation/fallback delay and corresponding side conditions would need to be specified.

Proposal 8: If the functionality/model activation, deactivation, or fallback to non-AI function are autonomously performed by UE, the requirements would be related with monitoring requirements.

- Performance Monitoring requirements

If performance monitoring is mainly executed at UE side, some metrics are to be defined so that UE itself can make decision of activation, deactivation, or fallback to non-AI. Then from RRM requirement perspective, the requirements of monitoring periodicity or reporting delay when certain metric is met may need to be specified.

If performance monitoring is executed at network, the monitoring function is up to network implementation. If new measurement quantity is introduced, new requirements would be specified, otherwise no RRM requirements on monitoring is observed.

As performance monitoring are under discussion in RAN1 R19 AI/ML air interface, RAN4 can wait for more progress.

Proposal 9: Wait for more progress on performance monitoring from RAN1 R19 AI/ML air interface.

RRM event prediction

Measurement event prediction was ever preliminarily discussed in RAN2#125bis meeting. The following general agreements are made.

Measurement event prediction - At least measurement event evaluation based on RRM measurement prediction result will be studied. Direct measurement event prediction are is also allowed. - Clarifications on what is being as input should be provided with results - Start with A3 as a baseline. - Measurement event prediction study can start after some further progress on RRM measurement prediction has been made

In the subsequent RAN2#126 and RAN2#127 meeting, the agenda item was not treated. Due to limited input from RAN2 on this item, we think RAN4 can wait for more progress on measurement prediction.

Proposal 10: Measurement event prediction study can wait for more progress on measurement prediction.

RLF/HO failure prediction

RLF detection is prioritized in RAN2, and HOF is deprioritized. Both indirect RLF prediction and direct RLF are to be discussed. SINR in temporal domain based on measurement prediction is used for indirect RLF prediction. So the RRM impacts identified for measurement prediction in temporal domain can be used as a starting point for indirect RLF prediction.

In RAN2 the evaluation work and simulation results of RLF are not expected before February. So RLF study is also not hurry in RAN4.

Agreements in RAN2#126 1 Study Indirect: RLF prediction based on the temporal domain serving cell measurement predictions (e.g. SINR). 2 Study Direct: Directly RLF prediction by AI/ML models. 3 FR2 study will be prioritized for RLF prediction 4 The study should focus on RLF due to T310 expiry (i.e. in-synch/out-of-synch case) as the representative RLF case for direct and indirect prediction. 5 HOF prediction is downprioritized in our study. NO simulations/evaluations should be done/submitted 6 RLF prediction result is the RLF probability within a time window or at time instance, at least for direct case. FFS on expected RLF time and indirect case. 7 No evaluation/simulations are expected for August meeting for RLF 8 Simulation assumption specific to RLF will be discussed in August. The assumption is that we will reuse RRM simulation assumptions (where possible).

Agreements in RAN2#127 - Both direct and indirect are allowed. Companies should indicate what they used and what inputs they are using - Output for indirect: predicted SINR. Based on predicted SINR the time instance the RLF occurs can be determined without further AI/ML models. - Output for direct: probability of RLF within an window - Companies should report the prediction time window they have used in the simulations Agreements on KPI For indirect - SINR difference, missed RLF detection, false RLF detection, F1 score, time difference of true time RLF and predicted RLF, true RLF prediction. Direct - missed RLF detection, false RLF detection, F1 score. - Whether the RLF will happen based on the following methodology. Above a probability threshold we assume that RLF will happen. This is then compared with true RLF. - For the time being we don’t need HO procedure simulation in RLF simulation - FFS full buffer and assumption that all the cells are fully loaded. We will not simulate traffic. - Simulation results are not expected before February

Proposal 11:  The RRM impacts identified for measurement prediction in temporal domain case A can be used as a starting point for indirect RLF prediction, where RSRP is replaced by SINR.

Study of RAN2 assumption

It is observed that the RAN2 performed simulations for verifying the RSRP difference in AI RRM measurement prediction use case. Both the training dataset and inference dataset are generated from system level simulations. The prediction accuracy metric for RRM measurement cell level prediction is defined as “RSRP difference between predicted L3 cell level measurement result and actual L3 cell level measurement result of the same cell” for all RRM sub cases. We identify several aspects where there may be differences between RAN2 assumption and RAN4 legacy.

- Issue-1 Model input

For cell level measurement prediction model, RAN2 considers the following cases:

- Case 1: To predict beam level results, then generate cell level results based on the predicted beam results;

- Case 2: To directly predict cell level results based on cell level results.

- Case 3: To directly predict cell level results based on beam level results

In Case 1 and Case 3, it is up to company’s implementation whether L1 filtering is applied for L1 beam level measurement. Different L1 filtering methods are observed from companies reporting. For L3 filtering, a fixed value is considered as the starting point.

In all the cases, ideal RSRP is used as the model input, without considering the measurement impairment. Since in practical, only the non-ideal RSRP can be obtained, the impact of the measurement impairment needs to be captured when we verify the actual prediction accuracy enabled by AI.

Observation 1: Measurement impairment may impact the prediction accuracy.

Observation 2: The RSRP with measurement impairment can be obtained in reality, rather than the ideal RSRP.

Observation 3: In RAN2 assumption, the ideal RSRP is assumed both for model input and the label of model output.

Proposal 12: RAN4 to discuss the measurement impairment considered for verifying the actual prediction accuracy enabled by AI.

If the measurement impairment is agreed to be considered during model training, RAN4 needs to perform link level simulation to determine the RSRP measurement impairment in different SNR values.

Proposal 13: RAN4 can perform link level simulation to determine the RSRP measurement impairment in different SNR values if the measured RSRP is considered as model output.

- Issue-2 Model output

Figure 9.2.4-1: Measurement Model [in TS38.300]

In RAN4 legacy, the requirement of prediction accuracy is defined at point A1. While in RAN2, the prediction accuracy is defined at point C. In Case 1, it is up to companies to report L1 RSRP difference.

Observation 4: In RAN4 legacy, there is no requirement of prediction accuracy in point C, point A1 instead. In AI mobility, RAN2 uses the RSRP difference at point C as the prediction accuracy.

Proposal 14: RAN4 will discuss whether to define prediction accuracy in point C.

- Issue-3 Training and testing dataset

In RAN4 legacy, only link level channel model is considered for defining test cases. In RAN2, the AI model is trained by system level channel model and verified under the same channel model. There are three potential testing options for RAN4 in AI RRM measurement prediction.

  - Option 1: Use the prediction accuracy aligned in RAN2 to define the requirement in RAN4. The testing dataset is generated by link level channel model.

  - Option 2: Use system level channel model to generate the training dataset and train the AI prediction model, align the prediction accuracy in link level channel model to define the requirement in RAN4.

  - Option 3: Use link level channel model to generate the training dataset and train the AI prediction model, align the prediction accuracy in the same link level channel model to define the requirement in RAN4.

Proposal 15: RAN4 will study the following three potential options for defining prediction accuracy, training and testing dataset.  Pros and Cons need to be analyzed.

  - Option 1: Use the prediction accuracy aligned in RAN2 to define the requirement in RAN4. The testing dataset is generated by link level channel model.

  - Option 2: Use system level channel model to generate the training dataset and train the AI prediction model, align the prediction accuracy in link level channel model to define the requirement in RAN4.

  - Option 3: Use link level channel model to generate the training dataset and train the AI prediction model, align the prediction accuracy in the same link level channel model to define the requirement in RAN4.

There are two options for defining the testing and training dataset. One is using the channel model and the sampling rules. Another is that the dataset is specified or partially specified in a sample-by-sample manner.

Proposal 16: RAN4 will study the following two potential options for defining the testing and training dataset.

  - Option 1: Use the channel model and the sampling rules.

  - Option 2: Specify or partially specify the testing dataset in a sample-by-sample manner.

Conclusion

The contribution provides analysis on on RAN4 RRM requirements in AI/ML mobility. The following proposals are proposed:

Proposal 1: If new measurement quantity is introduced in AI/ML mobility, new requirements for data collection may be specified accordingly, otherwise no RRM requirement is observed.

Proposal 2: For intra-frequency Intra-cell temporal domain prediction case A, prediction window length (or called as prediction evaluation period) may need to be discussed and specified.

Proposal 3: For intra-frequency temporal domain prediction case, the RSRP prediction accuracy would be specified.

Proposal 4: For intra-frequency Intra-cell temporal domain prediction case B, RSRP prediction accuracy under the side condition of MRRT may need to be discussed and specified. Certain limitation on the prediction length may also need to be considered.

Proposal 5: For inter-frequency inter-cell frequency domain prediction, the side condition of frequency prediction (e.g., EPRE difference) and RSRP prediction accuracy would be specified.

Proposal 6: The processing time of inference function may need to be considered in measurement reporting delay.

Proposal 7: If the functionality/model activation, deactivation, or fallback to non-AI function are indicated by network, the activation/deactivation/fallback delay and corresponding side conditions would need to be specified.

Proposal 8: If the functionality/model activation, deactivation, or fallback to non-AI function are autonomously performed by UE, the requirements would be related with monitoring requirements.

Proposal 9: Wait for more progress on performance monitoring from RAN1 R19 AI/ML air interface.

Proposal 10: Measurement event prediction study can wait for more progress on measurement prediction.

Proposal 11:  The RRM impacts identified for measurement prediction in temporal domain case A can be used as a starting point for indirect RLF prediction, where RSRP is replaced by SINR.

Observation 1: Measurement impairment may impact the prediction accuracy.

Observation 2: The RSRP with measurement impairment can be obtained in reality, rather than the ideal RSRP.

Observation 3: In RAN2 assumption, the ideal RSRP is assumed both for model input and the label of model output.

Proposal 12: RAN4 to discuss the measurement impairment considered for verifying the actual prediction accuracy enabled by AI.

Proposal 13: RAN4 can perform link level simulation to determine the RSRP measurement impairment in different SNR values if the measured RSRP is considered as model output.

Observation 4: In RAN4 legacy, there is no requirement of prediction accuracy in point C, point A1 instead. In AI mobility, RAN2 uses the RSRP difference at point C as the prediction accuracy.

Proposal 14: RAN4 will discuss whether to define prediction accuracy in point C.

Proposal 15: RAN4 will study the following three potential options for defining prediction accuracy, training and testing dataset.  Pros and Cons need to be analyzed.

  - Option 1: Use the prediction accuracy aligned in RAN2 to define the requirement in RAN4. The testing dataset is generated by link level channel model.

  - Option 2: Use system level channel model to generate the training dataset and train the AI prediction model, align the prediction accuracy in link level channel model to define the requirement in RAN4.

  - Option 3: Use link level channel model to generate the training dataset and train the AI prediction model, align the prediction accuracy in the same link level channel model to define the requirement in RAN4.

Proposal 16: RAN4 will study the following two potential options for defining the testing and training dataset.

  - Option 1: Use the channel model and the sampling rules.

  - Option 2: Specify or partially specify the testing dataset in a sample-by-sample manner.

Reference

- RP-242393, Revised SID on AIML for mobility in NR, OPPO

- R4-2401570, WF on AI/ML, Qualcomm Incorporated

- TR38.843

- TR38.744

- R2-2405941，Summary of [POST125bis][021][AIML mobility] Simulation assumptions and methodology， OPPO