3GPP TSG-RAN WG4 Meeting #112-bis	R4-2415846

Hefei, China, 14 – 18 October, 2024

Title: 	Discussion on testability and interoperability issues in AIML mobility

Source: 	Huawei, HiSilicon

Agenda item:	6.18.3

Document for:	Discussion

Introduction

According to the revised SID on AIML for mobility in [1], RAN4 will start the study on AI mobility from this meeting until August 2025. In this paper, we discuss the general aspects in AIML mobility.

Discussion

Revised SID on AIML for mobility [1]: Study and evaluate potential benefits and gains of AI/ML aided mobility for network triggered L3-based handover, considering the following aspects: - AI/ML based RRM measurement and event prediction, - Cell-level measurement prediction including intra and inter-frequency (UE sided and NW sided model) [RAN2] - Inter-cell Beam-level measurement prediction for L3 Mobility (UE sided and NW sided model) [RAN2] - HO failure/RLF prediction (UE sided model) [RAN2] - Measurement events prediction (UE sided model) [RAN2]

For mobility enhancement, AI is introduced to predict the RRM measurement and event prediction. The identified use cases are AI based cell-level/beam-level measurement prediction, AI based HO failure/RLF prediction and AI based measurement event prediction.

Revised SID on AIML for mobility [1]: - Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4] - Study the impacts on requirements based on RAN2 assumptions, and coordinate with RAN2 if needed - Study the testability and interoperability based on RAN2 framework (e.g., number of cells to measure, beams etc.) - NOTE 4: Leverage the work from “AI/ML for NR air interface” led by RAN1 and avoid the duplicate study for testability and interoperability. - NOTE 5: Avoid the overlaps with RAN2 work for evaluation

For the identified use cases, the testability, interoperability, and impacts on RRM requirements and performance will be studied in RAN4.

For AI based cell-level/beam-level measurement prediction, RAN4 can study the testability of new reporting. The potential new reporting can be the predicted measurement.

For AI based AI based HO failure/RLF prediction and AI based measurement event prediction, if the eventual prediction is derived from predicted measurement, RAN4 can also study the testability of predicated measurement reporting.

For cell level measurement prediction model, RAN2 considers the following cases:

•	Case 1: To predict beam level results, then generate cell level results based on the predicted beam results;

•	Case 2: To directly predict cell level results based on cell level results.

•	Case 3: To directly predict cell level results based on beam level results.

In the following, we will discuss the testability in each case.

Testability of Case 1 and Case 3

It is observed that the testability issues in Case 1 and Case 3 is similar in that of AI enabled BM in [2], since the model input is the beam-level measurement. According to the latest progress in [3], the testability is still under discussion. The open issues are shown as follows.

RAN4 #112 WF [3]: Issue 2-3: Channel models Agreement: Further discuss and down-select the following options considering TE feasibility - Option 2: TDL - Option 5: simplified CDL - CDL channel model with a small number of clusters. CDL simplification needs to be further discussed Issue 2-4: Test setup needs - Issues listed below to be further investigated in order to determine the capabilities of the test equipment: - set B Tx beams; set A Tx beams - number of beams transmitted simultaneously - overall number of beams transmitted during the test - AoA - AoD - UE rotation during the test Other aspects can also be considered

In Case 1 and Case 3, the numbers of beams transmitted simultaneously and the overall number of beams transmitted during the test both need to check the capability of the test equipment. In order to avoid duplicate discussion, we proposed to postpone the discussion to wait the progress of the testability study in AI-BM.

Proposal 1: Postpone the testability discussion of Case 1 (i.e., to predict beam level results, then generate cell level results based on the predicted beam results) and Case 3 (i.e., to directly predict cell level results based on beam level results) to wait the progress of the testability study in AI-BM.

Testability of Case 2

In Case 2, since there is no beam-level measurement related to the model input and the model output, the test setup can reuse legacy if the number of cells is smaller than 2.

Proposal 2: RAN4 to consider whether to reuse the test setup in legacy for Case 2 (i.e., to directly predict cell level results based on cell level results).

Conclusions

In this paper we provided our views on testability and interoperability issues in AI mobility.

Proposal 1: Postpone the testability discussion of Case 1 (i.e., to predict beam level results, then generate cell level results based on the predicted beam results) and Case 3 (i.e., to directly predict cell level results based on beam level results) to wait the progress of the testability study in AI-BM.

Proposal 2: RAN4 to consider whether to reuse the test setup in legacy for Case 2 (i.e., to directly predict cell level results based on cell level results).

Reference

- R4-2410570, “Revised SID on AIML for mobility in NR”, 3GPP RAN#105, September 9-12, 2024.

- RP-242399, “Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, RAN #105, September 9-12, 2024.

- R4-2406617, “WF on AI/ML”, 3GPP RAN4 #110bis, 15th – 19th April, 2024.