3GPP TSG-RAN WG4 Meeting #112-bis	R4-2415850

Hefei, China, 14 – 18 October, 2024

Title: 	Discussion on core requirements for RedCap positioning and PRS/SRS BWA

Source: 	Huawei, HiSilicon

Agenda item:	4.2.1.4

Document for:	Discussion

Introduction

Maintenance of RRM core requirements for RedCap positioning and PRS/SRS BWA are discussed in RAN4#112, and the outcomes are captured in [1]. Further discussions are needed for the following issues.

- RedCap positioning

- PRS/SRS BWA

In this paper, we will provide our views on remaining issues in core requirements for RedCap positioning and PRS/SRS BWA.

Discussion

RedCap positioning

Issue 2-1: Requirement on N_(hops,effect)for RedCap positioning core requirements Proposals: - Option 1: the parameter NrepPRS shall take into account the number of unmuted inter-slot PRS repetitions that overlap with the sampling window within each hop - Option 2: Nhops,effect shall take into account the number of unmuted inter-slot PRS repetitions that overlap with the sampling windows of the hopping pattern with Nhopsslot hops per slot, within a single MG occasion, excluding the gap retuning times. - Option 3: Other options are not precluded.

RAN4 discussed how to update the requirements on Nhops,effect, which are defined based on PRS repetition. There are two issues with the current requirements

- It is not aligned with the current definition of PRS repetition

- It does not work with PRS muting option 2

Based on the discussion, we still see our proposal in last meeting [2] as the most accurate way to define Nhops,effect, which is reproduced below.

Proposal 1: Update the requirement on Nhops,effect as Nhops,effect=hminNrep,hPRS,Nrepmax where - h is the index of hops within an MG instance - Nrep,hPRS is the number of unmuted PRS repetitions that overlaps with the sampling window of hop h, - Nrepmax=2 if Nhopsslot = 2, otherwise Nrepmax=1.

On the other hand, we can understand the concern from other companies that it may cause big spec change. Among option 1 and option 2, the difference is that option 2 refers to number of hopping patterns per slot. However, the hopping pattern may not be aligned with slot timing, as “the first hop within a MG instance starts at no earlier than the earliest arrival time of the first unmuted PRS resource”, so instead of referring to number of hopping patterns per slot, the requirement should refer to the length of the hopping pattern Thop.

Proposal 1: Nhops,effect is defined as the number of unmuted inter-slot PRS repetitions that overlap with the sampling windows of the hopping patterns with length Thop, within a single MG occasion, excluding the gap retuning times.

PRS/SRS BWA

Issue 3-3: Core requirements for PRS aggregation Proposals - When UE is indicated by LMF in nr-DL-PRS-JointMeasurementRequest-r18 to perform measurements by aggregating resources from multiple PFLs - Option 1: UE only measures resources indicated by LMF as linked resources. - Remove requirement on Tnon-aggregate from the existing core requirements for PRS aggregation. - Option 2: UE measures resources that are indicated by LMF as linked resources and resources that are not indicated by LMF as linked resources.

Our understanding is option 2.

First, from spec point of view, based on 37.355 in measurement report UE is allowed to indicate whether a measurement is based on aggregated DL-PRS Resources or not. However, following option 1, the measurement can only be based on aggregated DL-PRS Resources when UE is indicated with nr-DL-PRS-JointMeasurementRequest-r18, and there is no need to have this indicator. Therefore, we believe it is RAN1/2 intention that UE also performs measurement on resources that are not indicated by LMF as linked resources, i.e. option 2.

NR-DL-TDOA-MeasElement-r16 ::= SEQUENCE {

dl-PRS-ID-r16					INTEGER (0..255),

nr-PhysCellID-r16				NR-PhysCellID-r16								OPTIONAL,

nr-CellGlobalID-r16				NCGI-r15										OPTIONAL,

nr-ARFCN-r16					ARFCN-ValueNR-r15								OPTIONAL,

nr-DL-PRS-ResourceID-r16		NR-DL-PRS-ResourceID-r16						OPTIONAL,

nr-DL-PRS-ResourceSetID-r16		NR-DL-PRS-ResourceSetID-r16						OPTIONAL,

nr-TimeStamp-r16				NR-TimeStamp-r16,

nr-RSTD-r16						CHOICE {

k0-r16						INTEGER (0..1970049),

k1-r16						INTEGER (0..985025),

k2-r16						INTEGER (0..492513),

k3-r16						INTEGER (0..246257),

k4-r16						INTEGER (0..123129),

k5-r16						INTEGER (0..61565),

...,

kMinus6-r18					INTEGER (0..126083073),

kMinus5-r18					INTEGER (0..63041537),

kMinus4-r18					INTEGER (0..31520769),

kMinus3-r18					INTEGER (0..15760385),

kMinus2-r18					INTEGER (0..7880193),

kMinus1-r18					INTEGER (0..3940097)

},

……

[[

nr-MeasBasedOnAggregatedResources-r18		ENUMERATED {true}					OPTIONAL,

nr-AggregatedDL-PRS-ResourceInfo-List-r18	SEQUENCE (SIZE (2.. 3)) OF

NR-AggregatedDL-PRS-ResourceInfo-Element-r18		OPTIONAL,

……

]]

}

nr-MeasBasedOnAggregatedResources This field indicates whether the RSTD measurement, RSRP measurement (if included), and RSRPP measurement (if included) is based on aggregated DL-PRS Resources.

nr-AggregatedDL-PRS-ResourceInfo-List This field provides the DL-PRS Resource Set IDs which are used for the aggregated RSTD, RSRP, or RSRPP measurement results. This field is optionally present if the field nr-MeasBasedOnAggregatedResources is present; otherwise, it is not present. If the field is present, the field nr-DL-PRS-ResourceID and nr-DL-PRS-ResourceSetID should not be included, and the dl-PRS-ID in IE NR-DL-TDOA-MeasElement shall be ignored by a receiver. The aggregatedDL-PRS-ID in the nr-AggregatedDL-PRS-ResourceInfo-List is always provided for the first measurement when nr-AggregatedDL-PRS-ResourceInfo-List is included in NR-DL-TDOA-MeasElement. The aggregatedDL-PRS-ID is not present in the nr-AggregatedDL-PRS-ResourceInfo-List when nr-AggregatedDL-PRS-ResourceInfo-List is included in NR-DL-TDOA-AdditionalMeasurementElement.

Second, from NW point of view, option 1 is too limiting. It means once LMF request UE to do aggregated measurement, all the TRPs must transmit PRS resources in aggregated manner, i.e. to transmit 2 or 3 PFLs with a single Tx chain, as otherwise UE would not measure those TRPs not fulfilling the condition. This may not be easy in real deployment, as each TRP may have its own consideration in determining how to transmit PRS, e.g. some TRPs may transmit 2 PFL while other TRPs may transmit only one PFL.

Based on above, we suggest RAN4 to stick to the existing agreement, i.e. option 2.

Proposal 2: RAN4 to stick to the agreed assumption for defining PRS BWA requirements, i.e. option 2: UE measures resources that are indicated by LMF as linked resources and resources that are not indicated by LMF as linked resources.

In requirements for PRS BWA, Taggregate_RSTD is defined as follows. There is a margin for UE to switch across different PFL combinations, which is defined as (G-1)*maxTeffect_agg,i. However, using maxTeffect_agg,i means UE has to measure all PFL combinations for aggregated measurement before it can measure single PFLs for non-aggregated measurement (or vice versa), since the switching margin as highlighted in yellow does not allow UE to switch between PFL combinations and single PFLs.

Taggregate_RSTD, total measurement period for RSTD measurements performed by UE by aggregating PRS resources from multiple PFLs as defined in TS 38.214 [26], is defined as: Taggregate_RSTD=i=1GTRSTD_aggregate,i +(G-1)*maxTeffect_agg,i,

In our view, such restriction is unreasonable, and it should be up to UE implementation to decide the order to measure each PFL combination and each PFL. To allow this flexibility, we suggest to replace maxTeffect_agg,i by Tmargin in current requirements which is for switching between any PFL combination and any single PFL.

Tmargin is delay margin to account for delay between RSTD measurement performed by UE on PFLs that contain PRS resources for aggregation and RSTD measurement performed by UE on PFLs that do not contain PRS resources for aggregation. Tmargin is calculated as max(Teffect,i), by considering both aggregated PFLs and non-aggregated PFLs configured for positioning measurement. Tmargin is only applicable when UE is configured to perform measurements on PFLs that contain PRS resources for aggregation and on PFLs that do not contain PRS resources for aggregation.

Proposal 3: In PRS BWA requirements for Taggregate_RSTD, replace the margin (G-1)*maxTeffect_agg,i as (G-1)*Tmargin.

Conclusions

In this paper we provided provide our views on remaining issues in core requirements for RedCap positioning and PRS/SRS BWA.

Proposal 1: Nhops,effect is defined as the number of unmuted inter-slot PRS repetitions that overlap with the sampling windows of the hopping patterns with length Thop, within a single MG occasion, excluding the gap retuning times.

Proposal 2: RAN4 to stick to the agreed assumption for defining PRS BWA requirements, i.e. option 2: UE measures resources that are indicated by LMF as linked resources and resources that are not indicated by LMF as linked resources.

Proposal 3: In PRS BWA requirements for Taggregate_RSTD, replace the margin (G-1)*maxTeffect_agg,i as (G-1)*Tmargin.

Reference

- R4-2414032, WF on RedCap positioning and PRS/SRS bandwidth aggregation, Ericsson

- Discussion on RedCap positioning and PRS/SRS BWA