3GPP TSG-RAN WG4 Meeting #112-bis	R4-2415854

Hefei, China, 14 – 18 October, 2024

Title: 	Discussion on performance requirements for SL positioning

Source: 	Huawei, HiSilicon

Agenda item:	4.2.2.2

Document for:	Discussion

Introduction

Maintenance of RRM performance requirements for SL positioning are discussed in RAN4#112, and the outcomes are captured in [1]. Based on [1], further discussions are needed for the test case.

In this paper, we will provide our views on remaining issues in performance requirements for SL positioning.

Discussion

### Issue 1-2-6: SL-PRS signal level configurations Agreements: - Check the RAN1 specification and the Uu TCs in relation to overlapping SL-PRS from different UEs. Further discuss the options: - Option 1: overlapping SL-PRS can be used in TCs. - Option 2: overlapping SL-PRS should not be used in SL TCs.

RAN4#111 agreed on some test configurations for SL PRS measurement, especially the number of UEs. There are also some other test configurations that need to finalized.

On resource pool types, RAN4 agreed to define TCs for dedicated resource pool and shared resource pool, but it is unclear from the TC list how the resource pool types are considered. To not increase the number of TCs, the pool type can be randomly distributed among different TCs.

For the Es/Iot, we suggest to use 3dB for the test although we suggest -3dB for the accuracy requirements. The reason is that based on demodulation requirements in 38.101, the PSCCH performance is defined at Es/Iot of 4.7dB. If we define the SL PRS test at -6dB, UE may miss the SCI thus not measure SL PRS. Since the test is to verify SL PRS performance, a better Es/Iot should be used to ensure 100% PSCCH decoding.

In last meeting there was also a discussion on whether to use overlapping SL-PRS in the TC. Our view is that overlapping SL-PRS should not be used as it is not used in Uu PRS TCs. For example, in existing RSTD delay TC in A.6.6.12.1, the PRS configuration for the 3 TRPs are as follows.

Table A.6.6.12.1.1-2 (excerpt): General test parameters for RSTD measurement reporting delay

| PRS Configuration | Config 1 |  | PRS.1.1 FR1 | As specified in clause A.3.31 |
| --- | --- | --- | --- | --- |
|  | Config 2 |  | PRS.1.1 FR1 |  |
|  | Config 3 |  | PRS.2.1 FR1 |  |
| PRS muting info |  |  | Cell 1: ‘10’ Cell 2: ‘01’ Cell 3: ‘10’ | Correponds to prs-MutingInfo defined in TS 37.355 [34] |
| PRS resource RE offset |  |  | Cell 1: 0 Cell 2: 0 Cell 3: 1 | Cell 1 and Cell 3 are configured with different resource offsets |

It can be seen that PRS from cell 2 are separated from that from cell 1 and cell 3 with muting, while PRS from cell 1 are separated from that from cell 2 with comb RE offset, i.e. PRS from any pair of TRPs are transmitted in different set of REs without overlapping. We suggest to follow the same principle for SL.

Proposal 1: For the test configuration,

- Shared and dedicated resource pools are randomly used among different TCs,

- Es/Iot: 3dB (to ensure 100% PSCCH decoding),

- Overlapping SL-PRS is not be used in SL TCs.

### Issue 1-2-8: Other test case configurations Agreements: - FFS: Shared and dedicated resource pools are randomly used among different TCs. - FFS: (symbol num, comb size): (4, 4) and (2, 4).

For SL PRS, RAN4 has simulated the performance for (symbol num, comb size) = (4, 4) and (2, 4). The partial staggering pattern is new for SL PRS and it is meaningful to test it together with full staggering.

To verify the performance in multiple TX UE scenario (which is typical in real deployment), we suggest o model 2 TX UEs at least in the delay test. Since the UE may process multiple active resources in multiple slots, it is suggested to have two TX UEs transmitted in consecutive slots, and the UE that can process multiple active resources in multiple slots should take both SL PRS TX. To account for UE that cannot process multiple active resources in multiple slots, one TX UE should TX after the UE processing time which can be up to 160ms, and in that case the UE should always take the second SL PRS TX.

Proposal 2: RAN4 to consider the following SL PRS related parameters for the test.

- (symbol num, comb size): (4, 4) and (2, 4)

- MUX of multiple TX UEs: TX UE1: slot n, TX UE2: slot n + 1 and slot n + 160ms

Conclusions

In this paper we provided our views on remaining issues in performance requirements for SL positioning.

Proposal 1: For the test configuration,

- Shared and dedicated resource pools are randomly used among different TCs,

- Es/Iot: 3dB (to ensure 100% PSCCH decoding),

- Overlapping SL-PRS is not be used in SL TCs.

Proposal 2: RAN4 to consider the following SL PRS related parameters for the test.

- (symbol num, comb size): (4, 4) and (2, 4)

- MUX of multiple TX UEs: TX UE1: slot n, TX UE2: slot n + 1 and slot n + 160ms

Reference

- R4-2414033, WF on SL positioning and carrier phase positioning, CATT