3GPP TSG-RAN WG4 Meeting #112-bis	R4-2415857

Hefei, China, 14 – 18 October, 2024

Title: 	Discussion on performance requirements for RedCap positioning

Source: 	Huawei, HiSilicon

Agenda item:	4.2.2.4

Document for:	Discussion

Introduction

Maintenance of RRM performance requirements for RedCap positioning are discussed in RAN4#112, and the outcomes are captured in [1]. Based on [1], further discussions are needed for the accuracy requirements and test case.

In this paper, we will provide our views on remaining issues in performance requirements for RedCap positioning.

Discussion

Issue 5-3: Group delay calibration margin for RedCap positioning Agreement: - RAN4 to use the following assumptions for group delay calibration margin for RedCap - Non-FH: existing values for non-RedCap for the applicable BW. - FH: existing values for non-RedCap for per-hop BW [FFS: plus an extra margin (value TBD)].

For FH, the calibration error is agreed to be limited by the per-hop BW rather than the total BW after FH. On top of this, we understand some additional margin may be needed as there can be additional jitter in the calibration error due to Rx chain working on different frequencies during the measurement. This is similar to the case when reference and target resource for RSTD are in different PFLs. Of course, for FH case, different working frequencies are within a PFL, so the margin may not need to be as large margin Δ for RSTD. We suggest to consider a constant margin of 4ns, but we are open to discussing the others value.

Proposal 1: On the group delay calibration margin for RedCap with FH, use existing values for non-RedCap for per-hop BW plus an extra margin of [4]ns.

Issue 5-4: PRS RMC for RedCap positioning TCs Agreement: - [For PRS BW for FH TCs, consider the BW in test configuration as UE BW and allow cell BW to be larger.] – double check with TE vendors - For FH TCs, introduce a new PRS RMC with large BW and repetitions. - A CR revision is needed to include the new PRS RMC for 15 kHz (FR1) and 120 kHz (FR2) – Qualcomm to revise a submitted CR Discussion in AH2: - TE(R&S) input on the earlier tentative agreement from AH1 [For PRS BW for FH TCs, consider the BW in test configuration as UE BW and allow cell BW to be larger.]: - Different BWs for cell and UE have never been used - There will be confusion between OCNG and UE BW, even if we go that way - Challenge for TE: interpret OCNG (and other RMCs) as the UE BW - Two steps could be considered: - Step 1: in the configuration, separately cell BW and UE channel BW are mentioned - Step 2: resolve the OCNG issue - FFS: UE total BW should match a cell BWP (added off-line) - Recommendation: consider the above input in the work on TCs

In our view, step 1 from TE input is same as the tentative agreement from AH1, and we believe it is a must to enable testing FH. Assuming cell BW and UE BW will be separately defined, the next issues are about RMC and OCNG. In our view, RMCs are for the UE under test, and OCNG is emulating scheduling of other UEs, so as a starting point, the RMC BW should be aligned with the UE BW, and OCNG BW should be aligned with the cell BW. As to the FFS on whether UE total BW should match a cell BWP, we understand it means e.g. a RedCap UE may be configured with a 20MHz BWP within a cell of 100MHz. We do not see an issue to have this test setup if it is desired by TE vendors.

Proposal 2: For PRS BW for FH TCs,

- Cell BW and UE BW are to be separately defined in test configuration

- RMC BWs are to be aligned with UE BW

- PRS BW and ONCG BW are to be aligned with cell BW

- UE BW matches a BWP BW, if needed.

Conclusions

In this paper we provided our views on remaining issues in performance requirements for RedCap positioning.

Proposal 1: On the group delay calibration margin for RedCap with FH, use existing values for non-RedCap for per-hop BW plus an extra margin of [4]ns.

Proposal 2: For PRS BW for FH TCs,

- Cell BW and UE BW are to be separately defined in test configuration

- RMC BWs are to be aligned with UE BW

- PRS BW and ONCG BW are to be aligned with cell BW

- UE BW matches a BWP BW, if needed.

Reference

- R4-2414032, WF on RedCap positioning and PRS/SRS bandwidth aggregation, Ericsson