3GPP TSG-RAN WG4 Meeting #112-bis	R4-2415864

Hefei, China, 14 – 18 October, 2024

Title: 	Simulation assumption and initial simulation results for LP-WUR measurement

Source: 	Huawei, HiSilicon

Agenda item:	6.23.4.1

Document for:	Discussion

Introduction

RRM requirements related to LP-WUR are discussed in RAN4#112, and the outcomes are captured in WF [1]. Based on the discussions, RAN4 needs to evaluate the link level performance of LP-WUR measurement, and part of simulation assumption is agreed as listed in [1].

In this paper, we will provide our views on the simulation assumption and also provide initial simulation results for LP-WUR measurement.

Discussion

Simulation assumption

Issue 2-1-1: SINR setting

Agreement: - As starting point for RAN4 RRM simulation purpose only, -3dB, 0.5dB, 2dB dB Ês/Iot value is used for serving cell for both OOK based and OFDM based LP-WUR. - The SINR value can be updated if any update on the noise figure value in RF session. - FFS the side condition for RAN4 requirement.

Issue 2-1-1-2: Same or different SINR setting for OOK based and OFDM based LP-WUR

Recommendations: Covered by issue 2-1-1. Close this issue

In our view, RAN4 should use -3dB SINR as side condition of accuracy requirements for LR as baseline. The values provided by RAN1 are for different NF assumptions, and we believe RAN4 should take the worst case assumption for defining requirements, which corresponds to -3dB SINR.

We understand RF session may take different assumptions for OOK based and OFDM based LR for defining the refsense requirements, but it does not mean RRM requirements should be defined with different side conditions. In particular, if we can assume NF for OFDM based LR will be smaller than that for OOK based LR, we can have 2 options:

- Option 1: define side condition for OOK based LR as -3dB and for OFDM based LR as -0.5dB

- Option 2: define side condition for OOK and OFDM based LR as -3dB

Option 1 assumes the coverage of both LRs is same, so OFDM based LR is only required to work in a better SINR side condition than OOK based LR. However, OFDM based LR is supposed to be more powerful than OOK based LR, and we should not relax its requirements for baseband measurement performance just because it has a better NF. Instead, it is more reasonable to assume OFDM based LR is able to work in the same SINR side condition as OOK based LR as in option 2, and the smaller NF can translate into better coverage, i.e. the feature can be more widely used in a cell.

Proposal 1: As baseline, RAN4 to use -3dB SINR as side condition of accuracy requirements for both OOK based and OFDM based LR.

Issue 2-1-3: Time/frequency error

Agreement: Residual frequency error: OFDM based receiver [5] ppm OOK based receiver [ 10 20] ppm

In RAN1#118 the following is agreed related to the frequency error for LP-WUS design.

Agreement For OOK-based LP-WUR, the LP-WUS design assumes the following: - As starting point, the time error is based on the 5-10ppm residual frequency error after frequency error correction without considering impact of drift. - The frequency error: up to company report. - Note: 5-10ppm assumes frequency error correction is performed, e.g., RTC calibration and/or MR assistance - Note: Companies can assume other values outside of 5-10ppm as long as the values are justified Agreement As a starting point, for both time error and frequency error, the overlaid OFDM sequence design of LP-WUS assumes that the residual frequency error is 0.1-5ppm for OFDM-based LP-WUR after frequency error correction without considering impact of drift. - Note: Companies can use any value within the above range with justification on the value (including impact on power consumption)

In our understanding, the frequency error for measurement based on LP-SS or SSB would be similar as for LP-WUS detection, as the frequency error does not drift much within time scale of 320ms. For OFDM based LR, the RAN4 agreement last meeting is aligned with the latest RAN1 agreement (worst case is 5ppm), and for OOK based LR, 20ppm needs not to be considered as the worst case in the latest RAN1 agreement is 10ppm. We suggest RAN4 to update the simulation assumption based on latest RAN1 agreement.

Proposal 2: RAN4 to adopt the following residual frequency error for the simulation.

- OOK based LR: 10 ppm

- OFDM based LR: 5ppm

For time error, we understand for each LP-SS or SSB occasion, UE will do time sync before performing the measurement, so there is no need to define time error in the simulation assumption. Instead, the residual time error should be reported by companies as part of simulation results.

Proposal 3: Simulate time sync together with RRM measurement in the simulation, and the residual time error for the simulation is reported by companies.

Issue 2-1-4: Accuracy baseline for simulation

RAN4 needs to set a target accuracy for evaluating the measurement delay and side condition based on the simulation results. Companies have proposed some existing accuracy requirements as target accuracy, e.g.

- Accuracy for 1RX RedCap UE in RRC_CONNECTED

- Accuracy for CA/DC Idle Mode Measurements

- Accuracy for 1RX RedCap in RRC_IDLE (margin for reselections based on absolute priorities)

- Accuracy for normal UE in RRC_CONNECTED

In our view, accuracy for CA/DC Idle Mode Measurements is a reasonable compromise. It is noted that the target accuracy is used to decide whether side condition needs to be improved or sample number needs to be increased, but it does not mean the accuracy requirements will be same as the target accuracy. Accuracy numbers will be based on the simulation results for the agreed side condition and sample number.

Proposal 4: RAN4 to use the accuracy requirements for CA/DC Idle Mode Measurements as the target accuracy for the simulation. i.e. 6dB for RSRP and 4dB for RSRQ.

Initial simulation results

In this section, we will provide our initial simulation results based on the simulation assumption in [1]. All the results are for AWGN channel, and results for TDL-C channel will be provided later.

The parameters selected for simulations are as follows.

- Serving cell SINR: -3dB

  - Serving cell SNR = -2.4dB, interfering cell SNR = -8.4dB, i.e. 6dB power difference

- Interference type: LP-SS or SSB, interfering cell is fully synchronous to serving cell

- Sample number: 1

- Sampling rate: 7.68MHz

- ADC bit width: 8bit

- Filter: 3th Order Butterworth with 3.96MHz bandwidth

LP-SS performance

The results in this section are for OOK based LR, i.e. without utilizing the overlaid sequence on LP-SS. The parameters related to LP-SS used in the simulation are as follows.

- Number of OFDM symbols: 8

- OOK scheme: OOK-4 with M=4

- Binary sequence: length-16 M sequence with Manchester coding

Table 1: simulation results for LP-SS measurement

| AWGN | Sample | Max(5%,95%) |
| --- | --- | --- |
| LP-RSRP | 1 | 1.90 |
| LP-RSRQ |  | 1.81 |

SSS performance

The results in this section are for OFDM based LR.

Table 2: simulation results for SSB measurement

| AWGN | Sample | Max(5%,95%) |
| --- | --- | --- |
| SS-RSRP | 1 | 1.54 |
| SS-RSRQ |  | 1.76 |

Conclusions

In this paper we provided provide our views on the simulation assumption and also provide initial simulation results for LP-WUR measurement.

Proposal 1: As baseline, RAN4 to use -3dB SINR as side condition of accuracy requirements for both OOK based and OFDM based LR.

Proposal 2: RAN4 to adopt the following residual frequency error for the simulation.

- OOK based LR: 10 ppm

- OFDM based LR: 5ppm

Proposal 3: Simulate time sync together with RRM measurement in the simulation, and the residual time error for the simulation is reported by companies.

Proposal 4: RAN4 to use the accuracy requirements for CA/DC Idle Mode Measurements as the target accuracy for the simulation. i.e. 6dB for RSRP and 4dB for RSRQ.

The initial simulation results show that

- For LP-SS, the measurement accuracy with the assumed LP-SS pattern can meet the target accuracy with single sample in AWGN channel at -3dB SINR.

- For SSB, the measurement accuracy can meet the target accuracy with single sample in AWGN channel at -3dB SINR.

Reference

- R4-2413899, WF for RRM core requirements for LP-WUSWUR, vivo