3GPP TSG-RAN WG4 Meeting #112-bis	R4-2415866

Hefei, China, 14 – 18 October, 2024

Title: 	Discussion on RRM requirements for SBFD

Source: 	Huawei, HiSilicon

Agenda item:	6.20.3

Document for:	Discussion

Introduction

RRM requirements for SBFD are discussed in RAN4#112, and the outcomes are captured in WF [1]. Based on [1] the following issues need further discussion.

- RRM impacts of UE-to-UE CLI handling

- RRM impacts of SBFD operation

In this paper, we will provide our views on RRM requirements for SBFD.

Discussion

RRM impacts of UE-to-UE CLI handling

Issue 2-1-2: Measurement quantities Recommended WF - RAN4 to define core requirements for L1-SRS-RSRP and L1-CLI-RSSI measurements. This can be revisited based on further RAN1 agreement. - FFS if other measurement quantities to be considered in the core requirements based on further RAN1 agreement.

RAN1#118 made the following agreement.

Agreement For L1 UE-to-UE CLI measurement and reporting, CLI measurements is performed within the active DL BWP and the following are supported - Method#1: UE measures RSSI within DL subband - Method#2: UE measures RSRP of aggressor UE within UL subband - FFS: Method#3: UE measures RSSI within UL subband

It can be seen that both L1-SRS-RSRP and L1-CLI-RSSI will be supported measurement quantities. This is also aligned with the WID. We suggest RAN4 to define core requirements for L1-SRS-RSRP and L1-CLI-RSSI measurements. If RAN1 introduces other measurement quantities, they can also be discussed in future.

Proposal 1: RAN4 to define core requirements for L1-SRS-RSRP and L1-CLI-RSSI measurements. FFS for other measurement quantities pending on RAN1 conclusion.

Issue 2-1-4: Measurement methods Recommended WF - RAN4 to wait for RAN1 conclusion on the supported methods of L1 based UE-to-UE CLI measurement. - Meanwhile, RAN4 to discuss the impacts of different measurement methods on the requirements.

RAN1#118 agreed to support Method#1 and #2, and Method#3 is FFS. In our understanding, RAN1 will further discuss the measurement behaviours for supported methods. In particular, some companies in RAN1 proposed to use DL timing for Method#1 and a different timing (e.g. DL timing with a constant offset as in L3 CLI measurement) for Method#2. This may have impacts on the RRM requirements, e.g. side condition of the measurement requirements and the scheduling restriction. We suggest RAN4 to further wait for RAN1 conclusions on UE measurement behaviour for different measurement methods.

Proposal 2: RAN4 to wait for RAN1 conclusions on UE measurement behaviour for different measurement methods, e.g. on measurement timing.

Issue 2-1-5: Rx beam Recommended WF - RAN4 to wait for RAN1 conclusion on the Rx beam for L1 based UE-to-UE CLI measurement. - Meanwhile, RAN4 to discuss the impacts of Rx beam configuration/determination on the requirements.

In R16 the Rx beam for L3 CLI measurement is assumed to be same as the last PDCCH or PDSCH. In R19, this assumption may change as the Rx beam for L1 CLI measurement may be configured by NW. However, we understand in both cases, Rx beam sweeping is not needed for the measurement in FR2, as UE already knows which Rx beam to use for the measurement thus no need to try different Rx beams. We suggest RAN4 to confirm that no Rx beam sweeping is assumed in measurement period requirements with Rx beam configuration/determination for L1 CLI measurement.

Proposal 3: RAN4 to confirm that no Rx beam sweeping is assumed in measurement period requirements with Rx beam configuration/determination for L1 CLI measurement.

Issue 2-1-6: Measurement resources Recommended WF - RAN4 to wait for RAN1 conclusion on measurement resources to discuss whether is an impact on the requirements.

In last meeting, some companies proposed to define RRM requirements for L1-CLI-RSSI measurement for different reporting BW (narrow band RSSI) or for different RB/symbols sizes of the measurement resource. In our view, the RSSI measurement simply reflects the total received power on the indicated time and frequency resources, and the measurement performance (delay or accuracy) does not depend on the RB or symbols size of the measurement resource. We do not see clear impact of measurement resource on the measurement delay or accuracy requirements for L1-CLI-RSSI, but we are also open to other views.

Proposal 4: RAN4 to discuss whether measurement delay or accuracy for L1-CLI-RSSI is impacted by the measurement resource configuration or by wideband/narrowband reporting.

Issue 2-1-7: Side condition Options for further discussion and down-selection (for information purpose): - Time offset between DL timing and SRS arrival timing - Option 1: Rel-16 CLI SRS RSRP assumption - Option 2: Remove cell phase error from option 1 - Other options are not precluded, pending on RAN1 progress. - SRS Es/Iot, SRS configuration and maximum/minimum SRS-RSRP - Option 1: Rel-16 CLI SRS RSRP assumption - Other options are not precluded.

On the time offset between DL timing and SRS arrival timing, 2 options are proposed last meeting. The difference between the 2 options is whether to consider cell phase error. In R16 (option 1), aggressor UE is always from a neighbour cell, so cell phase error is considered in the condition. In R19, in case the victim UE is not at cell edge, the aggressor UE would be from the same cell as the victim UE, and this is why in option 2 the cell phase error is not considered.

While we can understand the rationale of option 2, we are not sure if option 1 should be dropped. As noted in WID, one important motivation of SBFD is to improve UL coverage, so UE at cell edge should not be neglected when defining the requirements. For victim UE#1 at cell edge, aggressor UE#2 may be from neighbour cell considering the typical scenario where neighbour cells on the same carrier use same UL/DL and SBFD configuration, i.e. UE#1 in cell#1 may be receiving DL in SBFD symbols while UE#2 in cell#2 is transmitting UL in SBFD symbols.

If the time offset condition is based on option 2, there would be no requirement for above scenario, and the coverage of the requirements may be an issue. At current stage, we suggest to consider both options for the simulation. Based on the simulation results, e.g. whether performance gap is large between option 1 and option 2, RAN4 can further discuss whether to down select one option, or to keep both options with corresponding applicability.

For SRS Es/Iot, SRS configuration and maximum/minimum SRS-RSRP, we suggest to re-use R16 SRS-RSRP assumption as baseline. We are open to other options if changes are justified.

Proposal 5: On side condition for L1-SRS-RSRP,

- Time offset between DL timing and SRS arrival timing: both assumptions with and without cell phase error are considered for simulation, FFS on down-selection or applicability

- SRS Es/Iot, SRS configuration and maximum/minimum SRS-RSRP: re-use R16 SRS-RSRP assumption as baseline

Issue 2-1-8: Measurement period Agreements (from online session) - RAN4 to define measurement periods for L1 based UE-to-UE CLI measurement requirements, - RAN4 to discuss the number of shots for the measurement, which can be based on RAN4 simulation results and RAN1 design. - Number of shots for simulation: 1, 2, 3. - Companies to bring proposals on other simulation parameters in the next meeting.

For L1-SRS-RSRP, RAN4 agreed to discuss the number of samples based on simulation results. In Annex, we provide simulation assumption based on similar assumption in R16 discussion for L3 CLI measurement [2]. We are open to other views e.g. additional parameters or additional values, and we suggest RAN4 to approve the simulation assumption in this meeting.

Proposal 6: RAN4 to approve the simulation assumption in Annex for determining measurement requirements for L1-SRS-RSRP.

For L1-CLI-RSSI, the measurement period should be based on single shot. The measurement does not require filtering among multiple shots, and so far, all the RSSI related measurement, including L3 CLI-RSSI (clause 9.7.3.5), RSSI measurement in NR-U (clause 9.2A.7.1 and 9.3A.8) and SL-RSSI (clause 12.6) are all based on single shot. We do not see technical difference in L1-CLI-RSSI.

Proposal 7: Measurement period for L1-CLI-RSSI is defined based on single shot.

Issue 2-1-9: Measurement reporting Recommended WF: - RAN4 to define measurement reporting requirements for L1 based UE-to-UE CLI measurement at least for aperiodic reporting - FFS on reporting requirements for periodic and semi-persistent reporting pending on RAN1 agreement - FFS whether L1-RSRP measurement reporting requirement can be re-used.

In WID it is clarified that aperiodic reporting will be supported for L1 CLI measurement, so RAN4 should define corresponding measurement reporting requirements. In our view, the reporting requirements for L1-RSRP in clause 9.5.3.3 can be used as baseline since both are L1 measurements.

#### 9.5.3.3 Aperiodic Reporting Reported L1-RSRP measurements contained in aperiodic triggered, aperiodic triggered periodic and aperiodic triggered semi-persistent L1-RSRP reports shall meet the requirements in clauses 10.1.19 for FR1 and 10.1.20 for FR2, respectively. The UE shall only send aperiodic L1-RSRP measurement reports, if a DCI trigger has been received. After the UE receives CSI request in DCI, the UE shall transmit the aperiodic L1-RSRP reporting on PUSCH over the air interface at the time specified according to clause 6.1.2.1 in TS 38.214 [26].

RAN4 can further discuss the reporting requirements for periodic and semi-persistent reporting pending on RAN1 agreement.

Proposal 8: On measurement reporting requirements,

- RAN4 to define measurement reporting requirements for L1 based UE-to-UE CLI measurement at least for aperiodic reporting. L1-RSRP measurement reporting requirement can be re-used.

- FFS on reporting requirements for periodic and semi-persistent reporting pending on RAN1 agreement.

Issue 2-1-10: Measurement accuracy Recommended WF: - RAN4 to define measurement accuracy requirements for L1 based UE-to-UE CLI measurement based on the agreed side condition and measurement period.

For L1-SRS-RSRP, the measurement accuracy should be defined in Perf part of the WI, based on simulation results for the agreed side condition and measurement period. If the sample number and side conditions are same as R16 L3 CLI measurement, then the accuracy numbers can be re-used. Otherwise, the accuracy numbers can be derived from the simulation results.

Proposal 9: RAN4 to define measurement accuracy requirements for L1-SRS-RSRP in Perf part based on simulation results for the agreed side condition and measurement period.

For L1-CLI-RSSI, the measurement accuracy should be defined in Perf part of the WI. Similar to other RSSI related measurements, there is no signal power estimation at baseband, so the accuracy is mainly based on RF calibration error. The exact number can be discussed later.

Proposal 10: RAN4 to define measurement accuracy requirements for L1-CLI-RSSI in Perf part.

Issue 2-1-11: Scheduling and measurement restriction Recommended WF - RAN4 to define scheduling and measurement restriction for L1 based UE-to-UE CLI measurement if it is needed. - FFS the impact of measurement methods and Rx beam assumption

In R16, scheduling restriction are defined for both SRS-RSRP and CLI-RSSI assuming UE may use a different (earlier) timing than DL timing. RAN4 needs to define similar requirements for L1 CLI measurement. As the measurement timing for L1 CLI measurement is still under discussion in RAN1, RAN4 can wait for RAN1 conclusion at this stage. Besides the measurement timing, the Rx beam may also need to be considered in defining scheduling restriction.

On measurement restriction, in R16 SRS-RSRP and CLI-RSSI measurements are assumed to not overlap with other L3 or L1 measurement, otherwise no requirements for SRS-RSRP and CLI-RSSI apply. RAN4 can discuss whether to follow the same principle or not for L1-RSR-RSRP and L1-CLI-RSSI. Of course, whether measurement restriction exists also depends on the measurement timing and Rx beam assumption, for which further RAN1 conclusion is needed.

#### 9.7.2.5 SRS-RSRP measurement period If the SRS resources configured for measurement are partially or fully overlapping with SMTC window, SSB or CSI-RS configured for RLM, BFD, CBD or L1-RSRP measurement or measurement gaps, requirements are not specified for TSRS_RSRP_measurement_period. #### 9.7.3.5 CLI-RSSI measurement period If the CLI-RSSI measurement resources configured for measurement are partially or fully overlapping with SMTC window, SSB or CSI-RS configured for RLM, BFD, CBD or L1-RSRP measurement or measurement gaps, requirements are not specified for TCLI_RSSI_measurement_period.

Proposal 11: RAN4 to define scheduling and measurement restriction for L1 based UE-to-UE CLI measurement based on RAN1 conclusion e.g. measurement timing and Rx beam assumption.

Issue 2-1-12: Measurement capability Recommended WF - FFS whether RAN4 needs to discuss measurement capability for L1 based UE-to-UE CLI measurement in terms of number of resources UE shall be able to monitor.

In last meeting, some companies proposed to define requirements on measurement capability in terms of number of measurement resources. For L3 CLI measurement in R16, the number of resources is defined by RAN1 (see LS from RAN1 [3]). For L1-RSRP measurement, the number of resources is defined by RAN1 as UE capability. Based on those existing references, we understand the measurement capability in terms of number of measurement resources for L1 CLI measurement should be also in RAN1 scope, so we suggest RAN4 not to further discuss this issue unless triggered by RAN1.

Proposal 12: RAN4 not to discuss measurement capability in terms of number of measurement resources unless triggered by RAN1.

Issue 2-1-13: Report mapping Recommended WF - FFS whether RAN4 needs to discuss report mapping for L1 based UE-to-UE CLI measurement. - FFS whether R16 report mapping can be re-used.

In last meeting, we proposed to define report mapping for L1 CLI measurement. After checking with RAN1, we understand RAN1 will also discuss this issue as this will impact how the UCI payload is generated for the L1 reporting. To avoid duplicated discussion, we suggest RAN4 not to further discuss this issue unless triggered by RAN1.

Proposal 13: RAN4 not to discuss measurement report mapping for L1 CLI measurement unless triggered by RAN1.

RRM impacts of SBFD operation

Issue 2-3-2: Requirements for SSB based measurement Recommended WF RAN4 further discuss impact on SSB based neighbour cell measurement in RAN4.

In last meeting, RAN4 agreed that existing requirements apply for SSB-based serving cell measurement, and it is open for SSB based neighbor cell measurement. Our view is that existing requirements apply for SSB-based neighbor cell measurement.

For measurement outside MG, RAN4 has defined scheduling restriction for TDD where the L3 measurement within SMTC is prioritized over UL. In our view, there is no difference between UL transmission in UL symbol or SBFD symbol, so the same principle/prioritization should apply.

##### 9.2.5.3.1 Scheduling availability of UE performing measurements in TDD bands on FR1 When the UE performs intra-frequency measurements in a TDD band, the following restrictions apply due to SS-RSRP or SS-SINR measurement - The UE is not expected to transmit PUCCH/PUSCH/SRS on SSB symbols to be measured, and on 1 data symbol before each consecutive SSB symbols to be measured and 1 data symbol after each consecutive SSB symbols to be measured within SMTC window duration. If the high layer in TS 38.331 [2] signalling of smtc2 is configured, the SMTC periodicity follows smtc2; Otherwise SMTC periodicity follows smtc1. When the UE performs intra-frequency measurements in a TDD band, the following restrictions apply due to SS-RSRQ measurement - The UE is not expected to transmit PUCCH/PUSCH/SRS on SSB symbols to be measured, RSSI measurement symbols, and on 1 data symbol before each consecutive SSB to be measured/RSSI symbols and 1 data symbol after each consecutive SSB to be measured/RSSI symbols within SMTC window duration. If the high layer signalling of smtc2 is configured in TS 38.331 [2], the SMTC periodicity follows smtc2; Otherwise the SMTC periodicity follows smtc1.

For measurement within MG, UE is not required to transmit UL during MG. We do not see any reason to change this principle for UL transmission in SBFD symbols.

Proposal 14: SSB based neighbour cell measurement is prioritized over UL transmission in SBFD symbols, i.e. existing requirements apply for SSB-based neighbour cell measurement.

Issue 2-3-3: Requirements for CSI-RS based measurement Recommended WF - RAN4 to discuss the impact of SBFD operation on the CSI-RS measurement requirements.

For CSI-RS measurement, several aspects of SBFD operation are raised up by companies that may impact the RRM requirements. In our view, all the raised aspects are valid and need to be further discussed.

- Proposals - Proposal 1 (vivo, Samsung, HW, QC, MTK): - For SBFD operating, RAN4 discuss the impact on the CSI-RS measurement requirements. - Proposal 2 (Samsung): - L1 CSI-RS measurement requirements may need to be updated to following RAN1 update on CSI report configuration between SBFD slots and non-SBFD slots - Proposal 3 (HW): - L1 and L3 CSI-RS measurement requirements may need to be updated due to collision between CSI-RS measurement and dynamic UL transmission on SBFD symbols - Proposal 4 (HW, QC, MTK): - CSI-RS measurement requirements may need to be updated due to CSI-RS resources not mapped outside DL usable PRBs in frequency domain

CSI-RS resource with occasions across SBFD and non-SBFD symbol

In RAN1#117, following is agreed regarding UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols.

Agreement For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) for an SBFD aware UE, the SBFD-aware UE is provided with one of the configurations. - Configuration 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only - Configuration 2: The transmissions/receptions can be in SBFD symbols and non-SBFD symbols - FFS granularity of the configuration, e.g. per UE, per channel/signal etc. - FFS whether support of configuration 2 is subject to UE capability

In our view, this means if a CSI-RS resource has occasions across SBFD and non-SBFD symbol,

- If Configuration 1 is provided, then UE is only supposed to measure those occasions on the selected symbol type (either SBFD symbol or non-SBFD symbol). The measurement period requirements would depend on the time duration to collect enough samples on the resource occasions on the selected symbol type rather than the time duration to collect enough samples on all resource occasions (defined by sample number multiplied by the resource periodicity).

- If Configuration 2 is provided, then UE is supposed to measure occasions on 2 symbol types separately. Separate measurement periods may need to be defined for 2 symbol types, and requirements for SBFD (non-SBFD) symbol would depend on the time duration to collect enough samples on the resource occasions on the SBFD (non-SBFD) symbol.

Collision handling between UL and DL on SBFD symbols

In RAN1#116-bis, following is agreed regarding collision handling between UL and DL on SBFD symbols.

Agreement If link direction indication is not supported nor provided for a SBFD symbol, for collision Case 2 (semi-statically configured DL reception vs. dynamically scheduled UL transmission) in the SBFD symbol for SBFD-aware UEs, reuse the existing collision handling principles in NR for operation on flexible symbols on a single carrier in unpaired spectrum, i.e. UE does not receive DL channel/signal. - The above does not imply link direction indication is supported - FFS on dynamically scheduled UL transmission with repetition

In our view, this means CSI-RS measurement, which is a semi-static DL reception except for aperiodic CSI-RS resource, will be impacted, if the CSI-RS resource is on SBFD symbols. When dynamic UL transmission is scheduled to be colliding with the CSI-RS resource, the measurement occasion will be dropped and the measurement period should be extended accordingly. RAN4 can further discuss whether and how to define the extension, and in our view, we can re-use the approach as in NR-U where the measurement occasion may get unavailable due to dynamic LBT outcome.

CSI-RS resource mapping in frequency domain

In RAN1#117, following is agreed regarding CSI-RS resource mapping in frequency domain.

Working Assumption For frequency resource allocation for CSI-RS across downlink subbands for SBFD-aware UEs, support one contiguous CSI-RS resource allocation with non-contiguous CSI-RS resource derived by excluding frequency resources outside DL usable PRBs. - No impact on CSI-RS sequence generation. - CSI-RS sequence mapping is applied to CSI-RS resources within DL usable PRBs only (effectively, this is same as the case when the CSI-RS sequence mapped to the RBs outside the DL usable PRBs are punctured). - RAN1 to further study the impact on CSI processing timeline in SBFD symbols to process the CSI-RS across the two DL subbands.

It can be seen that on SBFD symbols, CSI-RS REs mapped to RBs outside DL usable PRBs are punctured. It will impact the requirements which apply when CSI-RS BW is larger than a threshold. For example, CSI-RS based RLM requirements apply when the CSI-RS resource is transmitted over the bandwidth ≥ 24 RBs. It is clear that CSI-RS REs mapped outside DL usable PRBs should not be counted in the transmitted BW, and some clarification may be needed on the transmitted BW.

Proposal 15: RAN4 to discuss the impact of following SBFD operation on CSI-RS measurement.

- CSI-RS resource with occasions across SBFD and non-SBFD symbol

- CSI-RS measurement colliding with dynamically scheduled UL on SBFD symbols

- CSI-RS resources puncturing on REs outside DL usable PRBs

Issue 2-3-4: Requirements for scheduling restriction Recommended WF - RAN4 to discuss whether scheduling restriction requirements are impacted due to SBFD operation based on RAN1 agreements.

Although the SSB measurement itself is not impacted by SBFD operation, its scheduling restriction may need update. As discussed above, RAN4 has defined scheduling restriction for TDD for L3 measurement but not for L1 measurement. The reason is that there is always guard period for the DL-to-UL transition, and UL transmission on UL symbols would never overlap with SSB reception for the serving cell in a reasonable configuration. However, there is no guard period between SBFD symbol and SSB symbol. UL transmission on SBFD symbol immediately after SSB symbol may collide with SSB reception due to TA.

One way to handle the collision is to extend scheduling restriction for TDD to SSB based L1 measurement. Alternatively, RAN4 can further check if the collision is already addressed by the following RAN1#117 agreement, i.e. whether the UL transmission includes the effect of TA.

Agreement Re-use the existing collision handling principles for NR TDD that SSB is prioritized over configured UL transmission and dynamically scheduled UL transmission - FFS whether a slot consisting of SSB symbols is considered as a full DL slot or SSB symbols configured with SBFD subbands are SBFD symbols and only DL receptions within DL usable PRBs are allowed for SBFD aware UEs.

Proposal 16: RAN4 to discuss if scheduling restriction is needed for handling collision between L1 SSB measurement and UL transmission on SBFD symbol immediately after the SSB symbol.

Issue 2-3-7: Requirements for MG and BWP switch Recommended WF - RAN4 to discuss whether to clarify the term “UL slot” in MG and BWP switch requirements when SBFD operation is enabled.

We do not see impacts of SBFD operation to BWP or CBW switch, as those procedures do not rely on measurement. However, some clarification may be needed because the requirements for DL (and UL) BWP switch are currently defined based on DL (and UL) transmission only on DL (and UL) slot. DL (and UL) transmission on SBFD symbols needs to be accounted.

For DCI-based BWP switch, after the UE receives BWP switching request at DL slot n on a serving cell, UE shall be able to receive PDSCH (for DL active BWP switch) or transmit PUSCH (for UL active BWP switch) on the new BWP on the serving cell on which BWP switch on the first DL or UL slot occurs right after a time duration of TBWPswitchDelay + Y which starts from the beginning of DL slot n. Where, …

We do not see impacts of SBFD operation to the MG related requirements, but one clarification may be needed for the UL transmission immediately after MG. Currently it is only defined for UL slot, while with SBFD operation, the UL transmission can be on SBFD symbol.

It is up to UE implementation whether or not the UE is able to conduct transmission in the following slot(s), - when MGTA is not applied, in the L consecutive UL slots with respect to the SCS of the UL carrier with the same slot indices as the DL slots occurring immediately after measurement gap - when MGTA is applied and the SCS of the UL carrier is other than 15kHz, in the L consecutive UL slots with respect to the SCS of the UL carrier with the same slot indices as the DL slots occurring immediately after measurement gap - when MGTA is applied and the SCS of the UL carrier is 15kHz, in the L consecutive UL slots with respect to the SCS of the UL carrier with the same slot indices as the DL slots occurring immediately after the slot partially overlapped with measurement gap where UL slot denotes that all the symbols in the slot are uplink symbols, and L=1 if for the UL transmission is less than the length of one slot; L=2 otherwise.

Proposal 17: RAN4 to update the term “UL slot” to “UL slot and SBFD slot” in MG and BWP switch requirements when SBFD operation is enabled.

Issue 2-3-8: Requirements for UL resource muting Recommended WF - RAN4 further to discuss whether additional RRM requirements would be defined for UL resource muting for PUSCH feature.

In last meeting, some companies proposed that no additional RRM requirements would be defined due to UL resource muting on PUSCH. We agree with this proposal, and we do not see any RRM requirement is impacted by UL resource muting.

Proposal 18: RAN4 to confirm that no RRM requirements would be defined for UL resource muting.

Issue 2-3-9: Requirements for generic SBFD operation Recommended WF - RAN4 to discuss whether there is any RRM impact e.g. to scheduling restrictions/interruptions requirements due to SBFD operation.

In last meeting, some companies proposed to study scheduling restrictions/interruptions requirements for generic SBFD operation. Here generic SBFD operation means UL and DL data transmission with SBFD, and no specific procedure, such as RRM measurement or BWP switching, is considered. In our view, there is no RRM impact due to generic SBFD operation, and the data scheduling should be within RAN1 scope instead of RAN4 RRM.

Proposal 19: RAN4 to confirm that no scheduling restrictions/interruptions requirements would be defined for generic SBFD operation.

Issue 2-3-10: Limits on the maximum number of DL/UL switching Recommended WF - RAN4 to discuss whether to put limits on the maximum number of DL/UL switching during SBFD slots within the SBFD periodicity

In last meeting, some companies proposed to put limits on the maximum number of DL/UL switching during SBFD slots. In our view, this issue is also related to UL and DL data transmission, thus should not be related to any RRM requirement. In particular, in R15 for normal TDD operation, the issue has been addressed by introduction of FG 5-1 and 5-1b.

FG 5-1 is for “Basic scheduling/HARQ operation”, and component 9 is defined as follows.

9) In TDD support at most one switch point per slot for actual DL/UL transmission(s)

FG 5-1b is for “More than one DL/UL switch point in a slot”, and is defined as follows.

In TDD support more than one switch points in a slot for actual DL/UL transmission(s)

Since FG 5-1 and FG 5-1b are both defined by RAN1 in R15, we believe it should also be up to RAN1 to decide how many DL/UL switching is supported in an SBFD slots, e.g. based on different UE capabilities. We suggest RAN4 not to discuss this issue unless triggered by RAN1.

Proposal 20: RAN4 not to discuss limits on the maximum number of DL/UL switching during SBFD slots unless triggered by RAN1.

Conclusions

In this paper we provided our views on RRM requirements for SBFD.

Proposal 1: RAN4 to define core requirements for L1-SRS-RSRP and L1-CLI-RSSI measurements. FFS for other measurement quantities pending on RAN1 conclusion.

Proposal 2: RAN4 to wait for RAN1 conclusions on UE measurement behaviour for different measurement methods, e.g. on measurement timing.

Proposal 3: RAN4 to confirm that no Rx beam sweeping is assumed in measurement period requirements with Rx beam configuration/determination for L1 CLI measurement.

Proposal 4: RAN4 to discuss whether measurement delay or accuracy for L1-CLI-RSSI is impacted by the measurement resource configuration or by wideband/narrowband reporting.

Proposal 5: On side condition for L1-SRS-RSRP,

- Time offset between DL timing and SRS arrival timing: both assumptions with and without cell phase error are considered for simulation, FFS on down-selection or applicability

- SRS Es/Iot, SRS configuration and maximum/minimum SRS-RSRP: re-use R16 SRS-RSRP assumption as baseline

Proposal 6: RAN4 to approve the simulation assumption in Annex for determining measurement requirements for L1-SRS-RSRP.

Proposal 7: Measurement period for L1-CLI-RSSI is defined based on single shot.

Proposal 8: On measurement reporting requirements,

- RAN4 to define measurement reporting requirements for L1 based UE-to-UE CLI measurement at least for aperiodic reporting. L1-RSRP measurement reporting requirement can be re-used.

- FFS on reporting requirements for periodic and semi-persistent reporting pending on RAN1 agreement.

Proposal 9: RAN4 to define measurement accuracy requirements for L1-SRS-RSRP in Perf part based on simulation results for the agreed side condition and measurement period.

Proposal 10: RAN4 to define measurement accuracy requirements for L1-CLI-RSSI in Perf part.

Proposal 11: RAN4 to define scheduling and measurement restriction for L1 based UE-to-UE CLI measurement based on RAN1 conclusion e.g. measurement timing and Rx beam assumption.

Proposal 12: RAN4 not to discuss measurement capability in terms of number of measurement resources unless triggered by RAN1.

Proposal 13: RAN4 not to discuss measurement report mapping for L1 CLI measurement unless triggered by RAN1.

Proposal 14: SSB based neighbour cell measurement is prioritized over UL transmission in SBFD symbols, i.e. existing requirements apply for SSB-based neighbour cell measurement.

Proposal 15: RAN4 to discuss the impact of following SBFD operation on CSI-RS measurement.

- CSI-RS resource with occasions across SBFD and non-SBFD symbol

- CSI-RS measurement colliding with dynamically scheduled UL on SBFD symbols

- CSI-RS resources puncturing on REs outside DL usable PRBs

Proposal 16: RAN4 to discuss if scheduling restriction is needed for handling collision between L1 SSB measurement and UL transmission on SBFD symbol immediately after the SSB symbol.

Proposal 17: RAN4 to update the term “UL slot” to “UL slot and SBFD slot” in MG and BWP switch requirements when SBFD operation is enabled.

Proposal 18: RAN4 to confirm that no RRM requirements would be defined for UL resource muting.

Proposal 19: RAN4 to confirm that no scheduling restrictions/interruptions requirements would be defined for generic SBFD operation.

Proposal 20: RAN4 not to discuss limits on the maximum number of DL/UL switching during SBFD slots unless triggered by RAN1.

Reference

- R4-2413900, WF on SBFD RRM, Huawei, HiSilicon

- R4-1907392, Simulation assumption for SRS-RSRP measurement requirements, LG Electronics

- R1-1903677, LS on UE-UE CLI measurement/reporting and Network coordination mechanism for CLI, RAN1

Annex: Simulation assumption for L1-RSRP-RSRP

In this section, we provide link level simulation assumption for evaluating measurement performance for L1-RSRP-RSRP. The suggested assumptions are based on the approved simulation assumption for L3 SRS-RSRP in R16 discussion [2], with the following adaptations.

- UE2 is removed from the assumption. As noted in [2], UE2 (the second aggressor UE besides UE1) is considered only evaluating the impact of cyclic shift. The impact of cyclic shift has been concluded in R16 with the following side conditions, and we do not see difference in L1 CLI measurement.

- One of the following conditions is met - There is no other SRS resource with the same root sequence and on the same symbol and with same comb as the relevant SRS resource. - If multiple SRS resources are on the same symbol and with same comb, the distance between cyclic shifts of any two resources is no less than 6 if transmissionComb = n4, and no less than 4 if transmissionComb = n2.

- The timing error for UE1 (aggressor UE) is clarified as residual timing error and is changed to

  - Option 1: 4.67us for FR1 and 3.67us for FR2

  - Option 2: 1.67us for FR1 and 0.67us for FR2

Option 1 is same as condition in the R16 requirements, and option 2 is removing cell phase error from option 1. The two options are from Issue 2-1-7 in [1] as discussed in section 2.1. Other options can be added based on the discussion during the meeting.

- Number of measurement samples is changed to 1, 2 and 3 based on agreement for Issue 2-1-8 in [1].

- Measurement period [slot] is changed to measurement interval. 20ms interval is selected based on the lower bound of 60ms in the R16 measurement period requirements. The actual measurement period depends on the number of measurement samples and the measurement interval, thus does not need to be specified separately as simulation assumption.

- Number of SRS BW is changed to 48 which is same as in the R16 requirements.

- The SINR is changed to 1dB which is same as in the R16 requirements.

The resulted simulation assumptions are listed in Table 1. All the changes compared to [2] except removal of UE2 are marked in yellow.

Table 1: Simulation assumptions for SRS-RSRP measurement

| Parameters |  | Value |  |
| --- | --- | --- | --- |
|  |  | UE0 [victim UE] | UE1 [aggressor UE] |
| Carrier frequency |  | 4GHz for FR1 30GHz for FR2 |  |
| Subcarrier spacing |  | 15kHz, 30kHz, 60kHz for FR1 60kHz, 120kHz for FR2 |  |
| SRS configuration | Number of port | N/A | 1 |
|  | TransmissionComb |  | 2 |
|  | combOffset |  | 0 |
|  | Cyclic shift |  | 0 |
|  | StartPosition (SRS symbol start position) |  | 0 |
|  | Number of symbol |  | 1 |
|  | Number of RB |  | 48 |
|  | Frequency domain position and shift |  | 0 |
|  | Periodicity [slot] |  | 5 |
| Residual timing error (time offset between measurement timing at UE0 and arrival timing of SRR from UE 1) |  | - Option 1: 4.67us for FR1, 3.67us for FR2 - Option 2: 1.67us for FR1, 0.67us for FR2 |  |
| CP Length |  | Normal |  |
| Number of measurement samples |  | 1, 2, 3 | N/A |
| Measurement bandwidth [RB] |  | Same as SRS transmission RB |  |
| Measurement interval |  | 20ms | N/A |
| Number of UE receive antennas |  | 2 Rx | N/A |
| Propagation conditions |  | AWGN, TDL-A10, TDL-C100 for FR1, AWGN, TDL-A75, TDL-C300 for FR2 |  |
| SINR |  | 1dB | N/A |

Performance metric of the simulation is

- Delta L1-SRS-RSRP = [estimated L1-SRS-RSRP – ideal L1-SRS-RSRP] dB

No impairment margin should be included for provided results.