3GPP TSG-RAN WG4 Meeting #112-bis	R4-2415867

Hefei, China, 14 – 18 October, 2024

Title: 	Discussion on RRM requirements for RedCap UE in NTN

Source: 	Huawei, HiSilicon

Agenda item:	6.26.4.1

Document for:	Discussion

Introduction

RRM requirements Rel-19 NTN are discussed in RAN4#112, and the outcomes are captured in WF [1]. Based on the discussions, RAN4 needs to further discuss the RRM requirements for RedCap UE in NTN.

- General consideration

- RedCap features

- NTN features

- Spec structure

In this paper, we will provide our views on RRM requirements for RedCap UE in NTN.

Discussion

General consideration

Issue 6-1-2: The bandwidth of (e)Redcap UE with FR1-NTN <Way Forward> - Both bandwidth with and without reduction of R18 eRedCap UE should be supported for (e)RedCap UE with FR1-NTN bands. - Note: There is no RRM impact on the two different types of eRedCap UEs (bandwidth with and without reduction).

We support the WF. In the WID it is clear that both Rel-17 RedCap and Rel-18 eRedCap UEs are considered in tis WI, and BW reduction should be included as part of Rel-18 eRedCap. Besides, since there is no RRM impact due to BW reduction, there is no motivation to preclude it from the current WI.

Proposal 1: Both BWs with and without reduction of R18 eRedCap UE should be supported for (e)RedCap UE with FR1-NTN bands.

Issue 6-1-5: The network scenario considered for (e)Redcap UE with FR1-NTN <Way Forward> - For scenario considered for (e)Redcap UE with FR1-NTN bands, RAN4 only supports NR SA operation mode with single carrier.

We support the WF. So far, NTN can only be deployed in NR SA operation mode, and CA is not supported for either in NTN or for RedCap. There is no other scenarios to be considered for the current WI.

Proposal 2: For scenario considered for (e)Redcap UE with FR1-NTN bands, RAN4 only supports NR SA operation mode with single carrier.

Issue 6-1-6: The SCS considered for (e)Redcap UE with FR1-NTN <Way Forward> - For RRM requirements, the SCS considered for (e)Redcap UE with FR1-NTN support 15kHz, 30kHz and 60kHz.

We do not support the WF. In timing requirements for normal UE in NTN, only 15kHz and 30kHz SCS are considered because the timing error with 60kHz SCS would be larger than CP/2 following the assumption on GNSS error and satellite location error, and we believe the same scope should be followed for RedCap UE. For other requirements, they are either SCS agnostic or all SCS-es are explicitly supported for normal UE, and we believe the same scope should be also followed for RedCap UE.

Proposal 3: 60kHz SCS is not considered for timing requirements for (e)Redcap UE with FR1-NTN bands (same as normal UE with FR1-NTN bands).

Issue 6-2-1: The general principle for defining the RRM requirements for (e)RedCap UE with FR1-NTN <Way Forward> : Further discuss the following proposal - Proposal 1 (CATT, MTK, Xiaomi, Samsung, HW, vivo, ZTE): - A general principle is that to define the RRM requirements for (e)RedCap UE with FR1-NTN bands based on the existing requirements for FR1-NTN. - Proposal 2 (Ericsson): - RRM requirements for RedCap UE in NTN can be started by taking RedCap UE in TN as baseline. - The requirements and enhancements for NR NTN in Rel-17 and Rel-18 shall be introduced for RedCap UE in NTN.

We do not think RAN4 need to spend too much efforts on this issue about generic principle. Technically, both existing requirements for RedCap and for NTN should be considered in the current WI. RAN4 should focus on how to define those requirements as agreed in last meeting, e.g. whether and how RedCap and NTN features are to be considered in each of them.

Proposal 4: RAN4 to focus on the exact requirements developments rather than generic principle to define requirements.

Issue 6-2-2: What RRM requirements are defined for (e)RedCap UE with FR1-NTN <Way Forward> - FFS for the following requirements: - Minimization of Drive Tests (MDT) in RRC_IDLE state and RRC_INACTIVE state - NR Rel-17/18 Conditional Handover, and RACH-less Handover - Note: further check whether they are supported for RedCap NTN in RAN1/2 spec. - FFS: Send LS to RAN1/2 to ask the status. RAN4 does not trigger the RAN1/2 discussion to support these features. - Configured Grant based Small Data Transmissions (CG-SDT) - Random access based Small Data Transmissions (RA-SDT)

We do not support to define requirements for MDT. We do not have MDT requirement for RedCap UE in TN, and there is no strong reason why RedCap UE performing MDT is important in NTN.

We support to define requirements CHO. In our understanding, CHO is not limited to normal UE in RAN1/2 spec, so they are by default supported for RedCap UEs. Of course, RRM based CHO is also supported for RedCap UEs in TN in RAN1/2 spec, but there is no RAN4 requirement in Rel-17 RedCap WI, so we are open to discuss whether to define requirements for (time/frequency + RRM) based CHO and pure RRM based CHO. However, time/frequency based CHO is quite specific and useful for NTN, at least they should be supported in the current WI.

We support to define requirements RACH-less HO. Requirement wise RACH-less HO is no different from normal HO except the definition of Tiu. It is functional and does not impose additional performance requirement compared to normal HO, so we do not see the motivation to preclude it for RedCap UE.

We support to define requirements for SDT except for those related to TA validation.

- In theory, as we do not have SDT requirements for normal UE in NTN, there is no strong motivation to define SDT requirements for RedCap UE in NTN. However, part of the SDT requirements defines 1RX UE should add an offset to the configured threshold, and RAN4 has agreed to consider 1RX impact, so requirements are anyway be needed for requirements completeness.

- On the other hand, we do not support to define requirements related to TA validation. RSRP change is used reflect the between UE and BS, and when the change in distance is larger than a threshold, TA will be considered as invalid. This works for TN, but for NTN the change in distance is already compensated by UE in UE specific TA, so RSRP change is not a good metric for TA validation. It is also noted that for PUR in IoT NTN, requirements related to RSRP change based TA validation are not defined.

Based on above analysis, we support define requirements for CHO, RACH-less HO and part of SDT for (e)Redcap UE with FR1-NTN bands. It is noted that all those features are optional UE features, so defining requirements for RedCap UEs does not mean every RedCap UE needs to support them, but support of them is still based on UE capability.

Proposal 5: Besides the agreed list of requirements, RAN4 to define the following requirements for (e)Redcap UE with FR1-NTN bands.

- RACH-less HO

- CHO, at least time/frequency based CHO, and FFS (time/frequency + RRM) based CHO and RRM based CHO

- SDT, excluding those related to TA validation

Issue 6-2-3: Features to be considered for (e)RedCap UE with FR1-NTN <Way Forward> - RAN4 to analyze the RRM requirement impacts from following features for (e)RedCap UE with FR1-NTN case by case: - 1Rx - FFS: NCD-SSB - HD-FDD - FFS: R17 relaxed measurement - FFS: RRC_IDLE/RRC_INACTIVE mode eDRX enhancement - FFS: Determine if CSSF and concurrent gap is supported in Redcap NTN - FFS: FH in PRS measurement - Single cell search and measurement engine

We will discuss whether and how to support those features in section 2.2 and 2.3.

RedCap features

Issue 6-3-2: How to relax the requirements for 1Rx (e)RedCap UEs with FR1-NTN ? <Way Forward> : Further discuss the following proposal - Proposal 1 (vivo, ZTE): - Introduce corresponding RRM requirements for 1 Rx and current requirements for Redcap UE with 1Rx can be the reference. - Proposal 2 (Xiaomi): - For 1RX RedCap: Relax measurement requirements from the aspects of extending the number of measurement samples or relaxing the measurement accuracy. - Proposal 3 (ZTE): - RAN4 shall define the Qout requirements for RedCap UEs in FR1-NTN bands. The evaluation period for 2Rx shall be reused and the evaluation period shall be extended for 1Rx by factor 2. Qin shall be reused. - RAN4 shall reuse the 1dB offset as legacy for 1Rx RedCap UE in NR NTN. - Proposal 4 (QC): - For 1Rx RedCap NTN UE, RAN4 to adopt the same requirement relaxations and modifications as follows: - Extended delays for PSS/SSS detection and SSB index identification (PBCH decoding) - Handover delay Extension - Extended RLM OOS and BFD evaluation periods - Aggregation level of 16 for RLM/BFD hypothetical PDCCH parameters - Measurement accuracy relaxation for SS-RSRP, SS-RSRQ, SS-SINR, L1-RSRP

RAN4 has agreed to define separate sets of RRM requirements for 1RX and 2RX (e)Redcap UE, if needed, and as baseline, to follow the same principle for (e)RedCap UE in TN network. Based on the impact of 1RX in TN as summarised in our paper in last meeting [2], we have the following proposal.

Proposal 6: RAN4 to consider the impacts of 1RX in the following requirements in the same way as requirements for RedCap in TN:

- Cell reselection: offset and relaxation for accuracy

- HO: relaxation for Tsearch

- Re-establishment and Re-direction: relaxation for Tidentify

- RLM/BFR: change in PDCCH parameters for RLM, relaxation for evaluation period, and relaxation for accuracy

- L3 and L1 measurement: relaxation for delay and accuracy

- NW verified location: relaxation for accuracy

Issue 6-5-1: How to consider the impact of eDRX enhancement for (e)Redcap UE with FR1-NTN? <Way Forward> : Further discuss the following proposal - Proposal 1 (CATT, CMCC, Samsung): - The eDRX enhancement introduced for (e)RedCap UEs should be considered when defining RRC_IDLE or/and RRC_INACTIVE state mobility requirements for (e)RedCap UEs with FR1-NTN bands. - Proposal 1a (CATT): RAN4 need to discuss whether the eDRX enhancement for both Rel-17 RedCap and Rel-18 eRedCap UEs should be introduced for (e)RedCap UEs with FR1-NTN bands. - Option 1: Only introduce the eDRX enhancement for Rel-17 RedCap UEs. - Option 2: Introduce the eDRX enhancement for both Rel-17 RedCap and Rel-18 eRedCap UEs. - Proposal 1b (CMCC): Support eDRX configuration for Redcap over NTN, and define following applicability rule for each deployment scenario: - For GEO deployment, all DRX and eDRX cycle can be supported. - For earth-fixed LEO deployment, requirements are applicable for up to 10.24s eDRX cycle - For earth-moving LEO deployment, requirements are not applicable for eDRX cycle - Proposal 1c (Samsung): - RAN4 to discuss and specify the requirements related to eDRX for GSO and NGSO (LEO) separately. Not all eDRX cycle can be applicable for NGSO (LEO) scenario. - Proposal 1d (QC): For eRedCap UE, RAN4 can consider the following aspect for RRM requirement definition if necessary and applicable for NR NTN support. - Enhanced eDRX in RRC INACTIVE - Proposal 2 (LG): - For NTN RRM requirements for RedCap UE, - For IDLE/INACTIVE mode: Preclude eDRX_IDLE cycle configuration for RedCap NTN

We support to define requirements for RedCap UE in NTN for cell reselection under eDRX at least for GSO. In our view, GSO scenario is close to TN, and eDRX is an important feature for RedCap UE, so even we do not have eDRX requirements for normal UE in NTN, we believe we can define them for RedCap UE.

For NGSO more discussions are needed. As the service time of a satellite for NGSO scenario is short, eDRX may not be applicable. For LEO with earth moving cell, RAN4 already concluded that DRX =2.56s is not applicable, and it would be reasonable to consider eDRX for this scenario. For LEO with earth fixed cell, it is also questionable if eDRX is really useful or not.

Proposal 7: RAN4 to define requirements for (e)Redcap UE with FR1-NTN bands for cell reselection under eDRX

- The requirements are applicable for GSO, and FFS for LEO

- eDRX related requirements for RedCap UE in TN are re-used as baseline

We support to define requirements for Rel-17 relaxed measurement for cell reselection. RAN4 has defined requirements for Rel-16 relaxed measurement for cell reselection in 4.2C.2.7 and 4.2C.2.8. We believe same principle can be followed for Rel-17 relaxed measurement.

Proposal 8: RAN4 to define requirements for (e)Redcap UE with FR1-NTN bands for Rel-17 relaxed measurement for cell reselection. Requirements apply provided target cell is GSO (same as Rel-17 NTN requirements for normal UE).

For RedCap positioning, frequency hopping (FH) is supported. FH in PRS measurement aims at improving measurement accuracy where the measurement is not restricted by the UE RF BW (20MHz) but can reach cell BW (100MHz). However, the cell BW in FR1-NTN bands are limited (~30MHz), so the gain from FH may be limited. On the other hand, for NW verified location, most likely the requirements for positioning will be re-used, so supporting FH for NW verified location requires very little effort. We suggest RAN4 to further discuss whether to support FH in the requirements for NW verified location.

Proposal 9: RAN4 to discuss whether to support FH in the requirements for NW verified location for (e)Redcap UE with FR1-NTN bands.

NTN features

NTN is mandatorily required to support 2 SMTCs per MO, and supporting more is an optional capability. When the two SMTCs do not overlap there is no scaling in measurement period meaning UE has to support parallel measurement for the two SMTCs. This may be a bit challenging for RedCap UE with reduced complexity. In our view, multiple-SMTC should still be supported for RedCap UE in NTN from requirement perspective, and there are two ways to address the complexity issue:

- Option 1: make it optional for RedCap UE to support 2 SMTC per MO,

- Option 2: change the requirements so that measurement for different SMTCs are not parallel.

We suggest RAN4 to discuss how to support multiple SMTCs for (e)Redcap UE with FR1-NTN bands.

Proposal 10: RAN4 to discuss support of multiple SMTCs for (e)Redcap UE with FR1-NTN bands.

- Option 1: make it optional for RedCap UE to support 2 SMTC per MO

- Option 2: change the requirements so that measurement for different SMTCs are not parallel

In last meeting, some companies proposed to discuss support of concurrent gaps for (e)Redcap UE with FR1-NTN bands. In our view, concurrent gap in NTN is an optional UE feature, and the requirements for normal UE will most likely be re-used for RedCap UE, so there is no motivation to preclude it.

Proposal 11: Concurrent gaps are supported for (e)Redcap UE with FR1-NTN bands, and existing requirements for normal UE in NTN are re-used as baseline.

Another feature that needs discussion is enhanced cell reselection measurement. This is also an optional UE feature, but a little bit different from concurrent gaps, the existing requirements for normal UE in NTN may not be directly re-usable for RedCap UE with 1RX, i.e. RAN4 needs to discuss new requirements for 1RX if enhanced cell reselection measurement is to be supported for (e)Redcap UE with FR1-NTN bands. We do not think it requires large efforts, so we are open to discuss whether to support it or not.

Proposal 12: RAN4 to discuss whether to support enhanced cell reselection measurement for (e)Redcap UE with FR1-NTN bands.

Spec structure

Issue 6-8-1: Specification structure for (e)Redcap UE with FR1-NTN <Way Forward> : Further discuss the following proposal - Proposal 1 (CATT, CMCC): - Use the following principles to define the RRM requirements for (e)RedCap UEs with FR1-NTN bands: - Define them in the new sections of section number with new suffix X in the specification. - Utilize the reference method for the requirements that can be reused. - Proposal 2 (CMCC): - Slightly prefer to introduce the Redcap over NTN requirements in new subsections in TS 38.133 or new subclauses in current NTN section.

We support option 1 to define requirements for (e)RedCap UE with FR1-NTN bands. In our view, this is a reasonable way to make the spec clear while avoid too much duplication.

Proposal 13: Use new sections and section number with new suffix X in the specification to capture requirements for (e)RedCap UE with FR1-NTN bands. Utilize the reference method for the requirements that can be reused.

Conclusions

In this paper we provided provide our views on RRM requirements for RedCap UE in NTN.

Proposal 1: Both BWs with and without reduction of R18 eRedCap UE should be supported for (e)RedCap UE with FR1-NTN bands.

Proposal 2: For scenario considered for (e)Redcap UE with FR1-NTN bands, RAN4 only supports NR SA operation mode with single carrier.

Proposal 3: 60kHz SCS is not considered for timing requirements for (e)Redcap UE with FR1-NTN bands (same as normal UE with FR1-NTN bands).

Proposal 4: RAN4 to focus on the exact requirements developments rather than generic principle to define requirements.

Proposal 5: Besides the agreed list of requirements, RAN4 to define the following requirements for (e)Redcap UE with FR1-NTN bands.

- RACH-less HO

- CHO, at least time/frequency based CHO, and FFS (time/frequency + RRM) based CHO and RRM based CHO

- SDT, excluding those related to TA validation

Proposal 6: RAN4 to consider the impacts of 1RX in the following requirements in the same way as requirements for RedCap in TN:

- Cell reselection: offset and relaxation for accuracy

- HO: relaxation for Tsearch

- Re-establishment and Re-direction: relaxation for Tidentify

- RLM/BFR: change in PDCCH parameters for RLM, relaxation for evaluation period, and relaxation for accuracy

- L3 and L1 measurement: relaxation for delay and accuracy

- NW verified location: relaxation for accuracy

Proposal 7: RAN4 to define requirements for (e)Redcap UE with FR1-NTN bands for cell reselection under eDRX

- The requirements are applicable for GSO, and FFS for LEO

- eDRX related requirements for RedCap UE in TN are re-used as baseline

Proposal 8: RAN4 to define requirements for (e)Redcap UE with FR1-NTN bands for Rel-17 relaxed measurement for cell reselection. Requirements apply provided target cell is GSO (same as Rel-17 NTN requirements for normal UE).

Proposal 9: RAN4 to discuss whether to support FH in the requirements for NW verified location for (e)Redcap UE with FR1-NTN bands.

Proposal 10: RAN4 to discuss support of multiple SMTCs for (e)Redcap UE with FR1-NTN bands.

- Option 1: make it optional for RedCap UE to support 2 SMTC per MO

- Option 2: change the requirements so that measurement for different SMTCs are not parallel

Proposal 11: Concurrent gaps are supported for (e)Redcap UE with FR1-NTN bands, and existing requirements for normal UE in NTN are re-used as baseline.

Proposal 12: RAN4 to discuss whether to support enhanced cell reselection measurement for (e)Redcap UE with FR1-NTN bands.

Proposal 13: Use new sections and section number with new suffix X in the specification to capture requirements for (e)RedCap UE with FR1-NTN bands. Utilize the reference method for the requirements that can be reused.

Reference

- R4-2414037, The WF on RRM requirements for Rel-19 NR NTN phase3, CATT

- R4-2412672, Initial discussion on RRM impacts of R19 NR NTN, Huawei, HiSilicon