3GPP TSG-RAN WG4 Meeting #112bis	R4-2415870

Hefei, Anhui, China, 14 October – 18 October 2024

Agenda Item:	6.8.4

Source:	Ericsson

Title:	TE-emulated channel model for NTN UE demodulation requirements

Document for:	Discussion

# 1	Introduction

RAN4#112 agreed with the way forward on the TE-emulated channel modeling for NGSO NTN testing [1], and agreed that RAN4 first focus on the varying Doppler and delay shift models.

Agreement: - Decouple TE-emulated time-varying Doppler (frequency) and delay shifts due to satellite mobility from traditional fading channel

In RAN4#112, several models were proposed to realize the varying Doppler shift and propagation delay, but it was agreed the model should be well aligned with the Doppler shift and propagation delay generated by GMAT [2].

Methodology for Time varying Doppler and Delay shifts modelling - RAN4 aims to introduce TE-emulated time varying doppler shift and delay shift model for satellite motion with following candidate options: - Option 1: Eckstein-Hechler model - Option 2: Keplerian model - Option 3: Existing model from TR 38.811 5.3.4.3 - Option 4: Proposal 2 from R4-2412552 - Option 5: Proposal 2 from R4-2412783 - Other options not precluded - Note: Companies are encouraged to bring more information for option 1 and option2. - Predefined files which generated by GMAT can be considered as reference for the comparison purpose of proposed channel model - FFS on the initial ephemeris information and parameters

This contribution focuses on the evaluations of the varying Doppler shift and propagation delay models proposed in the WF.

# 2	Varying Doppler shift and propagation delay model

## 2.1	NGSO NTN test environment

RAN4#112 discussed several options for varying Doppler shift and propagation delay modelling. In our understanding, the NTN-capable UE estimates the Doppler shift and propagation delay based on the satellite ephemeris information and DUT’s (UE’s) location information. The satellite ephemeris information is signaled via SIB19 for NR and SIB31 for IoT-NTN, and they should be generated according to the actual satellite mobility (See A.1 and A.2). The UE location information consists of longitude, latitude, and altitude, and this information is pre-configured or provided via AT command, according to TS 38.508 (See A.3). The estimated Doppler shift and propagation delay are used to perform the pre-compensation before the baseband processing in the NTN UE. Figure 1 illustrates the test environment for NGSO NTN UE testing. In our understanding RAN4 performance requirements target the ‘Baseband processing’ in the figure.

Figure 1	Assumed test environment for NGSO NTN UE testing.

As shown in Figure 1, the NTN-capable UE relies only on the ephemeris information and its configured location information to estimate and pre-compensate the Doppler shift and propagation delay due to the satellite moving. Therefore, it is very important the Doppler shift and propagation delay applied by the TE is well aligned with the satellite ephemeris data and UE location data. Otherwise, UE may fail to test because of the mismatch between Doppler shift and propagation delay applied by TE and Doppler shift and propagation delay estimated by UE.

Observation 1: During the NTN UE conformance test, NTN-capable UE (DUT) estimates and pre-compensates the Doppler shift and delay shift according to the signaled ephemeris information and configured UE location information.

Observation 2: TE-emulated varying Doppler shift and propagation delay should be well aligned with the signaled ephemeris information and configured UE location information.

## 2.2	Comparison of candidate models

This section shows our evaluation results of candidate models by comparing the Doppler shift and propagation delay with the reference GMAT data (For example, [2]), according to WF. To generate the reference GMAT data, we have reused the configuration presented in [3], except the initial satellite velocity/location (‘Sat1’ in [3]) and UE location (‘GDS’ in [3]). See also A.5 for GMAT script we have used to generate the reference data.

The evaluated options as listed as follows:

- Option 2 in WF:

  - Keplerian model, Ref: [4])

- Option 3 in WF:

  - Existing model from TR 38.811 5.3.4.3

- Option 4 in WF:

  - Proposal 2 from R4-2412552

- Option 5 in WF:

  - Proposal 2 from R4-2412783

- Interpolation method from ephemeris information:

  - Estimated the satellite location/velocity information based on the ephemeris information signaled to the DUT, where it is assumed the ephemeris information is signaled every 10.24 seconds. The detailed interpolation method is provided in A.4.

Case 1: Satellite and UE located on the same longitude/latitude

Case 1 is a scenario the satellite moves right above the UE. The initial satellite and UE locations are set as shown in Table 1. This means the satellite (LEO-600) is moving to the true north with 7.56km/s.

Table 1	Initial Satellite/UE location for case 1.

| Parameters | Values |
| --- | --- |
| Sat1.X | 7078.137 km (600km from the ground) |
| Sat1.Y | 0 km |
| Sat1.Z | 0 km |
| Sat1.VX | 0 km/s |
| Sat1.VY | 0 km/s |
| Sat1.VZ | 7.56 km/s |
| GDS Latitude | 10 deg |
| GDS Longitude | 0 deg |
| GDS Altitude | 0 m |

Figure 2 and Figure 3 shows the comparison of Doppler shift and propagation delay, respectively, between the proposed models and reference GMAT data. For the GMAT reference we plot ‘Doppler’ for Doppler shift, and ‘Delay’ for propagation delay in [3]. We should point out the Keplerian model (Option 2) and Interpolation method (‘Interp’) estimate the satellite position and velocity only. This means we also need to consider the UE location/velocity to derive the Doppler shift and propagation delay from the UE viewpoint. On the other hand, Options 2, 3, and 4 do not need to consider the UE location/velocity because these models implicitly assume the UE is stationary and located in the origin.

| - Propagation delay | - Difference from GMAT data |
| --- | --- |

Figure 2	Model comparison (Propagation delay) with GMAT (Case 1).

| - Doppler shift | - Difference from GMAT data |
| --- | --- |

Figure 3	Model comparison (Doppler shift) with GMAT (Case 1).

Figure 4 shows the elevation angle viewed from the UE, which is generated from GMAT (‘Sat_El_GDS’ in [3]). According to Figure 4, Figure 2 and Figure 3 are the case the satellite is located at the elevation angle of 25.9 degrees at time 0, becomes 90 degrees at time 163 (sec), and then becomes 15.4 degrees at time 400 (sec).

Figure 4	Satellite elevation angle from UE.

It is observed from our evaluation result that the huge deviations of Options 3, 4, and 5 from the reference GMAT. Note the curves of Options 3 and 5 are almost same. In the worst case the deviations of the propagation delay are more than 300ns (Figure 2 (b)) and the deviation of the Doppler shift is more than 5,000Hz (Figure 3 (b)). This means UE may not be able to pass the tests because of the huge deviation of Doppler shift and propagation delay between TE and UE.

On the other hand, the Doppler shift and propagation delay from Option 2 and the interpolated method are well aligned with the reference GMAT data; the errors are only up to (delay, Doppler) = (2.1ns, 27Hz) for Option 2 and (delay, Doppler) = (3.1ns, 35Hz) for the interpolation method, we think these errors are low enough to be considered as ‘impairment margin’.

Please note that we assume UE is stationary and keep the UE location at the initialization because the impact of UE mobility (including the earth rotation) is negligible compared with satellite velocity if we assume short test time - a few hundred seconds.

Case 2: Satellite and UE located on the different longitude/latitude

Case 2 consider more realistic scenario – the satellite does not move on the top of the UE, as shown in Table 2.

Table 2	Initial Satellite/UE location for case 2.

| Parameters | Values |
| --- | --- |
| Sat1.X | 7078.137 km |
| Sat1.Y | 0 km |
| Sat1.Z | 0 km |
| Sat1.VX | 0 km/s |
| Sat1.VY | 0 km/s |
| Sat1.VZ | 7.56 km/s |
| GDS Latitude | 10 deg |
| GDS Longitude | 4 deg |
| GDS Altitude | 0 m |

| - Propagation delay | - Difference from GMAT data |
| --- | --- |

Figure 5	Model comparison (Propagation delay) with GMAT (Case 2).

| - Doppler shift | - Difference from GMAT data |
| --- | --- |

Figure 6	Model comparison (Doppler shift) with GMAT (Case 2).

Figure 5 and Figure 6 show the comparison of Doppler shift and propagation delay, respectively, between the reference GMAT data and proposed Options 2, 3, 4, and 5. As observed from the figures, the deviations of options 3, 4, and 5 are even worse compared with Case 1 because these Options assume the satellites move above the UE.

On the other hand, the Doppler shift and propagation delay from Option 2 and the interpolated method are well aligned with the reference GMAT data; the errors are only up to (delay, Doppler) = (1.1ns, 16Hz) for Option 2 and (delay, Doppler) = (2.0ns, 23Hz) for interpolation method.

## 2.3	Discussion

From our evaluation results, it is observed that we need to consider the UE location to align the Doppler shift and propagation delay with the reference GMAT data precisely. The Keplerian based satellite location/velocity estimation is very accurate as expected, and thus the estimation is well aligned with the GMAT reference data. However if we assume the ephemeris information is updated every 10.24 seconds, the simple interpolation method is also good enough to emulate the accurate Doppler shift and propagation delay. Since the Keplerian-based modeling is a bit complex (For example, [4]) to specify in e.g., TS38.101-5, we prefer to apply the simple interpolation method. Figure 7 shows an example of NGSO UE test procedure, but the detail procedure should be discussed in RAN5.

Figure 7	Example of procedure for NGNO UE testing.

Observation 3: Doppler shifts and propagation delays generated from Options 3, 4, and 5 are deviated significantly compared with the reference GMAT data.

Observation 4: Need to consider the UE location information to align with the reference GMAT data.

Observation 5: Doppler shifts and propagation delays generated from Keplerian model are well aligned with the reference GMAT.

Observation 6: Doppler shifts and propagation delays generated from the interpolated ephemeris data are well aligned with the reference GMAT with the assumption the ephemeris is updated every 10.24 seconds.

Proposal 1: TE should generate the Doppler shift and propagation delay from the satellite ephemeris information and UE location information. Consider the Keplerian model or interpolation method to generate the Doppler shift and propagation delay.

Proposal 2: Same UE location information should be set for both DUT and TE before the test start and it does not need to update during the tests if the test time is short.

Proposal 3: Satellite ephemeris information should be updated every 10.24 seconds.

As we discussed in the last meeting, since the Doppler shift and propagation delay model discussed in this WI tries to emulate the actual satellite moving scenario, the simulation time may become longer compared with fixed-satellite based scenarios. For example, according to Figure 4, if we cover the elevation angels above 30 degrees, the test time is about 300 seconds. We need to discuss the range of satellite elevation angles considering the test time. Alternatively, RAN4 only need to specify the varying Doppler shift and propagation delay model only, and the detailed parameters (e.g., satellite location/velocity and UE location) are left to RAN5 as of today.

Proposal 4: RAN4 discuss whether RAN4 decides the detailed parameters (e.g., satellite location/velocity and UE location) of varying Doppler shift and propagation delay model, or it is left to RAN5.

In this contribution, we evaluated the Doppler shift and propagation delay from one satellite only. RAN4 may need to discuss whether we consider the scenario the serving satellite changes.

Proposal 5: RAN4 discuss whether to consider the scenario the serving satellite changes, i.e., multiple satellite scenario.

# 3	Summary

Observation 1: During the NTN UE conformance test, NTN-capable UE (DUT) estimates and pre-compensates the Doppler shift and delay shift according to the signaled ephemeris information and configured UE location information.

Observation 2: TE-emulated varying Doppler shift and propagation delay should be well aligned with the signaled ephemeris information and configured UE location information.

Observation 3: Doppler shifts and propagation delays generated from Options 3, 4, and 5 are deviated significantly compared with the reference GMAT data.

Observation 4: Need to consider the UE location information to align with the reference GMAT data.

Observation 5: Doppler shifts and propagation delays generated from Keplerian model are well aligned with the reference GMAT.

Observation 6: Doppler shifts and propagation delays generated from the interpolated ephemeris data are well aligned with the reference GMAT with the assumption the ephemeris is updated every 10.24 seconds.

Proposal 1: TE should generate the Doppler shift and propagation delay from the satellite ephemeris information and UE location information. Consider the Keplerian model or interpolation method to generate the Doppler shift and propagation delay.

Proposal 2: Same UE location information should be set for both DUT and TE before the test start and it does not need to update during the tests if the test time is short.

Proposal 3: Satellite ephemeris information should be updated every 10.24 seconds.

Proposal 4: RAN4 discuss whether RAN4 decides the detailed parameters (e.g., satellite location/velocity and UE location) of varying Doppler shift and propagation delay model, or it is left to RAN5.

Proposal 5: RAN4 discuss whether to consider the scenario the serving satellite changes, i.e., multiple satellite scenario.

# References

- R4-2413516, “Way Forward for [112][329] NTN_testing_NGSO_channel_model”, Samsung.

- https://gmat.atlassian.net/wiki/spaces/GW/overview

- R5-237216, “NGSO satellite Ephemeris file generation methodology for NTN NR UE testing”, THALES.

- Roger R. Bate, Donald D. Mueller, Jerry E. White, William W. Saylor, “Fundamentals of Astrodynamics: Second Edition”, Dover Publication, 2020.

# Appendix

## A.1	SIB19 NTN Ephemeris Information for zero Doppler conditions specified in TS 38.508

Table 5.6.2.1-1: SIB19- NR NTN Ephemeris information for GSO satellites (zero Doppler conditions)

| Derivation Path: Table 5.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| ntn-Config-r17 SEQUENCE { |  |  |  |
| epochTime-r17 | Not present |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -16944523 |  |  |
| positionY-r17 | 27362209 |  |  |
| positionZ-r17 | 4019393 |  |  |
| velocityVX-r17 | -336 |  |  |
| velocityVY-r17 | -208 |  |  |
| velocityVZ-r17 | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 35.04 degrees, one-way delay equal to 127.33 ms and Doppler equal to 4.89E-06 ppm |  |  |  |

Table 5.6.2.1-2: SIB19- NR NTN Ephemeris information for NGSO (LEO-600) satellites (zero Doppler conditions)

| Derivation Path: Table 5.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| ntn-Config-r17 SEQUENCE { |  |  |  |
| epochTime-r17 | Not present |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -2550081 |  |  |
| positionY-r17 | 4151097 |  |  |
| positionZ-r17 | 2263034 |  |  |
| velocityVX-r17 | 21970 |  |  |
| velocityVY-r17 | -48164 |  |  |
| velocityVZ-r17 | 114109 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 89.61 degrees, one-way delay equal to 2.03 ms and Doppler equal to 0.013 ppm |  |  |  |

Table 5.6.2.1-3: SIB19- NR NTN Ephemeris information for NGSO (LEO1200) satellites (zero Doppler conditions)

| Derivation Path: Table 5.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| ntn-Config-r17 SEQUENCE { |  |  |  |
| epochTime-r17 | Not present |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -2771542 |  |  |
| positionY-r17 | 4509243 |  |  |
| positionZ-r17 | 2455572 |  |  |
| velocityVX-r17 | 20837 |  |  |
| velocityVY-r17 | -46176 |  |  |
| velocityVZ-r17 | 109528 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 89.54 degrees, one-way delay equal to 4.04 ms and Doppler equal to 3.6E-3 ppm |  |  |  |

## A.2	SIB19 NTN Ephemeris Information for zero Doppler conditions for UL frequency error test specified in TS 38.521-5

Table 6.4.1_2.4.3-1: SIB19 for RF tests with NR NTN Ephemeris Information for NGSO (LEO-600) satellites (maximum positive Doppler)

| Derivation Path: TS 38.508-1, Table 5.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EphemerisInfo-r17 ::= CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -2717617 |  |  |
| positionY-r17 | 4550419 |  |  |
| positionZ-r17 | 852799 |  |  |
| velocityVX-r17 | 6164 |  |  |
| velocityVY-r17 | -19424 |  |  |
| velocityVZ-r17 | 124281 |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 10 degrees, one-way delay equal to 6.44 ms and Doppler equal to 22.65 ppm. |  |  |  |

Table 6.4.1_2.4.3-2: SIB19 for RF tests with NR NTN Ephemeris Information for NGSO (LEO-600) satellites (maximum negative Doppler)

| Derivation Path: TS 38.508-1, Table 5.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EphemerisInfo-r17 ::= CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -2199272 |  |  |
| positionY-r17 | 3404229 |  |  |
| positionZ-r17 | 3535794 |  |  |
| velocityVX-r17 | 35394 |  |  |
| velocityVY-r17 | -74414 |  |  |
| velocityVZ-r17 | 94682 |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 169.97 degrees, one-way delay equal to 6.60 ms and Doppler equal to -22.62 ppm. |  |  |  |

Table 6.4.1_2.4.3-3: SIB19 for RF tests with NR NTN Ephemeris Information for NGSO (LEO-600) satellites (maximum positive Doppler/2)

| Derivation Path: TS 38.508-1, Table 5.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EphemerisInfo-r17 ::= CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -2592823 |  |  |
| positionY-r17 | 4245650 |  |  |
| positionZ-r17 | 2024520 |  |  |
| velocityVX-r17 | 19359 |  |  |
| velocityVY-r17 | -43278 |  |  |
| velocityVZ-r17 | 116553 |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 60.25 degrees, one-way delay equal to 2.30 ms and Doppler equal to 11.29 ppm. |  |  |  |

## A.3	UE location (TS 38.508-1)

### 5.6.1 UE location UE shall determine its location during the test using any of the following means. 1. UE location for the test shall be provided to the UE via pre-configured means. During the test the UE location is not expected to change unless explicitly stated as a requirement for the test. 2. Other options such as providing UE location via AT command are not precluded. UE location provided to the UE before the start of the test is as follows for RF Tx/Rx and DEMOD test cases for both GSO and NGSO config is as follows: Longitude: 121.56076999 Latitude: 25.08439333 (NGSO satellites), 55.0 (GSO satellites) Altitude: 0

## A.4	Interpolation based method

Table 3	Input parameters

| Parameters | Description | Example of values |
| --- | --- | --- |
| R | Earth radius (km) | 6378.137 |
| e | Earth eccentricity | 0.0167 |
| UElatitude | UE latitude (degree) | 10.0 |
| UElontitude | UE longitude (degree) | 4.0 |
| UEaltitude | UE altitude (km) | 0.0 |
| Satx(tref) | X coordinate of satellite position at time tref (positionX in EphemerisInfo) | 7078.137 |
| Saty(tref) | Y coordinate of satellite position at time tref (positionY in EphemerisInfo) | 0.0 |
| Satz(tref) | Z coordinate of satellite position at time tref (positionZ in EphemerisInfo) | 0.0 |
| Satvx(tref) | X coordinate of satellite velocity at time tref (velocityVX in EphemerisInfo) | 0.0 |
| Satvy(tref) | Y coordinate of satellite velocity at time tref (velocityVY in EphemerisInfo) | 0.0 |
| Satvz(tref) | Z coordinate of satellite velocity at time tref (velocityVZ in EphemerisInfo) | 7.56 |
| Tephemeris | Ephemeris update period (sec) | 10.24 |
| c | Speed of light (km/s) | 300000 |
| fc | Carrier frequency (Hz) | 2e9 |

Table 4	Output values

| Parameters | Description |
| --- | --- |
| Delay(t) | Propagation delay (sec) at time t |
| Doppler(t) | Doppler shift (Hz) at time t |

ϕ=π180UElatitude

λ=π180UElongitude

N=R1-e2sin2ϕ

UEx=N+UEaltitudecosϕcosλ UEy=N+UEaltitudecosϕsinλUEz=N1-e2+UEaltitudesinϕ

Sataxtref=Satvxtref-Satvztref-TephemerisTephemerisSataytref=Satvytref-Satvytref-TephemerisTephemerisSataztref=Satvztref-Satvztref-TephemerisTephemeris

Satvxt=Satvxtref+Sataxtref⋅(t-tref)Satvyt=Satvytref+Sataytref⋅(t-tref)Satvzt=Satvztref+Sataztref⋅(t-tref)

Satxt=Satxtref+Satvxtref⋅(t-tref)Satyt=Satytref+Satvytref⋅(t-tref)Satzt=Satztref+Satvztref⋅(t-tref)

ρxt=Satxt-UExρyt=Satyt-UEyρzt=Satzt-UEz

Ranget=ρx2t+ρy2t+ρz2t

RangeRate(t)=Satxt⋅Satvxt+Satxt⋅Satvyt+Satxt⋅SatvztRange(t)

Delayt=Rangetc

Dopplert=-RangeRatet×fcc

## A.5	Example of GMAT script

This is the part of GMAT script to generate the reference data shown in Figure 2 and Error! Reference source not found..

%---------------------------------------- %---------- Arrays, Variables, Strings %---------------------------------------- Create Variable Range_km Delay_ms DelayRate Sat_El_GDS Sat_Az_GDS x2 y2 z2 Vx2 Vy2; Create Variable Vz2 ax2 ay2 az2 x1 y1 z1 Vx1 Vy1 Vz1; Create Variable Doppler_hz DopplerRate DelayShift Min_El Max_El LightCelerity_kmps CarrierFreq_hz; GMAT Min_El = 10; GMAT Max_El = 180; GMAT LightCelerity_kmps = 300000; GMAT CarrierFreq_hz = 2000000000; %---------------------------------------- %---------- Mission Sequence %---------------------------------------- BeginMissionSequence; While Sat1.ElapsedSecs <= 1000 GMAT 'Initialize x2 with Sat1.GDSLocalFrame.X' x2 = Sat1.GDSLocalFrame.X; GMAT 'Initialize y2 with Sat1.GDSLocalFrame.Y' y2 = Sat1.GDSLocalFrame.Y; GMAT 'Initialize z2 with Sat1.GDSLocalFrame.Z' z2 = Sat1.GDSLocalFrame.Z; GMAT 'Initialize Vx2 with Sat1.GDSLocalFrame.VX' Vx2 = Sat1.GDSLocalFrame.VX; GMAT 'Initialize Vy2 with Sat1.GDSLocalFrame.VY' Vy2 = Sat1.GDSLocalFrame.VY; GMAT 'Initialize Vz2 with Sat1.GDSLocalFrame.VZ' Vz2 = Sat1.GDSLocalFrame.VZ; GMAT 'Initialize ax2 with Sat1.GDSLocalFrame.VX' ax2 = Sat1.LowEarthProp1_ForceModel.AccelerationX; GMAT 'Initialize ay2 with Sat1.GDSLocalFrame.VY' ay2 = Sat1.LowEarthProp1_ForceModel.AccelerationY; GMAT 'Initialize az2 with Sat1.GDSLocalFrame.VZ' az2 = Sat1.LowEarthProp1_ForceModel.AccelerationZ; GMAT 'Initialize x1 with Sat1.EarthFixed.X' x1 = Sat1.EarthFixed.X; GMAT 'Initialize y1 with Sat1.EarthFixed.Y' y1 = Sat1.EarthFixed.Y; GMAT 'Initialize z1 with Sat1.EarthFixed.Z' z1 = Sat1.EarthFixed.Z; GMAT 'Initialize Vx1 to 0' Vx1 = Sat1.EarthFixed.VX; GMAT 'Initialize Vy1 to 0' Vy1 = Sat1.EarthFixed.VY; GMAT 'Initialize Vz1 to 0' Vz1 = Sat1.EarthFixed.VZ; GMAT 'Calculate satellite Local Elevation' Sat_El_GDS = RadToDeg(asin(z2/sqrt(x2^2+y2^2+z2^2))); GMAT 'Calculate satellite Local Azimuth' Sat_Az_GDS = RadToDeg(acos(x2/sqrt(x2^2+y2^2+z2^2))); If 'Check visibility' Sat_El_GDS >= Min_El & Sat_El_GDS <= Max_El GMAT 'Calculate Range' Range_km = sqrt((x2)^2+(y2)^2+(z2)^2); GMAT 'Calculate Doppler' Doppler_hz = - CarrierFreq_hz*(x2*Vx2+y2*Vy2+z2*Vz2)/Range_km/LightCelerity_kmps; GMAT 'Calculate Delay' Delay_ms = 1000*Range_km/LightCelerity_kmps; GMAT 'Calculate Delay Shift' DelayShift = Delay_ms*Doppler_hz; Else GMAT 'Non visible set Range to 0' Range_km = 0; GMAT 'Non visible set Doppler to 0' Doppler_hz = 0; GMAT 'Non visible set Doppler Rate to 0' DopplerRate = 0; GMAT 'Non visible set Delay to 0' Delay_ms = 0; GMAT 'Non visible set DelayShift to 0' DelayShift = 0; GMAT 'Non visible set DelayRate' DelayRate = 0; EndIf; Propagate 'LowEarthProp1 Sat1' LowEarthProp1(Sat1); EndWhile;

Estimate satellite acceleration based on the satellite velocity information using the latest ephemeris information at (tref) and the previous ephemeris information at (tref-Tephemeris).