3GPP TSG-RAN WG4 Meeting #112bis	R4-2415871

Hefei, Anhui, China, October 14th – 18th 2024

Agenda Item:	6.17.3.1

Source:	Ericsson

Title:	Discussion on CSI prediction

Document for:	Discussion

# 1	Introduction

During RAN#105, normative work on CSI prediction was agreed [1]. Until this time, there has not been much discussion on the CSI prediction in RAN4. An initial WF was agreed on requirement metric at RAN4#112:

Use relative throughput as requirement metric for CSI prediction

- companies are invited to bring contribution on how the requirement could be defined:

  - what is the baseline that throughput would be compared to

This contribution discusses several aspects of RAN4 work on CSI prediction, including:

- Performance requirement metric and definition

- Considerations on the needed range of requirement conditions

- “Monitoring” considerations

Although the objective calls the CSI prediction, RAN1 has focused on the PMI prediction by reusing the Rel-18 Enhanced Type II codebook for predicted PMI. In this contribution, we therefore discuss the AI/ML-based PMI prediction.

# 2. Performance requirement metric

When defining the performance requirement metric, it is important to bear in mind that the basic purpose of the requirement is to enable interoperability by means of ensuring that UEs present consistent behaviour and performance. The purpose of the requirement is not to establish a “gain” compared to the legacy non-AI-based PMI reporting. In deciding to standardize the use case, RAN1 and RAN have already identified that gain is achieved, and the purpose of RAN4 requirements is to establish consistent behaviour.

The goal with the RAN4 requirement is not to demonstrate AI performance relative to performance with non-AI (“show a gain”), but it is to establish a minimum performance level for the AI/ML-based CSI (PMI) prediction.

It was agreed at RAN4#112 that the metric will be based on relative throughput, but the baseline for the relative throughput (i.e., denominator of throughput ratio) was not agreed [2].

Issue 4-5: CSI prediction accuracy metric Agreement: Use relative throughput as requirement metric for CSI prediction - companies are invited to bring contribution on how the requirement could be defined: - what is the baseline that throughput would be compared to

One option for the baseline is e.g. Rel-16 eType II codebook. However, it may not be the case that the UE under test implements this codebook because it is optional capability. Furthermore, there is no need to use an eTypeII codebook since the purpose is not to establish a “gain” from AI.

In our understanding, defining the throughput relative to random PMI is sufficient and satisfactory to ensure proper performance with the predicted CSI.

For the performance metric, use throughput with AI predicted CSI (PMI) relative to random PMI with e.g., Rel-15 Type I Single-Panel Codebook.

# 3. Range of testing conditions

The existing non-AI framework has requirements defined for a couple of specific channel models and SNR levels. For example, RAN4 usually use the low-dispersion and low-Doppler spread channel models such as TDLA30 with 5Hz or 10Hz for FR1 PMI reporting tests. It is assumed that, if the PMI calculation or prediction works well in these conditions then the PMI will perform similarly in other conditions.

For AI, on the other hand, there is a need to understand whether a model that is trained and can work well in one specific channel and SNR can also work consistently in other scenarios (generality). The scenario in which the PMI prediction operates may change for any of the following reasons:

- Channel variation: depending on the location the multipath profile of the channel may look different.

- SNR level (SNR test point): depending on the location, the SNR may differ.

- SNR changes: due to the activity in neighbour cells, the SNR may shift rapidly with time, in particular for cell-edge UEs.

- Doppler variation: the UE Doppler may differ depending on the location.

- Differences in the antenna array design for the BS serving the UE.

Examining each of these in detail as follows:

Channel variation: A study is needed whether the prediction performance for an AL prediction model can vary significantly depending on the multi-path channel profile. If this is the case, then requirements are needed for a variety of multi-path profiles. Moreover RAN4 may need to study the AI model(s) trained with CDL-based dataset work well for TDL-based test setup, and vice versa.

SNR level: A study is needed whether the prediction performance is consistent across the SNR levels. If not, the requirements need to be defined at several SNR test points. For example, the exiting NR PMI reporting tests set the test point to one SNR point achieving 90% of the peak rate with follow PMI. We may need to consider lower SNR test points, e.g., 30%, 60% of the peak rate with follow PMI.

SNR changes: If the prediction performance does not vary with SNR then no consideration is needed. However, if the performance does depend on SNR, then the requirement may need to also specify performance when SNR changes significantly due to e.g. varying neighbour cell scheduling. A realistic model for the SNR change profile would be needed. Note although we have no such requirements in NR and LTE, 3G HSPA CQI reporting test verifies the reported CQI with varying SNR scenario (See TS 25.101 9.3.1.3).

Doppler variation: A study is needed on the sensitivity of the prediction to Doppler. Requirements may be needed for several levels of Doppler spread. Instead of meeting all requirements, a UE capability could be created relating to the ability of the CSI prediction to handle different Doppler levels and the requirements would apply accordingly. In our understanding, RAN1 study the CSI prediction with 30km/h and 60km/h. However RAN4 performance test may need to consider more scenarios such as higher/lower UE mobility such as 120km/h (high speed vehicles) or 3km/h (pedestrian).

Study the variation of performance of CSI prediction with different channel model, SNR level, changes in SNR and Doppler with well trained models and with over-fitted models.

Depending on the results of the study, define sufficient requirement variations that proper model training (i.e., a model/functionality that can perform well across variations in cell conditions) is achieved.

Consider whether to make conformance to CSI prediction with lower/higher Doppler scenario, according to RAN1 study.

# 4. “Monitoring” considerations

“Monitoring” is a term used to refer to the action on either the UE or network side of determining whether the model, in current operation can continue to meet performance expectations. It is important to consider that monitoring should not just indicate whether the model is functioning or not in a binary sense. If the predicted CSI is obviously trash and the performance is poor, then it is obvious that a switch back to non-AI functionality (fallback) is needed as illustrated in Figure 1. The challenge with monitoring is to detect when the performance has degraded to a non-negligible, but not catastrophic extent (e.g. 5-10% degradation).

Figure 1	Monitoring of AI/ML-based CSI prediction model.

It is important that monitoring can observe and quantify partial reduction in performance (not just a large or catastrophic loss).

Such wrong predictions could lead to unacceptable degradations in network performance. However, identifying that degradations have come from a specific AI model may be difficult to achieve. What may also be difficult is to identify whether changing to non-AI may actually improve performance or not. The reason is that UE performance (i.e., data throughput) depends not only on the reported CSI, but on a very large range of factors, including but not limited to e.g. the radio environment, interference conditions, RF performance, adjacent channel interferers, network traffic and scheduling and potentially interaction between a variety of AI algorithms in future UEs.

It is non-trivial to ensure that a monitoring function can determine the performance of an AI model in isolation, since in the real world a very large number of interacting factors influence performance.

Monitoring of CSI prediction is simpler than 2-sided monitoring since a single model needs to be monitored. Nevertheless, some attention from RAN4 may be needed.

Since RAN plenary agreed to include the CSI prediction in the scope of the WI AI/ML for air interface, we expect RAN1/2 are going to specify the performance monitoring mechanism for CSI prediction. From a RAN4 perspective, though, the following considerations are important although we need to wait for the RAN1/RAN2 conclusion. If the UE sends measurements, or monitoring reports to the network (e.g., NMSE or SGCS), then it is important that the measurements or reports are consistent and comparable between different UE types like other CSI quantities such L1-RSRP/L1-SINR as defined in TS38.215. For measurements, this may necessitate setting requirements on e.g. accuracy and reporting delay in a similar style to RRM measurement requirements. For UE monitoring reports, it is important that different UEs provide similar reports when their models have a similar level of reliability. However, since the reliability of different UEs models may differ, a discussion is needed on how requirement definition and testing could be carried out in a way that forces UEs to specific reliability levels and then tests the output. Also, a definition of metrics for testing of the output is needed.

Monitoring requirements should define the constancy/accuracy of monitoring reports from the UE if they are defined.

Testability of monitoring report accuracy needs discussion, since the reliability of underlying models will differ between UEs and a test of accuracy needs to force UEs into known reliability levels.

If monitoring is entirely on the network side, then it may be discussed in RAN1/2. Since the network needs to manage a large amount of UEs, the complexity of monitoring, ability to diagnose specific models within the UE need improvement and also identify what improvement is needed may be challenges.

In particular, to perform network sided monitoring it may be necessary that the UE, at certain times, provides both predicted CSI value (CSI value for time t+δ, predicted in report time t) and measured CSI value (CSI value based on the CSI-RS measurement at time t+δ). This would be needed because the underlying ground truth, CSI is calculated internally in the UE and so not known to the network. It may be also debated whether it is useful to set requirements to ensure that both the reported CSI measured at t+δ for time t+δ and the CSI predicted at t for time t+δ meet performance requirements when the UE is operating in this mode.

Figure 2	Example of network-sided monitoring.

For network sided monitoring, if the UE reports both predicted and measured CSI, there may be some need to consider setting conformance requirements on the performance of both measured and predicted when operating in this monitoring mode.

# Conclusion

In the previous sections we made the following observations:

Observation 1	The goal with the RAN4 requirement is not to demonstrate AI performance relative to performance with non-AI (“show a gain”), but it is to establish a minimum performance level for the AI/ML-based CSI (PMI) prediction.

Observation 2	It is important that monitoring can observe and quantify partial reduction in performance (not just a large or catastrophic loss).

Observation 3	It is non-trivial to ensure that a monitoring function can determine the performance of an AI model in isolation, since in the real world a very large number of interacting factors influence performance.

Observation 4	Monitoring requirements should define the constancy/accuracy of monitoring reports from the UE if they are defined.

Observation 5	Testability of monitoring report accuracy needs discussion, since the reliability of underlying models will differ between UEs and a test of accuracy needs to force UEs into known reliability levels.

Observation 6	For network sided monitoring, if the UE reports both predicted and measured CSI, there may be some need to consider setting conformance requirements on the performance of both measured and predicted when operating in this monitoring mode.

Based on the discussion in the previous sections we propose the following:

Proposal 1	For the performance metric, use throughput with AI predicted CSI (PMI) relative to random PMI with e.g., Rel-15 Type I Single-Panel Codebook.

Proposal 2	Study the variation of performance of CSI prediction with different channel model, SNR level, changes in SNR and Doppler with well trained models and with over-fitted models.

Proposal 3	Depending on the results of the study, define sufficient requirement variations that proper model training (i.e., a model/functionality that can perform well across variations in cell conditions) is achieved.

Proposal 4	Consider whether to make conformance to CSI prediction with lower/higher Doppler scenario, according to RAN1 study.

# References

- RP-242399, “Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm.

- R4-2414323, “WF on testability and requirements for AIML air interface”, Qualcomm.