3GPP TSG-RAN WG4 Meeting # 112-bis	R4- 2415879

# Hefei, Anhui, China, 14th – 18th Octorber, 2024

Source:	LG Electronics

Title:	Discussion on RRM requirements for FR2-1 L3 measurement delay reduction

Agenda item:	6.15.2.1

Document for:	Discussion

Introduction

In the last RAN4 meeting, RAN4 discussed and agreed on general scenarios [1]. In this contritubtion, we provide our views on activating condition for multi-Rx simultaneous reception and how to define core requirements for L3 measurement delay reduction.

Discussion

RRM measurement with two panels activated, two searchers are occupied by this single carrier

[Way forward]: - Option A/1a: Two searchers are occupied by this single carrier to the requirements of enhanced BSF for single carrier. Discuss CA case later. - Option B: Consider one searcher for single carrier.

Based on the agreement of applicability requirement and the WID, the main target is L3 measurement delay reduction by Rx BSF for single carrier in FR2-1 as below.

Applicability requirement: Updated Agreement in online session: Baseline: L3 delay enhancements in Rel-19 by optimizing Rx BSF for UE supporting multi-rx simultaneous reception are applicable provided that: - the target carrier(s) to be measured: only one carrier in the single FR2-1 band is configured for L3 SSB measurement and - UE serving carrier(s): UE is configured with single carrier on FR2-1 band, i.e. FR2-1 PCell without CA/DC. Note: Target and serving carrier frequency can be the same or different. Note: The ‘other UE CA/DC modes (e.g., 1 or 2 FR2-1 bands CA, or FR1+FR2 CA/DC, or EN-DC)’ and/or the ‘other number of target to-be-measured carrier(s) on FR2-1 band’ can be FFS after concluding the baseline above. These extra FFS parts will NOT delay the WI completion. WID FR2-1 SSB based L3 measurement delay reduction for connected mode - For UE supporting multiple-Rx simultaneous reception on single carrier: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - Rx beam sweeping factor

And to reduce L3 measurement using multi-Rx simultaneous reception, two searchers should be occupied by the measured single carrier.

- Proposal 1: Two searchers are occupied by this single carrier to the requirements of enhanced BSF for single carrier to reduce L3 measurement delay in FR2-1.

Power consumption issue (including conditions to trigger UE using FBS for L3 measurement)

[Way forward]: discussion can be mainly focus on 3 directions: (1)“additional triggering for this R19 L3 measurement with FBS” (use option 8 for discussion) or (2)“L3 measurement with FBS can be activated/deactivation following on R18 mechanism” (use option 1 for discussion) or (3)“up to network indication” (use option 3 for discussion). Discuss option 1/3/8 together, and then add details from other options if needed.

RAN4 needs to define the condition to trigger multi-Rx simultaneous reception for L3 measurement since UE cannot always turn on the multi-Rx chains due to power consumption issue. In the last meeting, RAN4 agreed that the condition of in active mode defined in Rel-18 multi-Rx simultaneous reception is not assumed as below.

Agreement: - The conditions for UE to apply L3 measurement delay reduction by optimizing Rx BSF is that multi-Rx simultaneous reception of UE is in active mode. And it does not assume that the condition of in active mode is same as that for Rel-18 multi-Rx simultaneous reception.

Therefore, following two candidate options for activating multi-Rx simultaneous reception for L3 measurement could be considered based on [Way forward] from the last meeting.

1) additional triggering condition for activating multi-Rx simultaneous reception for L3 measurement

2) network indication to trigger multi-Rx simultaneous reception for L3 measurement

Considering scenarios to use L3 measurement delay reduction below, both candidate options for activating multi-Rx simultaneous reception for L3 measurement could be used. The FR2-1 L3 measurement delay reduction would be required for supporting UE mobility when UE is experiencing lower signal quality to serving cell (i.e., cell edge region). Therefore, if UE supports multi-Rx reception simultaneously for L3 measurement and signal quality for serving cell is low, L3 measurement delay reduction by optimizing Rx BSF could be applied. And to reduce handover delay, network can indicate to trigger multi-Rx simultaneous reception for target cell with handover command.

Agreement: - Scenario 1: SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra - Scenario 2: SSB based Intra-frequency measurement with MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra - Scenario 3: SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter - Scenario 4: SSB based Inter-frequency measurement with MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter - Scenario 5: Handover - Scenario 7: RRC Re-establishment/RRC Connection Release with Redirection

- Proposal 2: Based on the scenario to use L3 measurement delay reduction, network indication and/or triggering condition for activating multi-Rx simultaneous reception can be considered; for examples,

  - Introduce triggering condition for activating multi-Rx simultaneous reception when serving cell quality is lower for scenario 1~4.

  - Introduce network indication to trigger multi-Rx simultaneous reception for L3 measurement to reduce handover delay/interruption time with handover command.

If UE needs power saving during L3 measurement with multi-Rx simultaneous reception, Rel-18 UAI ‘multiRx-PreferenceFR2’ for power saving can be considered as starting point.

- Proposal 3: For power saving for operating multi-Rx simultaneous reception, Rel-18 UAI ‘multiRx-PreferenceFR2’ for power saving can be considered as starting point.

Solutions to apply/specify L3 measurement delay reduction by optimizing Rx BSF

The SSB based intra-frequency measurement requirements without gaps for FR2-1 in Table 1 has been defined considering UE Rx beam sweeping.

Table 1 Measurement period for intra-frequency measurements without gaps (FR2)

| DRX cycle | T SSB_measurement_period_intra |
| --- | --- |
| No DRX | max(400ms, ceil(Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra |
| DRX cycle≤ 320ms | max(400ms, ceil(1.5x Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320ms | ceil(Mmeas_period_w/o_gaps xKp x Klayer1_measurement ) x DRX cycle x CSSFintra |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

M values depending on power class are below.

Mmeas_period_w/o_gaps: For a UE supporting FR2-1 power class 1 or 5, Mmeas_period_w/o_gaps =40. For a UE supporting FR2-1 power class 2, Mmeas_period_w/o_gaps =24. For a UE supporting FR2-1 power class 3, Mmeas_period_w/o_gaps =24. For a UE supporting power class 4, Mmeas_period_w/o_gaps =24. For a UE supporting FR2-2 power class 1, Mmeas_period_w/o_gaps = 60. For a UE supporting FR2-2 power class 2, Mmeas_period_w/o_gaps = 36. For a UE supporting FR2-2 power class 3, Mmeas_period_w/o_gaps = 36.

For UE supporting FR2-1 power class 3, M=24, and it means that 4 measurement samples with 6 Rx beam sweeping or 3 measurement samples with 8 Rx beam sweeping. For example, for UE supporting multi-Rx simultaneous reception, the less Rx BSF can be considered and it could be UE capability, so 3 or 4 Rx BSF value or additional scaling factor (<1) could be considered to reduce M value. RAN4 needs to discuss how to reduce M values for SSB based intra- / inter-frequency measurements for UE supporting multi-Rx simultaneous reception. Then the solution can be applied for other scenarios (scenario 5 and 7).

- Proposal 4: RAN4 to discuss how to reduce M values for SSB based intra- / inter-frequency measurements for UE supporting multi-Rx simultaneous reception. For example, 3 or 4 Rx BSF value or additional scaling factor < 1 to reduce M value.

Conclusion

In this contribution, we provide views on activating condition for multi-Rx simultaneous reception and how to define core requirements for L3 measurement delay reduction, and we propose

- Proposal 1: Two searchers are occupied by this single carrier to the requirements of enhanced BSF for single carrier to reduce L3 measurement delay in FR2-1.

- Proposal 2: Based on the scenario to use L3 measurement delay reduction, network indication and/or triggering condition for activating multi-Rx simultaneous reception can be considered; for examples,

  - Introduce triggering condition for activating multi-Rx simultaneous reception when serving cell quality is lower for scenario 1~4

  - Introduce network indication to trigger multi-Rx simultaneous reception for L3 measurement to reduce handover delay/interruption time with handover command

- Proposal 3: For power saving for operating multi-Rx simultaneous reception, Rel-18 UAI ‘multiRx-PreferenceFR2’ for power saving can be considered as starting point.

- Proposal 4: RAN4 to discuss how to reduce M values for SSB based intra- / inter-frequency measurements for UE supporting multi-Rx simultaneous reception. For example, 3 or 4 Rx BSF value or additional scaling factor < 1 to reduce M value.

Reference

[1] R4-2414026, “WF for NR_RRM_Ph5_Part1,” Apple.