3GPP TSG-RAN WG4 Meeting #112bis	R4-2415880

Hefei, China, 14th-18th, October 2024

Title: 	Analysis on additional operating NR bands for HAPS (High Altitude Platform Station)

Agenda Item:	5.22.1

Source: 	SoftBank Corp.

Document for:	Discussion

- Introduction

This contribution provides our views and proposal of WI on introduction of HAPS additional operating bands [1], based on analysis of a comparison of the existing/specified 2 GHz with other additional bands, as well as cross-link-interference (CLI) between HAPS and TN when HAPS uses TDD bands.

- Discussion

In Rel-17 NTN WI, HAPS ACLR and ACS in 2 GHz band were proposed as 24.97 dB and 30 dB, respectively, based on the results of co-existing study, but finally the same value as TN, i.e., ACLR = 45 dB (20 dB higher) and ACS = 46 dB (16 dB higher) were agreed upon [2].

Observation 1: ACLR and ACS requirements for HAPS BS in the 2 GHz band were specified with a large margin compared to coexistence study results.

The parameters used in the above study are basically the same as those used in the ITU-R study. In the ITU-R study, the parameters were developed for three bands, Band 1 (694-960 MHz), Band 2 (1710-1980 MHz, 2010-2025 MHz, 2110-2170 MHz), and Band 3 (2500-2690 MHz), as shown in the table below [3].

Deployment related parameters of HIBS and the UE associated to HIBS (Note: HIBS stands for HAPS as IMT Base Station)

| No. | Parameter | Band 1 (694-960 MHz) | Band 2 (1 710-1 980 MHz 2 010-2 025 MHz 2 110-2 170 MHz) | Band 3 (2 500-2 690 MHz) |
| --- | --- | --- | --- | --- |
| 1 | Network topology and characteristics |  |  |  |
| 1.1 | BS density or ISD | 1 BS/HIBS area |  |  |
| 1.2 | HIBS area radius | 100 km | 100 km | 90 km |
| 1.3 | HIBS Network Configuration (Duplex Mode) | FDD/TDD | FDD/TDD | FDD/TDD |
| 2 | Base station characteristics/Cell Structure |  |  |  |
| 2.1 | HIBS Platform Altitude | 20-50 km |  |  |
| 2.2 | Number of cells/HIBS | 7 |  |  |
| 2.3 | Frequency reuse | 1 |  |  |
| 2.5 | HIBS Platform Antenna pattern | Recommendation ITU-R M.2101 |  |  |
|  | Element gain | 8 dBi |  |  |
|  | Horizontal/vertical 3 dB beamwidth of single element | 65º for both H/V |  |  |
|  | Horizontal/vertical front-to-back ratio | 30 dB for both H/V |  |  |
|  | Antenna polarization | Linear/±45 degrees |  |  |
|  | Antenna array configuration (Row × Column) | 2 x 2 elements (1st layer cell), 4 x 2 elements per cell (2nd layer cell) |  |  |
|  | Horizontal/Vertical radiating element spacing | 0.5 of wavelength for both H/V |  |  |
|  | Ohmic losses | 2 dB |  |  |
| 2.6 | HIBS Platform Antenna tilt | 90º (1st layer cell), 33º (2nd layer cell) | 90º (1st layer cell), 23º (2nd layer cell) | 90º (1st layer cell), 23º (2nd layer cell) |
| 2.7 | HIBS Conducted power per antenna element | 37 dBm (1st layer cell), 34 dBm (2nd layer cell) |  |  |
| 2.8 | HIBS Platform e.i.r.p./cell | 55 dBm (1st layer cell), 58 dBm (2nd layer cell) |  |  |
| 2.9 | HIBS Platform e.i.r.p. Spectral Density/cell | 42 dBm/MHz (1st layer cell), 45 dBm/MHz (2nd layer cell) |  |  |
| 3 | UE characteristics |  |  |  |
| 3.1 | UE density for equipment that are transmitting simultaneously | 3 UEs per cell |  |  |
| 3.2 | UE height | 1.5 m |  |  |
| 3.3 | Body loss | 4 dB |  |  |
| 3.4 | Typical antenna gain for UE | -3 dBi |  |  |
| 3.5 | Transmit power control model | Refer to Recommendation ITU-R M.2101 |  |  |
| 3.6 | Maximum UE transmitter output power | 23 dBm |  |  |

This table shows that the 1.7-2.1 GHz band uses the same parameters, while the 2.6 GHz band uses the same parameters with the only difference being the area radius. In other words, these are only differences in frequency, and the difference in propagation loss between the 2 GHz band and the other bands is about ±2 dB when LOS environment/free space loss is taken into account.

For 694-960 MHz, the only difference is that the antenna tilt of the 2nd layer cell. Although the propagation loss is greatly reduced compared to the 2 GHz band, the received power near the cell edge is less than or equal to that at 2 GHz due to the deeper antenna tilt. Therefore, comparing the received power of a 2.1 GH 2nd layer cell, the average is +4.3dB at 750 MHz and +2.7dB at 900 MHz. In reality, the conditions on the TN also change as the frequency changes, so a simple comparison from the ACIR perspective is not possible. However, even if we reflect these differences as a worst-case scenario by making the required ACLR and ACS stricter, they are well within the margins described in the above.

Observation 2: The difference by each frequency is sufficiently lower than the margin when specifying the required ACLR and ACS at 2 GHz.

For HAPS TDD operation, although CLI may occur between HAPS BS-TN BS and HAPS UE-TN UE due to propagation delays, an operational measure to avoid CLI can be considered. Considering transparent payloads, the maximum distance between UE- HAPS-gNB can be assumed about 150 km (RTT 1 ms), since the fixed link of HAPS-gNB as a feeder link can be considered in the range of 20-50 km that described in Report ITU-R F.2439 [4].

In this case, as an example, by synchronizing HAPS transmission with TN and using all guard symbols in a special slot at SCS 15 kHz, together with Timing Advanced, it is possible to avoid the occurrence of CLI as shown in the figure below.

If the distance between the UE-HAPS-gNB is less than 75 km, it is possible to avoid CLI by the same operation at SCS 30 kHz, e.g., replacing all special slots with guard symbol in FR1.30-4. (FR1.30-4 has TDD Slot Configuration pattern：DDDSUUDDDD which is used in Japanese NR TDD config and Special Slot Configuration：6D+4G+4U).

Observation 3: When HAPS uses TDD bands, it is possible to avoid CLI between TN and HAPS by operational measures such as using all special slots as guard symbols.

Based on the views above, we propose that the HAPS bands specified in this WI can be added without changing any other specifications.

Proposal: Bands n2, n3, n5, n7, n8, n20, n25, n26, n28, n34, n38, n39, n41, n67, n85 and n90 can be additionally specified for the use of HAPS in TS 38.104 without any other modifications.

- Conclusion

This contribution proposes our views and proposal for additional NR bands for HAPS use. The summary is as follows:

Observation 1: ACLR and ACS requirements for HAPS BS in the 2 GHz band were specified with a large margin compared to coexistence study results.

Observation 2: The difference by each frequency is sufficiently lower than the margin when specifying the required ACLR and ACS at 2 GHz.

Observation 3: When HAPS uses TDD bands, it is possible to avoid CLI between TN and HAPS by operational measures such as using all special slots as guard symbols.

Proposal: Bands n2, n3, n5, n7, n8, n20, n25, n26, n28, n34, n38, n39, n41, n67, n85 and n90 can be additionally specified for the use of HAPS in TS 38.104 without any other modifications.

- Reference

[1] RP-242373, New WID Proposal: Introduction of HAPS additional operating bands, SoftBank Corp., Deutsche Telekom, NTT DOCOMO, KDDI, RAN#112, September 2024

[2] TR 38.863, Solutions for NR to support non-terrestrial networks (NTN): Non-terrestrial networks (NTN) related RF and co-existence aspects

[3] Annex 4.10 in Doc. 5D/1668, [Working document towards a] Preliminary draft new Report ITU-R M.[HIBS-CHARACTERISTICS] - Technical and operational characteristics for the use of high-altitude platform stations as IMT base stations (HIBS) in the mobile service in certain frequency bands below 2.7 GHz already identified for IMT

[4] Report ITU-R F.2439-0, Deployment and technical characteristics of broadband high altitude platform stations in the fixed service in the frequency bands 6 440-6 520 MHz, 21.4-22.0 GHz, 24.25-27.5 GHz, 27.9-28.2 GHz, 31.0-31.3 GHz,38.0 39.5 GHz, 47.2-47.5 GHz and 47.9-48.2 GHz used in sharing and compatibility studies