3GPP TSG-RAN WG4 Meeting # 112-bis	R4- 2415882

# Hefei, Anhui, China, 14th – 18th Octorber, 2024

Source:	LG Electronics

Title:	Discussion on RRM requirements for regerative payload NTN

Agenda item:	6.26.4.2

Document for:	Discussion

Introduction

In this contritubtion, we provide our views on RRM requirements for downlink coverage enhancement and regenerative payload based on the approved WF [1].

Discussion

Downlink coverage enhancements

RAN1 is still discussing and evaluating SSB periodicity extention, and existing periodicity value also considered. Therefore, RAN4 should wait and discuss after RAN1 has concrete conclusion of extending SSB periodicity.

- Proposal 1: RAN4 should wait and discuss after RAN1 has conclusion of extending SSB periodicity

Regenerative payload

In the last RAN4 meeting, RAN4 agreed that exsitng requirements for NR NTN can be a baseline and it can be revisited based on agreements from other groups.

Timing requirements

<Way Forward in the last RAN4 meeting> - The common understanding is that NTA,adjcommon is configured by network. - Further discuss the impact on timing requirements in TS 38.133 for regenerative mode of NTN.

For timing requirements, even if regenerative payload is considered, there is no impact on timing requirements from the RAN4 perspective since the reference point is on the satellite.

<RAN2#126 Agreements> - RAN2 assumes that the typical setting for common TA and Kmac would be zero in case of regenerative mode with full gNB on board. FFS whether we capture in the spec that common TA and Kmac shall/should be set to zero. <RAN2#127 Agreements> - Send an LS to RAN1 and RAN4 asking whether in a regenerative payload scenario it would be a problem to stick to 0 as the minimum possible value for ta-Common or whether we should e.g. introduce negative values for ta-Common. Also indicate that in any case legacy UEs would have to rely on existing signalling and then on a minimum value equal to 0.

Based on the RAN2 agreements above, RAN2 is discussing the NTA,adjcommon value for regenerative payload, but since NTA,adjcommon is configured by network, RAN4 does not need to revise or remove the NTA,adjcommon in the specification.

- Proposal 2: Reuse existing NR NTN timing requirements for regenerative payload, i.e., keep NTA,adjcommon as it is.

RRM requirements for RRC Connection Mobility Control

<Way Forward in the last RAN4 meeting> : Further discuss the following proposal - Whether to update the specific RRM requirements for RRC re-establishment with regenerative payload should wait for RAN2/RAN3 progress. - Whether to update the specific RRM requirements for satellite switch and RACH-less handover with regenerative payload should wait for RAN2 progress. - FFS: RAN4 to clarify requirement of satellite switching without PCI change is not applied for regenerative mode of NTN.

For RRC connection mobility control, similar issues were discussed in RAN2, and RAN2 had some agreements for regenerative payload NTN.

<RAN2#126 Agreements> - Companies interested to support satellite switch with resync with regenerative payload are invited to bring this discussion to the RAN plenary - RAN2 aims at supporting RACH-less handover for regenerative architecture. Can further discuss whether any optimization is needed - RAN2 assumes that network verified UE positioning is supported for regenerative architecture <RAN2#127 Agreements> - RAN2 confirms the understanding that legacy UEs should be supported in a regenerative payload scenario. - RAN2 understanding is that for regenerative payload existing RACH-less HO could be applied directly, no further Uu enhancement is needed.

Based on the RAN2 agreements above, for RACH-less HO, exising RACH-less HO of NR NRN could be applided directly for regenerative payload NTN. Also, network verified UE positioning could be reused for regenerative payload NTN. Therefore, RAN4 can reuse existing requirements for RACH-less HO and network verified UE positioning.

- Proposal 3: The existing requirements of NR NTN can be reused for RACH-less HO and network verified UE positioning for regenerative payload NTN.

Conclusion

In this contribution, we provide views on downlink coverage enhancement and regenerative payload, and we propose

Downlink coverage enhancements

- Proposal 1: RAN4 should wait and discuss after RAN1 has conclusion of extending SSB periodicity

Regenerative payload

- Proposal 2: Reuse existing NR NTN timing requirements for regenerative payload, i.e., keep NTA,adjcommon as it is.

- Proposal 3: The existing requirements of NR NTN can be reused for RACH-less HO and network verified UE positioning for regenerative payload NTN.

Reference

[1] R4-2414037, “The WF on RRM requirements for Rel-19 NR NTN phase3,” CATT