3GPP TSG-RAN WG4 Meeting #112-bis		R4-2415888

Hefei, China, 14th – 18th October, 2024

Agenda Item:	6.15.2.2

Source: 		Qualcomm Incorporated

Title: 			Enhancement on FR2 CSSF

Document for:	Discussion

# 1.	Introduction

In this paper, we present our view on the following aspects:

- CSSF for outside MG in SA mode

# 2.  Past Agreements

Following agreements happened in the last meeting [1].

### Issue 2-2: UE measurement procedure to use L3 measurement delay reduction by optimizing CSSF Agreement in online session: - The following aspects in CA/DC to use L3 measurement delay reduction by optimizing CSSF shall be discussed, and further prioritization among the 3 aspects can be discussed in future meeting: - Aspect 1: SSB based Intra-frequency measurement without MG, including: - TPSS/SSS_sync_intra and TSSB_measurement_period_intra - CSSFintra for intra-frequency measurement without gap which is defined since Rel-15 - Aspect 2: SSB based Inter-frequency measurement without MG, including: - TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter - CSSFinter for inter-frequency measurement without gap. - Aspect 3: Inter-RAT SSB measurement without MG, including: - CSSFinterRAT for inter-RAT measurement without gap if the UE indicates ‘nogap-noncsg’ via NeedForGapNCSG-InfoEUTRA for the inter-RAT measurement. - MG related features to be considered in aspect 1/2/3 including: - R16 Inter-frequency measurement without gap where SSB is completely contained in active BWP - R17 NCSG measurement with ‘nogap-noncsg’ - R18 NeedForGaps measurement with ‘no-gap-no-interruption’ or with “no-gap-with-interruption” ### Issue 2-3: Applicability requirement of L3 measurement delay reduction by optimizing CSSF Agreement: - RAN4 to consider following CA/DC mode for L3 measurement delay reduction by optimizing CSSFoutside_gap,i - Final applicability of the solutions can be FFS - EN-DC - EN-DC with FR2 only intra band CA - EN-DC with FR2 only inter band CA - EN-DC with FR1 +FR2 CA (FR1 PSCell) - EN-DC with FR1 +FR2 CA (FR2 PSCell) - NE-DC - NE-DC with FR2 only intra band CA - NE-DC with FR2 only inter band CA - NE-DC with FR1 +FR2 CA (FR1 PCell) - SA - FR1+FR2 CA (FR1 PCell) - FR1+FR2 CA (FR2 PCell) - FR2 only intra-band CA - FR2 only inter-band CA - NR-DC - FR1 + FR2 NR-DC (FR1 PCell and FR2 PScell) ### Issue 2-4: Searcher assumption to apply L3 measurement delay reduction by optimizing CSSF [Way forward]: FFS: - Option 1 (CATT): Not to preclude the solutions based on 3 searchers assumption in current stage. - Option 1a (CMCC, HW): it is proposed to consider both the CSSF optimization based on 2 searchers and the CSSF optimization based on 3 searchers. - Option 1b (QC): RAN4 to consider introducing a new UE optional capability regarding the number of cell search/L3-measurement engines for CSSF enhancement, with the details to be finalized (FFS). - Option 2 (Apple, CTC, Nokia, Ericsson): RAN4 only consider the enhancement based on 2 searchers, i.e., same as previous release, for L3 measurement delay reduction by optimizing CSSF. - Option 2a (Apple): If companies cannot achieve consensus, RAN4 work can start with the baseline assumption in WID, i.e., 2 searchers. - Option 2b (Nokia) RAN4 to confirm if one of the searchers is assumed for PCC/PSCC measurement and the other is assumed for the measurements on all SCCs.

# 3. 	Discussion

CSSF for outside MG in SA mode

In Table 1, CSSF for outside MG in SA mode is summarized. Among the scenarios in Table 1, the grey elements do not seem to be relevant to the WI scope. Additionally, those components that are not relevant are crossed out in Table 1, e.g., components related to CSI-RS based L3 measurement.

Table 1. CSSF for outside MG in SA mode

| Scenario | CSSFoutside_gap,i for FR1 PCC | CSSFoutside_gap,i for FR1 SCC | CSSFoutside_gap,i for FR2 PCC | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is required | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is not required | CSSFoutside_gap,i for inter-frequency MO with no measurement gap |
| --- | --- | --- | --- | --- | --- | --- |
| FR1 only CA | 1+NPCC_CSIRS + NPCC_CCA_RSSI/CO | NSCC_SSB + Y + 2xNSCC_CSIRS + NSCC_CCA_RSSI/CO | N/A | N/A | N/A | NSCC_SSB + Y + 2xNSCC_CSIRS |
| FR2 only intra band CA | N/A | N/A | 1 + NPCC_CSIRS | N/A | NSCC_SSB + Y + 2xNSCC_CSIRS | NSCC_SSB + Y + 2xNSCC_CSIRS |
| FR2 only inter band CA | N/A | N/A | 1 + NPCC_CSIRS | 2*(1 + NSCC_CSIRS_FR2_NCM)Note 3,5 | 2×(NSCC_SSB + Y + 2xNSCC_CSIRS - 1 - NSCC_CSIRS_ FR2_NCM) | 2×(NSCC_SSB + Y + 2xNSCC_CSIRS - 1 - NSCC_CSIRS_ FR2_NCM) |
| FR1 +FR2 CA (FR1 PCell)Note 1 | 1 + NPCC_CSIRS | 2×( NSCC_SSB + Y + 2xNSCC_CSIRS - 1 - NSCC_CSIRS_FR2_NCM) | N/A | 2*(1 + NSCC_CSIRS_FR2_NCM)Note 3,5 | 2×(NSCC_SSB + Y + 2xNSCC_CSIRS - 1 - NSCC_CSIRS_ FR2_NCM) | 2×(NSCC_SSB + Y + 2xNSCC_CSIRS - 1 - NSCC_CSIRS_ FR2_NCM) |
| FR1 +FR2 CA (FR2 PCell)Note 1 | N/A | NSCC_SSB + Y + 2xNSCC_CSIRS | 1 + NPCC_CSIRS | N/A | NSCC_SSB + Y + 2xNSCC_CSIRS | NSCC_SSB + Y + 2xNSCC_CSIRS |
| Note 1: Only one FR1 operating band and one FR2 operating band are included for FR1+FR2 inter-band CA. Note 2: Selection of FR2 SCC where neighbour cell measurement is required follows clause 9.2.3.2. <One of the SCCs on which UE is configured to report SSB based measurements when neither PCC nor PSCC is in the same band, so that the selected SCC shall be an SCC where the UE is configured with SS-RSRP measurement reporting if such SCC exists, otherwise the selected SCC is determined by UE implementation. > Note 3: CSSFoutside_gap,i=1 if only one SCell is configured and no inter-frequency MO without gap and only SSB based L3 measurement is configured on SCC; CSSFoutside_gap,i=2 if only one SCell is configured and no inter-frequency MO without gap and either both SSB and CSI-RS based L3 configured or only CSI-RS based L3 measurement is configured on SCC. Note 4: Y is the number of configured inter-frequency MOs without MG that are being measured outside of MG; otherwise, it is 0. Note 5: Only two NR FR2 operating bands are included for FR2 inter-band CA. Note 6: NPCC_CSIRS=1 if PCC is with either both SSB and CSI-RS based L3 configured or only CSI-RS based L3 measurement configured; otherwise, NPCC_CSIRS=0. Note 7: NSCC_CSIRS=Number of configured SCell(s) with either both SSB and CSI-RS based L3 measurement configured or only CSI-RS based L3 measurement configured Note 8: NSCC_CSIRS_FR2_NCM=1 if FR2 SCC, where neighbour cell measurement is required, is with either both SSB and CSI-RS configured or only CSI-RS measurement configured; otherwise, NSCC_CSIRS_FR2_NCM=0. Note 9: NSCC_SSB=Number of configured SCell(s) with only SSB based L3 measurement configured, which is measured without MG. Note 10: NPCC_CCA_RSSI/CO=1 if PSCC is configured with RSSI/CO measurements without MG when RMTC and SMTC are overlapping; NSCC_CCA_RSSI/CO=Number of MOs for SCell(s) configured with RSSI/CO measurements without MG when RMTC and SMTC are overlapping. |  |  |  |  |  |  |

From the CSSF defined in Table 1, the principle of cell search/L3-measurement engine sharing can be summarized as follows. Note that the current CSSF assumes that the UE is equipped with at most two cell search/L3-measurement engines, although there are cases where up to three cell search/L3-measurement engines are required.

- One cell search/L3-measurement engine is dedicated to PCC

- The other one is shared with the rest CCs

  - For DC, the cell search/L3-measurement engine is dedicate to PSCC for 50% of the time, and for the other 50%, the engine is equally shared with the rest of the CCs.

  - For SA, the cell search/L3-measurement engine is dedicate to SCC where neighbor cell measurement is required for 50% of the time, and for the other 50%, the engine is equally shared with the rest of the CCs.

In Table 2, intra-frequency measurements period requirements without MG are summarized. As shown in Table 1, depending on the serving cell’s state, SMTC is replaced with measCycleSCell or measCyclePSCell. Irrespective of the serving cell’s state, the measurement period requirement per frequency layer is scaled by CSSF.

Table 2. Intra-frequency measurements period requirements without MG

| DRX | FR2 Active serving cell or neighbor cell | FR2 Deactivated SCell | FR2 Deactivated PSCell |
| --- | --- | --- | --- |
| No DRX | max(400ms, ceil(Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x SMTC period)Note 1 x CSSFintra | Ceil(Mmeas_period_w/o_gaps x Kp) x measCycleSCell x CSSFintra | Ceil(Mmeas_period_w/o_gaps x Kp) x measCyclePSCell x CSSFintra |
| DRX cycle ≤ 320ms | max(400ms, ceil(1.5 x Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x max(SMTC period, DRX cycle)) x CSSFintra | Ceil(Mmeas_period_w/o_gaps x Kp) x max(measCycleSCell, 1.5x DRX cycle) x CSSFintra | Ceil(Mmeas_period_w/o_gaps x Kp) x max(measCyclePSCell, 1.5x DRX cycle) x CSSFintra |
| DRX cycle>320ms | ceil(Mmeas_period_w/o_gaps x Klayer1_measurement x Kp) x DRX cycle x CSSFintra | Ceil(Mmeas_period_w/o_gaps x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra | Ceil(Mmeas_period_w/o_gaps x Kp) x max(measCyclePSCell, DRX cycle) x CSSFintra |
| Note 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified Definition of symbols - Mmeas_period_w/o_gaps: For PC3, 24 = 3 x 8 - Kp: Scaling factor for an SSB frequency layer to be measured without GAP, e.g. Kp = Ntotal / Navailable - SMTC period: {periodicityAndOffset, duration} where periodicity can be {5, 10, 20, 40, 80, 160}, offset can be any integer from 0 to the periodicity minus 1, and duration can be {1, 2, 3, 4, 5} with unit of subframe for all parameters. - measCycleSCell & measCyclePSCell: {160, 256, 320, 512, 640, 1024, 1280} in ms - Klayer1_measurement: If RSs configured for L1 measurement/evaluation (e.g. RLM, BFD, CBD or L1-RSRP) in the same band outside measurement gap are not fully overlapping with intra-frequency SMTC occasions, 1. Otherwise, 1.5 if L1 and L3 resources fully overlap, measurement opportunities are shared between L1 and L3 in the ratio of 2/3 for L1 and 1/3 for L3. And this sharing is applicable only for FR2 due to a single UE Rx beam constraint. - DRX cycle: For shortDRX = {2, 3,4, 5, …160, 256, 320, 512, 640} in ms. For longDRX = {10, 20, 32, … 160, 256, 320, 512, 640, 1024, … 10240} in ms. |  |  |  |

One possibility of measurement enhancement could be a measurement skipping based on the geographical relationship between cells. Figure 2 depicts one deployment example and shows how the cells can be decomposed in the frequency domain and how the cell can be grouped based on their geographic relationships. In case UE is provided with the geographical relationship, depending on where the UE is located (e.g. which cell the UE can detect and measure), it may skip L3 measurements on some of the frequency layers. Besides, depending of the status of the serving cells (e.g. SCells being activated or deactivated), the L3 measurement can be enhanced under the assumption that L3 measurement for the active SCell can be replaced with L1 measurement if the SCell is the only cell needing L3 measurement based on the geographical relationship based L3 measurement pruning. And the CSSF comparison is provided in Table 3. However, this enhancement scheme requires additional signaling from NW regarding their cell deployment, which is typically not preferred. Additionally, one might be skeptical about the underlying assumptions of the scheme. Therefore, we do not see much chance of this scheme being considered in RAN4.

Figure 1. Grouping of cells based on their geographical relationships

Table 3. CSSF where L3 measurement for the active SCell can be replaced with L1 measurement if the SCell is the only cell needing L3 measurement based on the geographical relationship based L3 measurement pruning

|  | CSSF |  |  |  |
| --- | --- | --- | --- | --- |
|  | Legacy | Enhancement in case L3 measurement for the active SCell can be replaced with L1 measurement if the SCell is the only cell needing L3 measurement based on the geographical relationship based L3 measurement pruning |  |  |
|  |  | No neighbor cell to measure on {1,0} and {2,0} |  | Otherwise |
| f3 | 2 | - Deactivated: 2 - Deactivated: 1 - Deactivated: 1 - Activated: 1 - Activated: 2 |  | - Deactivated: 2 - Deactivated: 2 - Deactivated: 2 - Activated: 2 - Activated: 2 |
| f2 | 0 | - Deactivated: 0 - Deactivated: 1 - Activated: 0 (L1-only) - Activated: 0 (L1-only) - Deactivated: 0 |  | - Deactivated: 0 - Deactivated: 0 - Activated: 0 (L1-only) - Activated: 0 (L1-only) - Deactivated: 0 |
| f1 | 2 (selected FR2 SCC based on clause 9.2.3.2) | - Deactivated: 2 - Activated: 0 (L1-only) - Activated: 0 (L1-only) - Activated: 0 (L1-only) - Deactivated: 2 |  | - Deactivated: 2 - Activated: 2 for {2,1} - Activated: 2 for {2,1} - Activated: 2 for {2,1} - Deactivated: 2 |
| f0 | 1 | 1 | 1 |  |

Based on the observations and analyses provided above, we couldn’t find a practical way of enhancing CSSF other than considering an optional UE capability regarding the number of cell search/L3-measurement engines which can be up to, e.g. 4.

Proposal 1: RAN4 to consider introducing a new UE optional capability regarding the number of cell search/L3-measurement engines for CSSF enhancement, with the details to be finalized (FFS).

# 4.	Conclusion

Proposal 1: RAN4 to consider introducing a new UE optional capability regarding the number of cell search/L3-measurement engines for CSSF enhancement, with the details to be finalized (FFS).

# 5.  Reference

[1] R4-2414026, Way Forward for NR_RRM_Ph5, Apple, RAN4 #112