3GPP TSG-RAN4 Meeting #112-bis	R4-2415889

# Hefei China, 14th – 18th October, 2024

Source: 	Qualcomm Incorporated

Title: 	CSI Compression: Testability and Interoperability

Agenda item:	6.17.2.1

Document for:	Discussion

# Introduction

In this paper, we are presenting updated simulation results of the CNN based encoder-decoder pair that got agreed during the last two meeting. We are also providing our views regarding the reference encoder-decoder pair.

# Discussion

## Prior Agreements

Following agreements occurred during the email discussion after RAN4 111 [1].

Based on the two rounds of discussions, the following parameters are proposed: Fig 1. Detailed model diagrams of encoder. Fig 2. Detailed model diagrams of decoder. Table 1. CNN model parameters for encoder. Model architecture parameters for encoder (convolutional); Model type; CNN (Resnet like design). FLOPs; 47M (vs. 1.6M for eTypeII). Model size; 448Kbyte. Num of Resnet blocks; 6. Layer after resnet blocks; 2 layers (one FC layer, one Conv layer). Layer before resnet blocks; 1 layer (one conv layer). Layer within a resnet block; 2 Conv layer and direct combination. CNN kernel parameters; - dilation =1 (which is the default parameter) - padding type = zero padding (which is the default parameter) - The value of groups =1 (which is default parameter). Kernel/Channel size within a resnet block; (3,3,32). Size of message (feature map) passed across layers; (13,32). Table 2. CNN model parameters for decoder. Model architecture parameters for decoder (convolutional); Model type; CNN (Resnet like design). FLOPs; 740M. Model size; 7.2Mbyte. Num of Resnet blocks; 6. Layer before resnet blocks; 2 layers (one FC layer, one Conv layer). Layer after resnet blocks; 1 layer (one conv layer). Layer within a resnet block; 2 Conv layer and direct combination. CNN kernel parameters; - dilation =1 (which is the default parameter) - padding type = zero padding (which is the default parameter) - The value of groups =1 (which is default parameter). Kernel/Channel size within a resnet block; (3,3,128). Size of message (feature map) passed across layers; (13,32). Other model related parameters: Activation function; ReLU. Latent message size; 32 float values. Payload size, bits; 64 per layer.. Quantization; 2 bits. System parameters: Frequency range; 2GHz (4GHz optional). MIMO Layers; 1 layer (2 layers optional, companies to. Sub-bands; 13. Channel estimation; Ideal. Metrics; SGCS of AI CSI feedback and eTypeII CSI feedback, both relative to ideal CSI.. Companies are also invited to provide the training parameters used.

Following agreements happened during the last meeting [2].

Parameter/Configurations; Configuration for alignment. Channel/data-set related. UE antenna element assumptions; Ideal omni-directional antenna elements following RAN1 assumptions. Sub-band channel averaging; - Average of covariance channel matrices per PRBs in the sub-band - 1 sample per PRB is selected. Any other pre-processing; - Apply phase normalization: - Normalize the phase of all entries of the eigenvector based on the phase of the 1st entry of the eigenvector - i.e. the phase of the first value of the eigenvector is 0. - L2 norm of the eigenvector is equal to 1.. Data collection assumptions; - Strive to ensure that the samples are decorrelated, especially when collected from the same UEs.. Training dataset size; - Aim to use the dataset around 600K. - At least 100K sample to be used.. Dataset quantization; - Float32. Other data-set characteristics/statistics; - Companies to propose. - Companies are encouraged to report some statistics about the dataset. - Statistics are FFS - Companies are encouraged to report the CDF of the SGCS across sub-bands and input samples. - If possible, also report CDF for eTypeII.. Model-related. Quantization of encoder output; - Report results without quantization of the FC layer outputs for validation purposes only, i.e., float32 is sent in CSI report - Companies still need to report quantization results: - Companies to report whether/how quantization aware training is implemented (e.g., how quantization is treated at back propagation) - Companies can try quantization aware training with bypassing of the quantization in back propagation - Quantization options, companies to provide the details: - Fixed unform quantization with equal ranges - Trainable quantization. Normalization before quantization in the encoder; - For the results with quantization, to use a sigmoid function in the encoder for normalization - Sigmoid in the decoder is optional.. Use and configuration of drop-out; - Not to use drop-out layers. Training loss function, use of regularization; - Use (1-SGCS) as loss function as reference. Optimizer; - Use Adam. Learning rate. - Fixed learning rate of 10^-4. Batch size. - 256. (Max) Number of epochs. - 150 epochs. Stopping criteria. - No improvement over 25 epochs.. Weight initialization. - Default. Kernel Parameters: - Dilation - Padding Type The value of groups (VG); - Default.

## Result

Figure 1 and 2 show CDF of CNN based SGCS without quantization and with quantization respectively. We aligned our parameters based on [2] and generated these results based on the assumptions of [1]. Trainable quantization was used to generate the SGCS CDF of Figure 2. The dataset used to generate SGCS of Figure 1 and 2 is different from the one that was used in our contribution for the last RAN4 meeting [2].

Figure 1: CDF of CNN based SGCS across samples and subbands (no quantization)

Figure 2: CDF of CNN based SGCS across samples and subbands (with trainable quantization)

Observation 1: The mean SGCS of CNN based model and eTypeII based on latest simulation assumptions are shown below:

| Method | Mean SGCS (across samples and subbands) | Median SGCS (across samples and subbands) |
| --- | --- | --- |
| CNN encoder – CNN decoder without quantization | 0.74 | 0.8 |
| CNN encoder – CNN decoder with quantization | 0.683 | 0.73 |
| eTypeII | 0.67 | -- |

## Reference Encoder Decoder Pair

RAN4 made the following agreement regarding reference encoder decoder pair during the last meeting [4]

Issue 4-4: Options for Option 4 Option 4 sub-options: - 4a: Dataset based: specify at least dataset based on which the test decoder can be implemented by TE vendors FFS whether anything else needs to be specified (e.g. model structure for decoder or a reference encoder) - 4b: Reference encoder based: encoder is documented in the specifications, used to derive a decoder to be implemented by TE vendors FFS whether training dataset (channel information) needs to be specified - 4c: do not specify either reference encoder or dataset. specify enough constraints on the decoder output ordering Companies are invited to bring further analysis on how to perform a feasibility study for these options.

In option 4b, UE vendor needs to know the reference decoder to train its encoder. Hence, for option 4b to work, reference decoder needs to be specified along with the reference encoder.

Observation 2: In option 4b, UE vendor needs to know the reference decoder to train its encoder. Hence, for option 4b to work, reference decoder needs to be specified along with the reference encoder.

Proposal 1: Option 4b is suggested to be modified in the following way (modifications are shown in blue):

- 4b: Reference encoder and reference decoder based: encoder is documented in the specifications, used to derive a decoder to be implemented by TE vendors. Reference decoder is documented in the specifications, used by UE to train its encoder.

  - FFS whether training dataset (channel information) needs to be specified

# Conclusions

Observation 1: The mean SGCS of CNN based model and eTypeII based on latest simulation assumptions are shown below:

| Method | Mean SGCS (across samples and subbands) | Median SGCS (across samples and subbands) |
| --- | --- | --- |
| CNN encoder – CNN decoder without quantization | 0.74 | 0.8 |
| CNN encoder – CNN decoder with quantization | 0.683 | 0.73 |
| eTypeII | 0.67 | -- |

Observation 2: In option 4b, UE vendor needs to know the reference decoder to train its encoder. Hence, for option 4b to work, reference decoder needs to be specified along with the reference encoder.

Proposal 1: Option 4b is suggested to be modified in the following way (modifications are shown in blue):

- 4b: Reference encoder and reference decoder based: encoder is documented in the specifications, used to derive a decoder to be implemented by TE vendors. Reference decoder is documented in the specifications, used by UE to train its encoder.

  - FFS whether training dataset (channel information) needs to be specified

# References

[1] R4-[111][133] NR_AIML_air-email discussions after RAN4#111 meeting, https://list.etsi.org/scripts/wa.exe?A2=3GPP_TSG_RAN_WG4;aa5369d9.2406C&S=

[2] R4-2414447, “Alignment of model training and data set parameters for AI/ML based CSI compression”, Nokia,  3GPP TSG-RAN4 Meeting #112.

[3] R4-2413169, “CSI Compression: Testability and Interoperability”, Qualcomm Incorporated, 3GPP TSG-RAN4 Meeting #112.

[4] R4-2414323, “WF on AI/ML”, Qualcomm Incorporated, 3GPP TSG-RAN4 Meeting #112.