# 3GPP TSG-RAN WG4 Meeting # 112-bis	R4-2415898

# Hefei, China, Oct.14th – 18th, 2024

# Title: 	Further discussion on PC3 n28 40MHz

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	5.20.1

# Document for:	Approval

# 1 Introduction

In RAN #104 meeting, a new WID on Power Class 2 and UE 40MHz Channel Bandwidth in NR band n28 was approved in [1], where the objectives are listed as below:

The objectives of this WI are to introduce requirements for UE Power Class 2 and 40MHz Channel Bandwidth for NR band n28 as optional features, including the following aspects: - Introduce requirements for PC2 and those associated with following bullets - Both 1Tx and 2Tx PC2 are supported - UL-MIMO is supported. - Specify A-MPR requirements for PC2 - Introduce 40MHz Channel bandwidth located in UL: 703 - 743MHz and DL: 758 - 798MHz for band n28 supporting PC3 and PC2. - Allow flexible channel allocations within the frequency range (UL: 703 - 743MHz and DL: 758 - 798MHz) for BW<40MHz. - Release independence of UE 40MHz Channel bandwidth to Rel-15 is anticipated. - No changes to the existing requirements (38.101-1 v18.5.0) (including A-MPR values and RB allocation regions) for power class 3 with existing channel bandwidths up to and including 30MHz to avoid impacts on existing power class 3 networks. In the event of new regulatory requirements, this assumption can be revisited.

In last meeting, some agreements are captured in the WF[2], in which there are several open issues.

In this contribution, we give some further discussions on the RF requirements of 40MHz for n28 PC2.

# 2	Discussion

In terms of the description of the WID, evolve duplexer implementation is desired from industry, i.e. single band duplexer implementation, would be used for studied. This is different with the existing specs in which dual duplexer architecture for UE is assumed. Also, it should be noted the existing requirements for power class 3 with existing channel bandwidths up to and including 30MHz are not changed.

1.	UE channel bandwidth in band 28

Issue 2-3-9 channel raster Agreement: - The following options can be considered for n28 40MHz - Add the exceptional channel raster point of n28 (UL: 723.04MHz, DL: 778.04MHz) to UE RF specification TS 38.101-1 for UE CBW 40MHz. - UE supporting 40 MHz channel bandwidth in band n28 shall support Enhanced channel raster.

In the TS38.104 specification, the exception channel raster points are defined for band n28, whichi is:

| NR operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n28 | 100 | 140600 – <20> – 149600 | 151600 – <20> – 160600 |
|  |  | 1446084 | 1556084 |
| NOTE 4: This exceptional raster point is applicable only to n28 and is only applicable for 40MHz BS channel bandwidth to ensure the guardband with 30MHz UE channel bandwidth. |  |  |  |

The exception channel raster points mainly resolve the issues of UE 30MHz reconfiguration fails when UE access 40MHz network of n28 after UE’s self-check of the minimum guard band RF requirements due to the minimum guard band of 30MHz is larger than 40MHz. In the end, some exceptions for n28 40MHz UE CBW are allowed, i.e. allow 40MHz UE CBW not aligned with 100kHz channel raster if future UE support 40MHz CBW.

It should be noted that the exception channel raster points were not introduced in the TS38.101-1 specification as of today. Also, it should be noted that the above exception channel raster points are 40kHz offset from the closest 100kHz channel raster grid. Also it should be noted that the exception channel raster points for band n28 were agreed before the agreements for enhanced channel raster.

Now, both 30MHz and 40MHz are supported in UE, which means the UE specific bandwiths can be RRC reconfigred to either 30MHz or 40MHz in the case of the exception channel raster points for gNB 40MHz. The exception channel raster points in gNB would cause the misalignment raster grid between 40MHz gNB and 40MHz UE when 100kHz channel raster is used in UE. It is our understanding why company proposed 40 MHz channel bandwidth for both BS and UE is only at the raster entry, 723.04 MHz for UL and DL 778.04 MHz for DL.

However, we think this issue can be solved thanks to the 10kHz enhanced channel raster was introduced and mandatory supported for band n28 regardless of channel bandwidth since the existing exception channel raster points are located in 10kHz grid. Therefore, we think there is no need to include both exceptional channel raster point and enhanced channel raster in TS38.101-1.

Observation 1. For n28 30MHz, there are no exception channel raster points defined in TS38.101-1.

Observation 2. For n28 40MHz, UE can supported the gNB exception channel raster points with the enhanced channel raster.

Proposal 1: There is no need to include both exceptional channel raster point and enhanced channel raster in TS38.101-1.

Issue 2-3-8 channel location Agreement: - FFS on how to capture the UL channel location for UE supporting maximum bandwidth of 40MHz in n28.

In current specification for band n28, NOTE 7 is applied to the supported channel bandwidths of 20MHz, 25MHz and 30MHz under the dual band duplexer architecture assumption. For 40MHz, single band duplexer architecture is assumed, and the minimum requirements are specified for NR UL transmission bandwidth configuration confined to either 703-743 MHz. In [4], It is proposed to allow the flexible channel allocation for <40MHz. In our understanding, it would be fine to do this since single band duplexer architecture is used, rather than dual band duplexer architecture.

Proposal 2: For the UL 40MHz bandwidth channel location:

- The minimum requirements are specified for NR UL channel bandwidth configuration confined to 703-743 MHz.

- Flexible channel allocation for <40MHz channel bandwidth should be allowed.

2.	∆MPR&A-MPR

Issue 2-3-4 ∆MPR for 40MHz Agreement: - Further check if ΔMPR= 0.5dB is sufficient

The relative channel bandwidth of n28 30MHz and 40MHz are around 4.1% and 5.5%, respective. Both of them are larger than the 3% criteria of FDD bands. So same with 30MHz, it’s necessary to specify the ∆MPR requirements for n28 40MHz.

According to TR38.888, it mentioned that general MPR requirements in the current spec can be met for UL 30MHz channel bandwidth in band n28. For 40MHz, We did some evaluations by using the existing assumptions for PC3 n28 40MHz MPR. Here we select 30kHz SCS for the evaluation due to the time limits, including DFT-s-OFDM and CP-OFDM. Due the the spectrum decay for the 30MHz is slower than 15kHz SCS, so the results of 30kHz SCS can also be applied to 15kHz SCS. The results can be found in Fig 1~Fig 6.

Fig 1.DFT-s-OFDM, QPSK									Fig 2.DFT-s-OFDM, 16QAM

Fig 3.DFT-s-OFDM, 64QAM                           Fig 4.CP-OFDM, QPSK

Fig 5. CP-OFDM, 16QAM                           Fig 4.CP-OFDM, 64QAM

As we can seen from the above figures, the current general MPR specified in existing standards can be met. Here we didn’t take the practical implementations margins, such as more good I/Q imbalance/carrier leakage values, into account, which can make the MPR value smaller.

Observation: The general MPR requirements in the current spec can be met for UL 40MHz channel bandwidth in band n28.

Similar to 30MHz, ∆MPR is still defined considering different implementations and accommodate the more pronounced PA memory effect. We think ΔMPR= 0.5dB is sufficient for n28 40MHz.

The above results for n28 40MHz are aligned with the tendency of the other similarity bands like band n71 given that band n71 falls within a comparable frequency band to band n28. For n71, the introduction of some additional channel bandwidths, namely 25 MHz, 30 MHz, and 35 MHz, did not significantly impact the emission requirements, which is the ΔMPR values are the same for the channel bandwidths of 25MHz, 30MHz and 35MHz.

Therefore, we propose:

Proposal 3: ΔMPR= 0.5dB for PC3 n28 40MHz.

# 3 Conclusion

In this contribution, we give some further discussion on the RF requirements of 40MHz for n28 PC3. The conclusions are summarized in the following:

Observation 1. For n28 30MHz, there are no exception channel raster points defined in TS38.101-1.

Observation 2. For n28 40MHz, UE can supported the gNB exception channel raster points with the enhanced channel raster.

Observation: The general MPR requirements in the current spec can be met for UL 40MHz channel bandwidth in band n28.

Proposal 1: There is no need to include both exceptional channel raster point and enhanced channel raster in TS38.101-1.

Proposal 2: For the UL 40MHz bandwidth channel location:

- The minimum requirements are specified for NR UL channel bandwidth configuration confined to 703-743 MHz.

- Flexible channel allocation for <40MHz channel bandwidth should be allowed.

Proposal 3: ΔMPR= 0.5dB for PC3 n28 40MHz.

# 4	Reference

[1] RP-241660, New WID: Introduction of Power Class 2 and UE 40MHz Channel Bandwidth in NR band n28, CMCC, CBN

[2] R4-2414274, WF on introduction of PC2 and 40MHz CBW in NR band n28, CMCC, CBN, China Broadnet, Approved

[3] R4-2411878, Discussion on PC3 n28 40MHz, ZTE Corporation, Sanechips.

[4] R4-2411743, UE RF requirements for introducing 40MHz, CMCC