# 3GPP TSG-RAN WG4 Meeting # 112-bis                              R4-2415902

# Hefei, China, Oct.14th – 18th, 2024

# Title: 	Further discussion on 6Rx SRS antenna IL imbalance

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	6.1.1.3.4

# Document for:	Approval

# 1 Introduction

In last meeting, some agreements are included in the approved WF[2], in which the REFSEN requirements were approved after long debate, as shown below:

# Topic 4: SRS IL imbalance issue ## Sub-topic 4-1: General considerations for SRS IL imbalance issue Issue 4-1-1: Whether to solve SRS IL imbalance issue in Rel-19 Way Forward: RAN4 to further discuss the following options. - Option 1: RAN4 should not continue the discussion on how to solve the SRS IL imbalance issue. - Option 2: Continue to pursue a solution to the SRS IL imbalance issue dependent on the outcome of Issue 4-1-2 in the approved WF in R4-2410751. Issue 4-1-2: Initial Considerations for SRS IL imbalance issue Way forward: Given the different views amongst companies, RAN4 to further discuss the set of initial considerations which will allow companies to have a common understanding for the study including existing UE behavior for SRS transmissions in case of SRS IL. - Companies are encouraged to bring analysis on the existing UE behavior and achievable power imbalance for SRS transmissions based on current specification and UE implementations in case of SRS IL. - Companies are encouraged to analyse the impact of SRS IL imbalance on NW performance degradation. ## Sub-topic 4-2: SRS IL imbalance issue solutions Issue 4-2-1: Candidate solutions for the SRS IL imbalance issue Way forward: Companies encouraged to provide compromised solutions for consideration with minimal impact to the specification and to indicate the specific impacts to RAN1, RAN2, and RAN4 specifications and performance gain.

It seems there were no concreted progress according to the above WF in comparison of the last WF. So we resubmit the contributions [2] for further discussion.

# 2	Discussion

The impacts on DL performance caused by SRS IL imbalance have been stuided in RAN1 during R18, although there were no consensus reached, majority companies have shown the non-negligible DL performance degradation in case SRS IL is not compensated at the UE and/or gNB sides in terms of the texts of ‘“It can be observed from most companies’ result that the existence of SRS IL imbalance will cause non-negligible performance degradation, especially under MU-MIMO and/or high-rank scenario.”. in the summary [3], also similar observations from some contributions in RAN4 such as [4].

Observation. Majority companies shows the SRS IL imbalance would cause non-negligible performance degradation according to the past discussions.

In our understanding, current NW has no knowlaedge about the UE SRS IL imbalance, instead the NW can dynamically adjust the power control parameters based on network conditions and UE performance, this helps in managing the power imbalance and optimizing the overall system performance. The UE adjusts the transmission power of SRS based on the power control parameters provided by the base station.

Generally speaking, the SRS transmission power should be sufficient to overcome the path loss and channel impairments. Too weak SRS transmission power would lead to poor measurement quality, but excessive power can lead to interference with other UEs or even affect network performance. Hence, in our understanding, the UE should adjusts its power dynamically to manage imbalance and maintain proper signal balance.

1n current RAN1 TS38.213 spec, the SRS transmission power are depends on the configured maximum output power PCMAX,f,c limit, and some paramers such as p0 , α, IL and etc.

For the term of Pcmax,f,c, it is defined in the TS38.101-1, wherein the ∆TRxSRS parameter in incoporated in the lower boundary. This parameter actually was defined under the worse case where SRS power is transmitted from the antenna other than main antenna who usually has longest RF path and includes more switchers. With more Rx branches introduced, the ∆TRxSRS requirements become larger and more serious.

We believe UE has ability to do some power compensation to overcome the larger IL imbalance among all the Tx/Rx SRS antenna port, which depends on the UE implementation. This compensation is aimed at maximizing the transmission of SRS power through antennas other than the primary Tx antenna, ensuring that as much power as possible is utilized efficiently.

For example, when the SRS transmission power is below the UE configured maximum output power (i.e. PCMAX,L), then UE can do the power compensation to compensate the IL and also there exist +2dB power tolerance for the Pcmax. Even can re-calculate the PCMAX,L with the new ΔIL to replace ∆TRxSRS to achieve higher boundary of PCMAX,L. But of course, if the UE has already achieve the PCMAX,H, and there is no power headroom, then UE is not able to perform the IL compensation. Once the UE performs this power compensation, the ∆TRxSRS between antennas will be significantly reduced, leading to a more balanced and optimized signal distribution. It is advantageous to inform the Network (NW) of this compensation information as it facilitates more precise CSI estimation.

Another way would be dynamic report although it is admittedly more complicated than the UE power compensation. It offers a wealth of real-time information that is invaluable in certain scenarios. This is particularly true when the RF routing for SRS changes over time, yet the RFFE/RF routing information remains unknown to the NW. UE can report actual SRS IL in dynamic reporting, providing crucial insights into the real-time performance of the system. Furthermore, the establishment of several thresholds for these actual SRS IL may be necessary to ensure that the NW can effectively monitor and manage the system's performance. Even the consideration of different capability classes based on the reported actual SRS IL could be explored taking into account the diverse capabilities and operational characteristics of individual UEs, which is to use the similar methodology of lower-MSD capability reporting.

Moreover, Due to ∆TRxSRS requirements are defined for 2Rx, 4Rx, 6Rx and 8Rx, we think the problem would be similar for all of them although the problem is minor for 2Rx. Thus if ∆TRxSRS indication from UE to NW is optionally introduced for 6Rx, it can also apply to 2Rx/4Rx/8Rx case.

With the above consideration, we have the following proposals:

Proposal 1: Introduce reporting for the SRS IL imbalance issue in Rel-19.

Proposal 2. UE power compensation is up to UE implementation, and UE needs to indicate the power compensation behaviour to NW.

Proposal 3. Dynamic reporting for actual SRS IL reporting for each SRS-TxSwitch pattern, and several thresholds associated with capability class for the actual SRS IL reporting can be considered.

Proposal 4. The SRS IL imbalance reporting mechanism should be also specified for 2Rx, 4Rx and 8Rx.

# 3 Conclusion

In this contribution, we give some further discussions on the SRS IL imbalance related to 6Rx. The conclusions are:

Observation. Majority companies shows the SRS IL imbalance would cause non-negligible performance degradation according to the past discussions.

Proposal 1: Introduce reporting for the SRS IL imbalance issue in Rel-19.

Proposal 2. UE power compensation is up to UE implementation, and UE needs to indicate the power compensation behaviour to NW.

Proposal 3. Dynamic reporting for actual SRS IL reporting for each SRS-TxSwitch pattern, and several thresholds associated with capability class for the actual SRS IL reporting can be considered.

Proposal 4. The SRS IL imbalance reporting mechanism should be also specified for 2Rx, 4Rx and 8Rx.

# 4	Reference

[1] R4-2414285, WF on requirements for 6Rx, AT&T

[2] R4-2411885, Further discussion on 6Rx SRS antenna IL, ZTE, Sanechips

[3] R1-2308458 FL summary on UE SRS IL imbalance issue Moderator (Huawei)

- [4] R4-2409759, On need for SRS insertion loss imbalance reporting, Ericsson

- [5] R4-2408614, Views on SRS insertion loss compensation and reporting enhancements, Intel