# 3GPP TSG-RAN WG4 Meeting # 112-bis	R4-2415903

# Hefei, China, Oct.14th – 18th, 2024

# Title: 	Further discussion on 6Rx SRS antenna switching requirements

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	6.1.1.3.3

# Document for:	Approval

# 1 Introduction

In last meeting, some agreements are included in the approved WF[2], in which the SRS antenna switching and ∆TRxSRS requirements were shown below:

# Topic 2: SRS antenna switching and ΔTRxSRS ## Sub-topic 2-1: General considerations for SRS antenna switching and ΔTRxSRS Issue 2-1-1: Whether to consider an additional breakpoint for bands whose FUL_high is higher than the FUL_low of n104 Way forward: Further discuss the following options. Option 1: Do not consider an additional breakpoint for bands whose FUL_high is higher than the FUL_low of n104 Option 2: Consider additional breakpoint for bands whose FUL_high is higher than the FUL_low of n104 Issue 2-1-2: Whether to consider separate ∆TRxSRS values when the device is power class 2 in the band and ΔPPowerClass = 0dB and not indicating Tx diversity capability Agreement: Have additional 3dB for power class 2 when ΔPPowerClass = 0dB and not indicating Tx diversity capability. ## Sub-topic 2-2: ΔTRxSRS values Issue 2-2-1: Proposed ∆TRxSRS values Way Forward: RAN4 agrees to adopt the average values for Bands n41, n77, and n78. For Band n79 and n104, further analysis is needed. Take Option 10 and option 11 as the starting point, not precluding any other numbers. - Adopt the average values from companies (updated to include company proposals from RAN4#111) as summarized below and specify the values for ΔTRxSRS for bands whose FUL_high is lower than the FUL_low of n79. Table: Averaged values based on proposals Operating Bands; ΔTRxSRS t1r6 (dB); ΔTRxSRS t2r6 (dB); ΔTRxSRS t1r6-t2r6 (dB). Average; Band n41, n77, n78; 4.0; 3.5; 4.5. - Option 10: Adopt the average values from companies (updated to include company proposals from RAN4#111) as summarized below and specify the values for ΔTRxSRS without considering an additional breakpoint for bands whose FUL_high is lower than the FUL_low of n104. Table: Averaged values based on proposals Operating Bands; ΔTRxSRS t1r6 (dB); ΔTRxSRS t2r6 (dB); ΔTRxSRS t1r6-t2r6 (dB). Average; Band n79, n104; 5.4; 4.7; 5.9. - Option 11: Adopt the average values from companies (updated to include company proposals from RAN4#111) as summarized below and specify the values for ΔTRxSRS considering an additional breakpoint for bands whose FUL_high is lower than the FUL_low of n104. Table: Averaged values based on proposals Operating Bands; ΔTRxSRS t1r6 (dB); ΔTRxSRS t2r6 (dB); ΔTRxSRS t1r6-t2r6 (dB). Average; Band n79; 5.4; 4.7; 5.9. Band n104; 5.7; 5.0; 6.3.

In this contribution, we give some further discussions on the SRS antenna switching related to 6Rx.

# 2	Discussion

It can be seen that the SRS antenna switching (ΔTRxSRS) requirements for band n41, n77 and n78 were approved in the WF. However, there are no consensus for band n79 and band 104 due to more discussions are needed on whether to consider an additional breakpoint for bands whose FUL_high is higher than the FUL_low of n104.

In current 38.101-1 specification, the breakpoint for bands for the ΔTRxSRS requirements requirements is only for band n79, even when the band n104 was introduced, there are no such additional breakpoint introduced for band n104 supporting 4Rx. We try to recall the history but we found there are no discussions on the ∆TRxSRS requirement when n104 was introduced. Given the current situation, it could be interpreted that the current values (breakpoint for n79) could be reused. For 6Rx, if RAN4 decide to introduce an additional breakpoint for bands whose FUL_high is higher than the FUL_low of n104, then it may be also needed to revisit the 4Rx ∆TRxSRS requirement for n104 since we think the situation for 4Rx and 6Rx are the same considering the similar reasons which are ultra-high frequency for band n104 where higher total loss would be foreseen.

Observation 1: There is no additional breakpoint introduced for band n104 supporting 4Rx.

Observation 2: If introduce an additional breakpoint for bands whose FUL_high is higher than the FUL_low of n104 for 6Rx, then it may be also needed to revisit the ∆TRxSRS requirement for n104 4Rx.

For the values, companies proposed different ΔTRxSRS requirements with differernt RFFE architecture and different implementations margin assumption. It is true that different RFFE architectures would cause different ∆TRxSRS values since the IL depends on which types of and how many switchers (such as SPDT, DPDT, SP4T, and etc). However, there are no discussions and agreements on the reference architectures and assumption, instead RAN4 discussed the value directly. Also the different implementations margin assumption will also impact the eventual results. Therefore, we think it is hard to say who is right or who is wrong although we think some values may not be reasonable.

By looking at the option 11, the delta values for n79 and n104 are quite close, i.e. only 0.3~0.4dB gap. We think such small gap could be negligible and the values in either option 10 and option 11 are already high which can be reused for band n104 to absorb the higher IL caused by higher frequency. We prefer not to introduce an additional breakpoint for bands whose FUL_high is higher than the FUL_low of n104. In an other words, to use the same ∆TRxSRS values for bands n79 and n104.

Proposal 1. Do not consider an additional breakpoint for bands whose FUL_high is higher than the FUL_low of n104.

For the values, we prefer to adopt the values in Option 10. But meanwhile, considering the gaps between n79 and n104 in Option 11 are small, alternatively the maximum values in Option 11 for a certain txry case can also be acceptable for us, which is.

| Operating Bands | ΔTRxSRS t1r6 (dB) | ΔTRxSRS t2r6 (dB) | ΔTRxSRS t1r6-t2r6 (dB) |
| --- | --- | --- | --- |
| Band n79, n104 | 5.7 | 5.0 | 6.3 |

Proposal 2: For the ∆TRxSRS requirements for bands n79 and n104, to adopt the values in Option 10. Alternatively, to consider the values below:

| Operating Bands | ΔTRxSRS t1r6 (dB) | ΔTRxSRS t2r6 (dB) | ΔTRxSRS t1r6-t2r6 (dB) |
| --- | --- | --- | --- |
| Band n79, n104 | 5.7 | 5.0 | 6.3 |

# 3 Conclusion

In this contribution, we give some further discussion on the 6Rx SRS antenna switching, including REFSEN ∆TRxSRS values. The conclusions are:

Observation 1: There is no additional breakpoint introduced for band n104 supporting 4Rx.

Observation 2: If introduce an additional breakpoint for bands whose FUL_high is higher than the FUL_low of n104 for 6Rx, then it may be also needed to revisit the ∆TRxSRS requirement for n104 4Rx.

Proposal 1. Do not consider an additional breakpoint for bands whose FUL_high is higher than the FUL_low of n104.

Proposal 2: For the ∆TRxSRS requirements for bands n79 and n104, to adopt the values in Option 10. Alternatively, to consider the values below:

| Operating Bands | ΔTRxSRS t1r6 (dB) | ΔTRxSRS t2r6 (dB) | ΔTRxSRS t1r6-t2r6 (dB) |
| --- | --- | --- | --- |
| Band n79, n104 | 5.7 | 5.0 | 6.3 |

# 4	Reference

[1]  R4-2414285, WF on requirements for 6Rx, AT&T

[2]  R4-2411884 Further discussion on 6Rx SRS antenna switching requirement, ZTE, Sanechips