# 3GPP TSG-RAN WG4 Meeting # 112-bis	R4-2415912

# Hefei, China, Oct.14th – 18th, 2024

Source:	ZTE Corporation, Sanechips

Title:	Views on power class indication for HPUE DL CA with single UL transmission

Agenda Item:	8.2

Document for:	Approval

- 1	Introduction

During the previous meetings, RAN4 spent nearly two years to discuss the power class indication for HPUE downlink CA with different UL configurations, also associated with the RAN2 LS issue on the relationship amongs the IEs of ue-PowerClassPerBandPerBC-r17 and the legacy IE ue-PowerClass/ue-PowerClass-v1610/ue-PowerClass-1700, powerClass/powerClass-v1610.

However,  there are no conclusions and the discussion was stopped in RAN4 if there is no further offline consensus

In last RAN planery, this issue was also raised and in the end, RAN task RAN4 to continue discusing this issue.

Way Forward -RAN tasks RAN4 to investigate how to enable HPUE maximum transmit power in downlink CA with a single configured UL -RAN task shall conclude by RAN#106 -RAN4 online meeting time should be limited to <0.1 TUs

- In last meeting, a WF try to capture the agreements but the WF was not approved in the end.

# Way forward Agreements: To enable UE to support higher power class than default power class for single configured UL carrier with DL CA configuration when HPUE requirements for the band combination have not been specified: - Reference sensitivity requirements (e.g. MSD) for the band combination are only verified with default power class transmission, unless otherwise specified - This approach is specific for the case under RAN task and de-coupled from MSD discussion under other WIs - UE is allowed to enable higher power class than default power class of single configured UL carrier with DL CA configuration when single carrier requirements with higher power class requirements and default power class requirements for the CA configuration exist in specification - It is optional for UE to support higher than default power class for the above band combinations.

- In this contribution, we provide some further discussions HPUE power class indication.

- 2	Discussion

In terms of the previous discussion, it seems the common understanding is that it allows UE optionally report higher power than the band combination power classes for HPUE DL CA with  single configured UL. One of the potential issue whether or not or how to reflect the Rx requirements (i.e. MSD) when there exist some HPUE DL CA combinations who have no MSD been defined but the MSD is defined for the corresponding PC3 DL CA. In this case, companies think it would cause ambiguity since no MSD is defined for the HPUE band combination.

For PC2 band combination, it seems there are no other choices but to use the PC3 MSD for the test if there is no corresponding MSD defined for PC2 band combination;

For PC1.5 band combination, there are two power back off behaviour, i.e. ΔPPowerClass = 3dB or ΔPPowerClass = 6dB, depends on the  P-max or the reported duty cycle  UE capabilities. For the former one, the power back off behaviour is from PC1.5 to PC2 and for the latter one, the  power back off behaviour is from PC1.5 to PC3. Therefore, we think either PC3 or PC2 MSD requirements can be selected.

In general, larger UL power class will cause larger MSD requirement, a UE support HPUE DL CA with single UL configured who can pass the test using PC3’s MSD can pass the test using PC2’s MSD as well if the corresponding PC2 MSD requirements not defined but introduced at the late stage.

Proposal: For a UE supports HPUE DL CA with single UL configured, if UE report UL higher power via ue-PowerClassPerBandPerBC-r17 than the band combination power classes, but the corresponding MSD is not defined , MSD of highest specified power class should be used as a reference.

The above proposal doesn’t mean the MSD for HPUE DL CA is not needed to be defined. Of course, it shall be needed to define the MSD requirements in the specification, and it belongs to basket WID HPUE band combination work.

- 3	Conclusion

In this paper, we provide some discussions on the issue on the ue-PowerClassPerBandPerBC-r17(R4 16-8). The conclusion are:

Proposal: For a UE supports HPUE DL CA with single UL configured, if UE report UL higher power via ue-PowerClassPerBandPerBC-r17 than the band combination power classes, but the corresponding MSD is not defined , MSD of highest specified power class should be used as a reference.

- 4	Reference

[1] RP-241625, Moderator's summary for offline discussion on CA Power Class Support, AT&T

[2] RP-241221, CA Power Class Support, Qualcomm Incorporated

# [3] R4-2411888,Views on power class indication for HPUE DL CA with single UL transmission, ZTE Corporation, Sanechips