# 3GPP TSG-RAN WG4 Meeting #112bis                               R4-2415913

# Hefei, Anhui, China, 14th – 18th October, 2024

# Title: 	Discussion on UE RF for type 4a and type 4b for NonCol_intraB_ENDC_NR_CA

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	6.5.2.1

# Document for:	Approval

# 1 Introduction

In RAN #103 meeting, A new work item on support of intra-band non-collocated EN-DC/NR-CA was approved[1].

The core part objectives of the new WID is as below.

The core part of the work item includes: - Specify the core requirements for type 4a/4b capable FWA UE that supports non-co-located scenarios for FR1 inter-band non-contiguous EN-DC with overlapping or partially overlapping bands and/or FR1 intra-band non-contiguous NR-CA for up to 4-layer DL MIMO per CC, where MRTD>CP is assumed. - RF power imbalance requirements of up to 25dB - RRM requirements including MTTD and MRTD requirements and other identified requirement such as scheduling restriction/availability, SCell operation related delay and interruption. - Work is limited to EN-DC/NR-CA for bands 42, n77/n78 - Discuss and if needed, specify the UE behavior when the UE supports type 4a/b but operates in co-located scenarios depending on whether network signaling is provided. R18 handling of UE behavior can be used as a reference.

This contribution shares our views about UE RF for type 4( or type 4a and type 4b).

- Discussion

Based on the conclusion in RAN4 #112 meeting, some RF requirements for type 4a/4b[2] are reached as below.

< Issue 2-1: Revised WID for Type 4 > Agreement: - RAN4 suggests to revise WID to clarify the max number of CCs for Type 4 EN-DC/NR-CA. < Issue 2-2-1: How to capture the power imbalance and REFSENS requirements > Agreement: - No rush to make decision on CR. Discuss it as big CR.

With regard to the specification implement for type 4a/4b, it is agreed in RAN4 # 111 to include RF requirements of type 4a/4b in the same table[3].

< Issue 2-1-2: How to capture the power imbalance and REFSENS requirements > Agreement: - Define the Type 4 receiver imbalance requirements in the same table as for Type 2. - FFS on the details of modification

Due to up to 4-layer DL MIMO(i.e. 4 antenna port) per CC is supported for type 4a/4b UE, the Rx requirements for four Rx ports are applicable for one component carrier. Therefore, the NOTE 4 in TS 38.101-1table 7.10A.2-1 should be updated to include type 4a/4b UE. In addition, one agreed WF[4] pointed out that in Rel-18, it is necessary to clarify the RF requirements for different UE types. Note 3 also needs refinement as type 2 and type 4 may appear in the specification. It is agreed[] that both the capability of type 2 and type 4 are to be reported to the network when type 4 is supported by a UE considering backward compatibility.

Proposal 1. To take the following modification of 7.10A into consideration for power imbalance of type 4 NRCA .

7.10A.1 General Power imbalance requirement is a measure of the receiver’s ability to receive a wanted signal in the presence of another signal with a power imbalance and a specific frequency offset from the wanted signal. Power imbalance requirement in this subclause is only applicable for a UE when A UE capable of intraBandNR-CA-non-collocated-r18 and is not provided with nonCollocatedTypeNR-CA-r18 and is configured with maxMIMO-Layers with value less than or equal to 2. Or a UE capable of intraBandNR-CA-non-collocated-r18 and [intraBandNR-CA-non-collocated-r19] and is not provided with nonCollocatedTypeNR-CA-r18 and [nonCollocatedTypeNR-CA-r19] and is configured with maxMIMO-Layers with value less than or equal to 4.

Table 7.10A.2-1: Power imbalance parameters for intra-band non-contiguous CA

| Test configurations | Carriers | Rx Power in transmission bandwidth configuration (dBm) | channel bandwidth | Center of BWanother Relative to edge of BWwanted |
| --- | --- | --- | --- | --- |
| 1 | Wanted carrier | REFSENS NOTE 4 + 1 | BWwanted ≤ BWanother | < max (5/2* BWanother, 50MHz |
|  | Another wanted carrier | Power of wanted carrier + 25 |  |  |
| 2 | Wanted carrier | REFSENS NOTE 4 + 1 | BWwanted > BWanother |  |
|  | Another wanted carrier | Power of wanted carrier + 25 – 10*log10(BWwanted /BWanother) |  |  |
| 3 | Wanted carrier | REFSENS NOTE 4 + 1 | NA | ≥ max (5/2* BWanother, 50MHz) |
|  | Another wanted carrier | Power of wanted carrier + 25 |  |  |
| NOTE 1: The transmitter shall be set to 24dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3.2-3 with PCMAX_L,f,c as defined in clause 6.2A.4. NOTE 2: BWwanted is the channel bandwidth of wanted carrier. BWanother is the channel bandwidth of another wanted carrier with 25 dB power imbalance. NOTE 3: It’s allowed to use one of test configurations to verify the RX power imbalance requirement for type 2 UE or type 4 UE. NOTE 4: REFSENS is the reference sensitivity level for two antenna port in Table 7.3.2-1b and/or for four antenna port in Table 7.3.2-1b modified by the ΔRIB,4R in Table 7.3.2-2. |  |  |  |  |

# 3 Conclusion

In this contribution, we give our viewpoint about requirements for Type 4a/4b for NonCol_intraB_ENDC_NR_CA.

The conclusions are:

Proposal 1. To take the following modification of 7.10A into consideration for power imbalance of type 4 NRCA .

7.10A.1 General Power imbalance requirement is a measure of the receiver’s ability to receive a wanted signal in the presence of another signal with a power imbalance and a specific frequency offset from the wanted signal. Power imbalance requirement in this subclause is only applicable for a UE when A UE capable of intraBandNR-CA-non-collocated-r18 and is not provided with nonCollocatedTypeNR-CA-r18 and is configured with maxMIMO-Layers with value less than or equal to 2. Or a UE capable of intraBandNR-CA-non-collocated-r18 and [intraBandNR-CA-non-collocated-r19] and is not provided with nonCollocatedTypeNR-CA-r18 and [nonCollocatedTypeNR-CA-r19] and is configured with maxMIMO-Layers with value less than or equal to 4.

Table 7.10A.2-1: Power imbalance parameters for intra-band non-contiguous CA

| Test configurations | Carriers | Rx Power in transmission bandwidth configuration (dBm) | channel bandwidth | Center of BWanother Relative to edge of BWwanted |
| --- | --- | --- | --- | --- |
| 1 | Wanted carrier | REFSENS NOTE 4 + 1 | BWwanted ≤ BWanother | < max (5/2* BWanother, 50MHz |
|  | Another wanted carrier | Power of wanted carrier + 25 |  |  |
| 2 | Wanted carrier | REFSENS NOTE 4 + 1 | BWwanted > BWanother |  |
|  | Another wanted carrier | Power of wanted carrier + 25 – 10*log10(BWwanted /BWanother) |  |  |
| 3 | Wanted carrier | REFSENS NOTE 4 + 1 | NA | ≥ max (5/2* BWanother, 50MHz) |
|  | Another wanted carrier | Power of wanted carrier + 25 |  |  |
| NOTE 1: The transmitter shall be set to 24dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3.2-3 with PCMAX_L,f,c as defined in clause 6.2A.4. NOTE 2: BWwanted is the channel bandwidth of wanted carrier. BWanother is the channel bandwidth of another wanted carrier with 25 dB power imbalance. NOTE 3: It’s allowed to use one of test configurations to verify the RX power imbalance requirement for type 2 UE or type 4 UE. NOTE 4: REFSENS is the reference sensitivity level for two antenna port in Table 7.3.2-1b and/or for four antenna port in Table 7.3.2-1b modified by the ΔRIB,4R in Table 7.3.2-2. |  |  |  |  |

# 4	Reference

- RP-240838, Rel-19 WID_Intra-band non-collocated EN-DC and NR-CA, Apple

- R4-2414453, WF on Rel-19 non-collocated scenario, KDDI

- R4-2410686, WF on Rel-19 non-collocated UE RF requirements, KDDI

- R4-2414454, WF on Rel-18 clarifications of RF requirements for different UE types, Huawei, HiSilicon