# 3GPP TSG-RAN WG4 Meeting # 112-bis	R4-2415917

# Hefei, China, Oct.14th – 18th, 2024

# Source:	ZTE Corporation, Sanechips, CHTTL, Samsung

# Title:	TP for TR 38.746 Annex part: Valid UL configuration

# Agenda Item:	5.8.5

# Document for:	Approval

- Introduction

In the PC3 TR38.719-02-01[1] and TR38.719-03-01[2], the valid UL configurations for NR inter-band CA are included in the Annex part based on the WF [3][4], which are:

TR38.719-02-01:

Annex B: Valid UL configurations For CA_nX-nY two band DL inter band combinations, the following UL configurations are applicable: - One band UL with one CC: nXA, nYA - One band UL with two CC: CA_nXB, CA_nXC, CA_nX(2A), CA_nYB, CA_nYC, CA_nY(2A) - two band UL with one CC per band: CA_nXA-nYA - two band UL with two CC in one band: CA_nXB-nYA, CA_nXC-nYA, CA_nXA-nYB, CA_nXA-nYC The following three UL cluster cases are not supported: CA_nX(2A)-nYA, CA_nXA-nY(2A) Combinations with four UL CCs are not supported.

TR38.719-03-01:

Annex <B> (normative): Valid UL configurations For CA_nX-nY-nZ three band DL inter band combinations with two UL bands, the following UL configurations are applicable: - Two band UL with one CC per band: CA_nXA-nYA. - Two band UL with two CC in one band: CA_nXB-nYA, CA_nXC-nYA, CA_nXA-nYB, CA_nXA-nYC. - The following three UL cluster cases are not supported: CA_nX(2A)-nYA, CA_nXA-nY(2A). - Combinations with four UL CCs are not supported.

The intention for the valid UL configuration are to guarantee the analysis for the valid UL configuration can be captured correctly, and meanwhile to exclude the invalid UL configurations which should not be discussed and introduced in the TR/TS, since the IMD analysis for three UL cluster and four UL CC are too complicated to be included in the RAN4 discussion.

However, there are no such information in the HPUE TR38.746 [5] since there were no HPUE TR template discussed before R19. Nevertheless, the agreements of PC3 NR inter-band CA can be re-used for HPUE NR inter-band CA considering the PC3 band combination is the power fallback behaviour for HPUE band combination, and the corresponding PC3 band combination should be completed in advanced before the introduction of HPUE band combination.

It is important to include the valid UL configuration for the HPUE NR inter-band CA in TR38.746 since it can avoid the UL configurations which are invalid for PC3 to be discussed/introduced to HPUE band combination.

Therefore, in order to keep the consistence with PC3 band combination, it is proposed to include the same information about the valid UL configuration in the TR38.746 [5].

Mo

In this contribution, a TP to TR38.746 is provided to introduce a new Annex to include Valid UL configuration.

- Reference

- [1] TR38.719-02-01,Rel-19 NR Inter-band Carrier Aggregation/Dual Connectivity for 2 bands DL with x bands UL (x=1,2), v0.1.0

- [2] TR38.719-03-01,Rel-19 NR Inter-band Carrier Aggregation/Dual Connectivity for 3 bands DL with x bands UL (x=1,2), v0.1.0

- [3] R4-2410652,WF on two band DL band combination template, ZTE, Skyworks, Nokia, Huawei, HiSilicon

- [4] R4-2410653,WF on three band DL band combination template, Skyworks Solutions Inc., Nokia, ZTE

- [5] TR 38.746, High power UE (power class 1.5 or 2) for NR inter-band Carrier Aggregation (CA) / Dual Connectivity (DC) with/without Supplementary Uplink (SUL) with high power on TDD band(s); (Release 19), v0.1.0

- Text Proposal

----- Start of TP -----

              - Annex A:		Valid UL configurations

For High Power UE CA of 2 or 3 bands DL with 1 or 2 bands UL, the following UL configurations are applicable, subject to the general requirements:

-  One band UL with one CC: nXA, nYA

-  One band UL with two CC: CA_nXB, CA_nXC, CA_nX(2A), CA_nYB, CA_nYC, CA_nY(2A)

-  two band UL with one CC per band: CA_nXA-nYA

-  two band UL with two CC in one band: CA_nXB-nYA, CA_nXC-nYA, CA_nXA-nYB, CA_nXA-nYC

The following three UL cluster cases are not supported: CA_nX(2A)-nYA, CA_nXA-nY(2A)

Combinations with four UL CCs are not supported.

# Annex <B> (informative): Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2024-08 | RAN4 #112 | R4-24xxxx |  |  |  | TR skeleton | 0.0.1 |

----- End of TP -----