3GPP TSG-RAN4 Meeting #112bis	R4-2415925

Hefei, Anhui, China, Oct 14 – 18, 2024

Agenda item:	4.2.1.4

Source: 	Qualcomm Incorporated

Title: 	On RRM core maintenance for RedCap positioning and PRS/SRS bandwidth aggregation

Document for:	Discussion

# Introduction

RRM core maintenance issues for RedCap positioning and PRS/SRS bandwidth aggregation were discussed in RAN4#112 [1]. In this brief contribution, we address open issues on these topics.

# RedCap PRS measurements with FH

In RAN4#111 and RAN4#112 two issues concerning the measurement period for PRS measurements with FH were brought up:

- The definition of the effective number of Rx hops Nhops,  effect used to define the BW requirement did not take into account inter-slot PRS muting (dl-PRS-MutingOption2)

- It was not clear whether number of PRS repetitions used to calculate the effective number of Rx hops Nhops,  effect referred to inter-slot PRS repetitions or intra-slot repetitions of the PRS comb pattern.

For reference, the current definition of effective number of Rx hops is shown below.

The number of hops within a single MG occasion Nhop,i,j is defined as

Nhop,i,j=minNhops,effect,i,j,N3iLper-hop,i,j,Nhop,max,i

where

-	Nhop,max,i is the maximum number of Rx hops signaled by UE in maximumFH-Hops via NR-DL-PRS-ProcessingCapability as specified in [34] for positioning frequency layer i

-	N3i is indicated via UE capability processingPRS-SymbolsDurationN3-r18 for positioning frequency layer i

-	Nhops,effect,i,j is the effective number of Rx hops within a MG instance j,

-	Nhops,effect,i,j=2*NrepPRS, if  Nhopsslot = 2,

-	Nhops,effect,i,j=NrepPRS, if  Nhopsslot = 1,

-	Nhops,effect,i,j=NrepPRS, if  Nhopsslot = 1/2 and MrepPRS >1,

-	Nhops,effect,i,j=NrepPRS-12+1, if  Nhopsslot = 1/2 and MrepPRS =1,

-	where NrepPRS is the number of PRS repetitions within the MG occasion excluding the gap retuning times, MrepPRS is the PRS repetition interval configured by dl-PRS-ResourceTimeGap, Nhopsslot is the applicable number of hops per slot as defined in Table 9.9A.2.6.1-1.

Regarding the second issue mentioned above, it was clarified during the discussion in RAN4#112 that the parameter NrepPRS referenced in the above definition refers to inter-slot PRS repetitions. That was the reason why the formulas above also refer to the repetition interval configured by dl-PRS-ResourceTimeGap, which applies to inter-slot repetitions.

With the above clarification, the remaining issue is how to account for inter-slot PRS muting. There are four different formulas for Nhops,  effect, depending on parameters Nhopsslot and MrepPRS. For the first two cases where  Nhopsslot≥1, i.e. at least one hop per slot, it is straightforward; clarifying that NrepPRS is the number of un-muted inter-slot PRS repetitions within the MG occasion should suffice. For the third case (Nhopsslot = 1/2 and MrepPRS >1) the same clarification applies because adjacent PRS repetitions are separated by at least one slot.

The fourth case requires a different solution because the unmuted PRS repetitions could occur in back-to-back slots or with some slots in between. As pointed out in [2], the current formula does not work for all muting patterns. Our proposal is to count the number of unmuted inter-slot PRS repetitions in even and odd slots and take the maximum between the two values. This solution ensures that at most one unmuted slot is counted per hop and it is a super-set of the solution proposed in [2].

Proposal 1: For PRS measurements with Rx hopping with Nhopsslot≥1 or MrepPRS>1, the current formulas for Nhops,  effect apply with the clarification that NrepPRS is the number of unmuted inter-slot PRS repetitions within the MG occasion, excluding the gap retuning times.

Proposal 2: For PRS measurements with Rx hopping with Nhopsslot=1/2 and MrepPRS=1,

Nhops,  effect=maxNrep, evenPRS, Nrep, oddPRS
where Nrep, evenPRS and Nrep, oddPRS are the number of unmuted inter-slot PRS repetitions in even and odd slots, respectively, within the MG occasion, excluding the gap retuning times.

# PRS bandwidth aggregation

The following issue was captured in the WF from RAN4#112 [1].

Issue 3-3: Core requirements for PRS aggregation

Proposals

- When UE is indicated by LMF in nr-DL-PRS-JointMeasurementRequest-r18 to perform measurements by aggregating resources from multiple PFLs

  - Option 1: UE only measures resources indicated by LMF as linked resources.

    - Remove requirement on Tnon-aggregate from the existing core requirements for PRS aggregation.

  - Option 2: UE measures resources that are indicated by LMF as linked resources and resources that are not indicated by LMF as linked resources.

Option 2 is consistent with the current requirements defined by RAN4. In our view, RAN4 would need a strong reason to revisit and change the current requirements at this point.

Proposal 3: When the LMF requests the UE to perform measurements by aggregating PRS resources from multiple PFLs via nr-DL-PRS-JointMeasurementRequest-r18, the UE measures resources that are indicated by LMF as linked resources and resources that are not indicated by LMF as linked resources. i.e. the current requirements apply.

# Conclusions

Proposal 1: For PRS measurements with Rx hopping with Nhopsslot≥1 or MrepPRS>1, the current formulas for Nhops,  effect apply with the clarification that NrepPRS is the number of unmuted inter-slot PRS repetitions within the MG occasion, excluding the gap retuning times.

Proposal 2: For PRS measurements with Rx hopping with Nhopsslot=1/2 and MrepPRS=1,

Nhops,  effect=maxNrep, evenPRS, Nrep, oddPRS
where Nrep, evenPRS and Nrep, oddPRS are the number of unmuted inter-slot PRS repetitions in even and odd slots, respectively, within the MG occasion, excluding the gap retuning times.

Proposal 3: When the LMF requests the UE to perform measurements by aggregating PRS resources from multiple PFLs via nr-DL-PRS-JointMeasurementRequest-r18, the UE measures resources that are indicated by LMF as linked resources and resources that are not indicated by LMF as linked resources. i.e. the current requirements apply.

# References

[1] R4-2414032, WF on RedCap positioning and PRS/SRS bandwidth aggregation, RAN4#112

[2] R4-2407878, Discussion on core requirements for redCap positioning and PRS bandwidth aggregation, OPPO, RAN4#111

[3] R4-2412647, Discussion on RedCap positioning and PRS/SRS BWA, Huawei, RAN4#112