3GPP TSG-RAN WG4 Meeting #112bis	R4-2415929

Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:	6.15.2.1

Source: 		NTT DOCOMO, INC.

Title: 	Discussions on FR2-1 SSB based L3 measurement delay reduction by optimizing Rx beam sweeping factor

Document for:	Discussion

- Introduction

During last meeting, RAN4 reached some agreements, but there are still many open items and it is summarized in WF [1]. This document focus on the beam sweeping factor reduction and proposes possible enhancements.

- Discussion

  - Searcher assumption

In the last meeting, assumption of number of searchers occupied by optimizing beam sweeping factor was discussed. WF is copied below.

Issue 1-2-3: FFS: RRM measurement with two panels activated, two searchers are occupied by this single carrier [Way forward]: FFS: - Option A/1a: Two searchers are occupied by this single carrier to the requirements of enhanced BSF for single carrier. Discuss CA case later. (HW, CATT, Apple, OPPO, LGE, Xiaomi) - Option B: Consider one searcher for single carrier. (MTK, ZTE, OPPO, QC, Nokia, Intel, vivo, Ericsson, Apple, Xiaomi, CATT)

Since the existing beam sweeping factor is not decided based on the number of searchers, the optimization can be achieved irrelevant to the number of searchers. Thus legacy assumption, i.e. one searcher for single carrier should be used.

Proposal 1: Consider one searcher for single carrier even if beam sweeping factor is optimized.

  - SSB processing delay

In the last meeting, SSB processing delay/time was discussed. WF is copied below.

Issue 1-2-4: FFS: SSB processing delay/time for processing multiple beams received in a SMTC [Way forward]: FFS: - Option 1 (CATT, vivo): No need to add additional processing time due to multiple SSBs within one SMTC. - Option 2 (Apple): allow additional processing time for UE supporting multiple-Rx simultaneous reception for L3 delay enhancement if there is only one searcher available for the processing - Option 3 (Ericsson): Extra SSB post-processing time in several ms, as side effect of BSF enhancement, may be acceptable, but it depends on how much we can gain with BSF enhancement - Option 4 (Intel): - Consider UE baseband processing capabilities when specifying the L3 delay reduction for simultaneous receptions on multiple FR2 SSB-s. - Different (or whether or not) delay reduction applies when the ratio of number of SSB within a burst and time duration of the measurement periodicity varies. - Option 5 (Nokia): - If additional time for SSB processing is needed when UE is measuring multiple beams in one SMTC, RAN4 to consider measurement delay with SSB processing as - a. Tidentify_intra_without_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra+TSSB_processing) ms - b. Tidentify_inter_without_index = (TPSS/SSS_sync_inter + T SSB_measurement_period_inter+TSSB_processing) ms - c. where TSSB_processing = 2 ms

The scenario discussed in this item is UE beam sweeping factor reduction by utilising two panels. The measured SSB should be only one at one time instance and UE uses two panels to measure it, i.e. UE simultaneously measures two spatial signal quality for one SSB. Thus, SSB processing delay/time for this BSF optimization should be same as the legacy requirements.

Proposal 2: SSB processing delay/time for this BSF optimization should be same as the legacy requirements because the BSF optimization should be achieved by simultaneously measurement of two spatial signal quality for one SSB.

  - Power consumption

In the last meeting, power consumption issue was discussed. WF is copied below.

Issue 1-2-5: FFS: Power consumption issue (including conditions to trigger UE using FBS for L3 measurement) - Option 1 (Apple, LGE): For power saving purpose, there is a need to have a mechanism to activate/de-activate L3 fast beam sweeping. The R18 mechanism (i.e., multi-RX operation definition and UAI indication of preference) can be considered as a baseline, while other conditions are not precluded. - Option 2 (HW): - RAN4 shall firstly identify the promising scenario(s) for L3 measurement delay reduction enabled by multi-Rx with clear/significant benefits, which could help to converge the discussion. - One possible scenario to be considered is when there is strong demand of mobility performance (e.g. UE at cell edge or the link is about to break). - It shall not be assumed that UE supporting this feature shall activate multiple panels all the time for all L3 measurement. RAN4 shall discuss the entering and/or exiting conditions for enhanced L3 measurement enabled by multi-Rx taking the targeting scenario into account. - Option 3 (Ericsson/Nokia): NW indicates UE enabling/disabling FBS through L3 or lower layers signaling. - Option 4 (vivo): Power consumption issue is important and BSF reduction of L3 measurement will not trigger UE to activate multi-Rx - Option 5 (Intel): Power consumption is not an issue in the scope since the total power consumption for a handover stays roughly the same even delay is reduced. Whether UE has prior knowledge or cell centre/edge conditions do not affect reduction in BSF but they are addressed in legacy side conditions. - Option 6a (Nokia): Mobility Event triggering BSF reduction - Option 6b (Nokia): FBS is triggered by conditional Handover configuration - Option 6c (Nokia): BSF reduction is always enabled, but used for reduced measurement delay in cell edge and used for reduced scheduling restrictions in cell center (e.g. by extending T_SMTC). - Option 7 (QC): RAN4 to adopt the following framework for the fast UE Rx beam sweeping based L3 measurement and mobility requirements: - NW provides the following criteria for fast beam sweeping application, and the signaling details are FFS - Cell edge condition: Threshold value of absolute L3 SSB-RSRP of SpCell - High speed condition: Threshold value of L3 SSB-RSRP variation on SpCell over a time period T - When the condition of not cell-edge (and not high-speed, if configured) is met, the UE is allowed to fallback Rx beam sweeping factor to the existing N value - Note: the existing criteria defined for the relaxed idle/inactive mode measurement and/or RLM/BFD evaluation can be reused or served as a baseline - Report configuration for the status of fast beam sweeping factor application, and the signaling details are FFS - A TTT-like time window or N310-like timer, which starts running or counting upon the first satisfaction of the condition “not cell-edge (and not high-speed)” is observed by the UE, can be configured to avoid frequent status transitions and reports - Note: the existing report defined for RLM/BFD relaxation status can be reused or served as a baseline - Besides, other explicit signaling (e.g. FR2 CHO, FR HO, GBBR, etc.) may disallow the fallback of UE Rx beam sweeping factor to the existing N value until the signaled configuration is no longer in effect or the relevant task has been completed. - FFS on the application delay of UE Rx beam sweeping factor switch. - Option 8 (MTK): Activating multi-Rx for L3 measurements (intra/inter-frequency) may or may not be always necessary, depending on UE current conditions: - UE location (cell centre or cell edge) - UE mobility (stationary or moving) - Both above - Option 8a (MTK): - On UE mobility status, RAN4 should consider low/medium speed mobility of the UEs as one of the conditions to activate multi-Rx for L3 measurement delay reduction. - RAN4 to discuss UE indication capability to the NW whenever UE requires to deactivate multi-Rx for FR2-1 SSB based L3 measurement delay reduction (e.g., indication due to overheating resulting from activating multiple panels for long time). - Option 9 (ZTE): Due to L3 measurement is long-term operation, power consumption issue could be considered, which may lead to some interaction signalling. But which would not be the applicability condition of applying fast beam sweeping. [Way forward]: discussion can be mainly focus on 3 directions: (1)“additional triggering for this R19 L3 measurement with FBS” (use option 8 for discussion) or (2)“L3 measurement with FBS can be activated/deactivation following on R18 mechanism” (use option 1 for discussion) or (3)“up to network indication” (use option 3 for discussion). Discuss option 1/3/8 together, and then add details from other options if needed.

Some companies assume only handover scenario to trigger optimizing BSF. However, according to previous agreement on issue 1-3, the target scenarios not only for handover.

===

Issue 1-3: Scenarios to use L3 measurement delay reduction by optimizing Rx BSF

Which scenarios are considered to use L3 measurement delay reduction by optimizing Rx BSF:

  - Scenario 1: SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra

  - Scenario 2: SSB based Intra-frequency measurement with MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra

  - Scenario 3: SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter

  - Scenario 4: SSB based Inter-frequency measurement with MG, including TPSS/SSS_sync_inter,  TSSB_time_index_inter and TSSB_measurement_period_inter

  - Scenario 5: Handover

  - Scenario 6: PSCell addition

  - Scenario 7: RRC Re-establishment/RRC Connection Release with Redirection

  - Scenario 8: SCell activation

  - Scenario 9: SCG activation

  - Scenario 10: CGI identification

  - Scenario 11: CSI-RS based intra-/inter-frequency measurements, the CSI-RS is configured associatedSSB. The discussion on CSI-RS configured with associatedSSB could be revisited if SSB based L3 measurement delay reduction is concluded.

Agreement:

  - Scenario 1/2/3/4/5/7 can be considered as first priority. After RAN4 has conclusion(s) on the solution(s) of L3 measurement delay reduction for the baseline scenarios 1/2/3/4/5/7, RAN4 can discuss whether and how the solutions(s) can be extended to the other scenarios. The discussion on other scenarios will NOT delay the WI completion.

===

Therefore if power consumption is really the problem, the solution should be considered case by case. In some scenarios, dynamic NW indication seems to be difficult. The feasible handling for these scenarios is that NW initially indicates FBS activation, then UE switches activation/deactivation via UAI.

Proposal 3: For power saving purpose, the solution should be considered case by case.

- For scenario 1 to 4 (SSB based Intra/Inter frequency measurement with/without MG), the R18 mechanism (i.e., multi-RX operation definition and UAI indication of preference) can be considered as a baseline

- For scenario 5 (handover) and 7 (RRC Re-establishment/RRC Connection Release with Redirection), dynamic NW indication can be used

  - DRX configuration

Since current L3 measurement delay already covers DRX cases, it should be considered as well.

Proposal 4: FBS should cover DRX cases, no need to deprioritize DRX case.

  - Solutions to apply/specify L3 measurement delay reduction

In the last meeting, solutions to apply/specify L3 measurement delay reduction was discussed. WF is copied below.

Issue 1-4: Solutions to apply/specify L3 measurement delay reduction by optimizing Rx BSF [Way forward]: FFS: - Proposal 1 (CATT): - For UE supporting Multi-Rx operation for L3 measurement, the Rx BSF can be reduced to 2, 4 or 6 according to the UE capability. - Proposal 2 (Apple, CTC): - For UE supporting multiple-Rx simultaneous reception, it is proposed to reduce L3 measurement delay by reducing Rx BSF, and Rel-18 Rx BSF reduction in L1 measurement can be used as baseline. - Proposal 3 (Xiaomi): - In order to shorten the overall L3 measurements delay, the smaller RX beam sweeping factor for SSB index acquiring and SSB measurement can be used in comparison with that for PSS/SSS detection. - Proposal 4 (LGE): - If UE supports multi-Rx reception simultaneously for L3 measurement and signal quality for serving cell is low, L3 measurement delay reduction by optimizing Rx BSF could be applied to scenario 1 to 4. - RAN4 to discuss how to reduce M values for SSB based intra- / inter-frequency measurements for UE supporting multi-Rx simultaneous reception - Proposal 5 (Ericsson): - As a result of FBS, RX beam sweep factor can be defined as [4]. - Proposal 6 (Ericsson): - RAN4 to study the requirements relevant to switching between different operations, including: - Transition time period when enabling and disabling FBS in Rel-19. - Transition time period when switching between FBS in Rel-19 and multi-Rx in Rel-18. - Proposal 7 (Samsung): - To define the delay requirements for intra-frequency and inter-frequency measurement w/o MG for L3 multi-Rx measurement, suggest to take N=4, M=20 as the baseline - Whether to/How to define the other values can be based on SLS results (if necessary)

The mechanism of BSF reduction may be same as that of Rel-18 Rx BSF reduction. Therefore the number of beam sweeping factor should be informed to NW as a UE capability as same as Rel-18. The exact value needs more discussion.

Proposal 5: For UE supporting Multi-Rx operation for L3 measurement, the Rx BSF can be reduced according to the UE capability. The exact value needs more discussion.

- Conclusion

In this contribution, we propose possible enhancements.

Proposal 1: Consider one searcher for single carrier even if beam sweeping factor is optimized.

Proposal 2: SSB processing delay/time for this BSF optimization should be same as the legacy requirements because the BSF optimization should be achieved by simultaneously measurement of two spatial signal quality for one SSB.

Proposal 3: For power saving purpose, the solution should be considered case by case.

- For scenario 1 to 4 (SSB based Intra/Inter frequency measurement with/without MG), the R18 mechanism (i.e., multi-RX operation definition and UAI indication of preference) can be considered as a baseline

- For scenario 5 (handover) and 7 (RRC Re-establishment/RRC Connection Release with Redirection), dynamic NW indication can be used

Proposal 4: FBS should cover DRX cases, no need to deprioritize DRX case.

Proposal 5: For UE supporting Multi-Rx operation for L3 measurement, the Rx BSF can be reduced according to the UE capability. The exact value needs more discussion.

References

[1] 3GPP, R4-2414026, “WF for NR_RRM_Ph5_Part1”, Apple