3GPP TSG-RAN WG4 Meeting #112bis	R4-2415930

Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:	6.17.1

Source: 		NTT DOCOMO, INC.

Title: 	Discussions on general aspects of AI/ML for NR Air Interface

Document for:	Discussion

- Introduction

During the past meetings, general aspects has been discussed and several agreements were obtained. This paper further discusses remaining issues in the summary [1].

- Discussion

  - Post deployment testing

The moderator summary for post deployment testing in the last meeting is copied below.

Issue 1-1: Post deployment testing - Proposals - Option 1: Amend the agreements above as below (R4-2413391): - Option 1: Conduct the conformance testing for new/updated AI model/functionality before its deployment in already deployed UEs (considering the UE hardware that it will be used in) - FFS on the feasibility - Option 2: Design the test to verify the performance monitoring and proactive recovery from potential performance degradation - Depend on the other WG progress - Monitoring can be used for managing fallback, model update/model switching/model transfer, if applicable - Other options are not precluded - Option 2: Add Option 3 as below (R4-2413391): - Option 3: Capture model input during conformance testing for later testing of new models. - Option 3: UE shall support at least some default/baseline AI/ML models/features that passed conformance requirements. The respective models can be used as a fallback mechanism. (R4-2411980, R4-2412021 & R4-2412331) - Option 4: Drop Option 1 as framework is not clear and burden on UE vendors is very high - Option 5: RAN4 to consider handling post-deployment verification of updated/new functionality in inactive/standby state/mode before it is allowed to be used along with the inference of the currently used functionality (R4-2412021) - Option 6: RAN4 to send an LS to RAN2 and RAN1 WGs on a need to consider conformance testability of AI/ML enabled functionalities when designing such LCM operations as monitoring/activation/update/fallback for the devices already deployed in the field. (R4-2412021) - Option 7: RAN4 to study and compare the feasibility of Options 1, 2, and 3. RAN4 can inform other groups about the conclusions of this study. (R4-2413391) - Option 8: Others - Recommended WF Options should be discussed separately to see if any agreements can be reached

Observing proposals, two directions are indicated. One is conducting as a conformance test, the other is conducting as a performance monitoring. Considering the feasibility of conformance test, it is difficult to cover all new/updated AI model/functionality testing before deployment from both testing effort and consuming time points. Thus, it can only be conducted as a performance monitoring.

Observation 1: It is difficult to cover all new/updated AI model/functionality by conformance test before deployment from both testing effort and consuming time points.

For performance monitoring, there are two key points to be considered. First is how to fallback if performance is degraded. If the degradation level is critical, the AI model/functionality should be deactivated or fallback to the default/initial AI model/functionality. In order to achieve this fallback method, the performance should be monitored, and fallback should be immediately executed. From RAN4 perspective, monitoring accuracy and execution delay are needed to be specified.

Proposal 1: If the performance degradation level is critical, the AI model/functionality should be deactivated or fallback to the default/initial AI model/functionality during performance monitoring. The monitoring accuracy and fallback execution delay are needed to be specified.

Second is how to reactivate the AI model/functionality. One possible way is conducting conformance test only for these degraded AI model/functionalities. However it may be also infeasible. Another possible way is monitoring the performance until the AI model/functionality becomes better after updating. In order to conduct this procedure, the updated AI model/functionality performance monitoring is needed to be done in parallel with normal measurement, i.e. if the inference accuracy is good enough compared with exact measured value, the AI model/functionality is reactivated. If the actual performance after reactivation is still worse, it can be deactivated or fallback to the default/initial AI model/functionality.

Proposal 2: For reactivating the deactivated AI model/functionality, performance monitoring is needed to be done in parallel with normal measurement, i.e. if the inference accuracy is good enough compared with exact measured value, the AI model/functionality is reactivated.

  - AI/ML model performance

The moderator summary for AI/ML model performance in the last meeting is copied below.

Issue 1-4: AI/ML model performance - Proposals - Option 1: AI/ML based performance should exceed legacy non-AI/ML requirements - Option 2: AI/ML based performance does not necessarily need to exceed legacy performance. There could be other consideration to use AI/ML based algorithms - Option 3: other - Recommended WF To be discussed

The AI/ML model performance requirement depends on usecases. If a usecase employs legacy metric, the AI/ML model performance should be at least as same as that of legacy. If a usecase employs new metric, the comparison between legacy non-AI/ML requirements is not needed.

Proposal 3: If a usecase employs legacy metric, the AI/ML model performance should be at least as same as that of legacy. If a usecase employs new metric, the comparison between legacy non-AI/ML requirements is not needed.

  - Generalization test

The moderator summary for generalization test in the last meeting is copied below.

Issue 1-5: Generalization - Proposals - Option 1: If non-static scenarios/configurations are supported for certain use cases, they can be included as part of generalization tests. - Option 2: take the requirements for inference as the minimum level performance for generalization. - Option 3: For generalization test, performance for other scenarios and/or configurations can be ensured by RAN4 legacy non-AI test - Option 4: Consider non-static scenarios separately from generalization over static conditions - Option 5: Others - Recommended WF To be discussed

The generalization is necessary functionality for all the scenarios. However, generalization test for static scenario can be excluded as same principle as legacy test. Although the specified test only covers the limited test condition, it does not mean the acceptance of significant performance degradation in the condition other than the specified test condition.

Proposal 4: Generalization test for static scenario is not necessarily to be specified. As same principle as legacy test, it does not mean the acceptance of significant performance degradation in the condition other than the specified test condition.

- Conclusion

In this contribution, we proposed our views on general aspects of AI/ML for NR Air Interface.

Observation 1: It is difficult to cover all new/updated AI model/functionality by conformance test before deployment from both testing effort and consuming time points.

Proposal 1: If the performance degradation level is critical, the AI model/functionality should be deactivated or fallback to the default/initial AI model/functionality during performance monitoring. The monitoring accuracy and fallback execution delay are needed to be specified.

Proposal 2: For reactivating the deactivated AI model/functionality, performance monitoring is needed to be done in parallel with normal measurement, i.e. if the inference accuracy is good enough compared with exact measured value, the AI model/functionality is reactivated.

Proposal 3: If a usecase employs legacy metric, the AI/ML model performance should be at least as same as that of legacy. If a usecase employs new metric, the comparison between legacy non-AI/ML requirements is not needed.

Proposal 4: Generalization test for static scenario is not necessarily to be specified. As same principle as legacy test, it does not mean the acceptance of significant performance degradation in the condition other than the specified test condition.

References

[1] 3GPP, R4-2412831, “Topic summary for [112][129] NR_AIML_air”, Qualcomm Incorporated