3GPP TSG-RAN WG4 Meeting #112bis	R4-2415932

Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:	6.18.2

Source: 		NTT DOCOMO, INC.

Title: 	Discussions on RAN4 requirement impacts of AIML for mobility

Document for:	Discussion

- Introduction

During the last RAN plenary meeting, SID of AI/ML for mobility was updated and RAN4 scope was clarified [1].

- Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4] - Study the impacts on requirements based on RAN2 assumptions, and coordinate with RAN2 if needed - Study the testability and interoperability based on RAN2 framework (e.g., number of cells to measure, beams etc.) - NOTE 4: Leverage the work from “AI/ML for NR air interface” led by RAN1 and avoid the duplicate study for testability and interoperability. - NOTE 5: Avoid the overlaps with RAN2 work for evaluation - NOTE 1: RAN1/3 work can be triggered via LS NOTE 2: To avoid duplicate study with “AI/ML for NG-RAN” led by RAN3 NOTE 3: Two-sided model is not included

According to this update and current RAN2 assumption, this paper proposes possible RAN4 study scope.

- Discussion

RAN2 has discussed the evaluation scenario and currently there are 6 study cases as follows.

| Case number | Prioritization | Evaluation scenario combination | target study goal |
| --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal |

According to the table above, there are two categories for this study. The 1st one is temporal domain prediction for both intra-frequency and inter-frequency. The 2nd one is FR2 spatial domain prediction for cell quality derivation. Although many points are under discussions, the KPI is based on how many numbers of measurement can be replaced by predicted value for cell quality evaluation.

Observation 1: Although many points are under discussions, the KPI is based on how many numbers of measurement can be replaced by predicted value for cell quality evaluation.

Based on this study assumptions, impacts on RAN4 requirements can be analyzed for each cases as follows.

In temporal domain case A, the AI/ML model uses L1-RSRP measured in the past time window to predict the L3-RSRP values in the future time sequence. Considering this definition, impacts on RAN4 requirements are at least measurement delay, interruption, scheduling availability, configurable number of cells and number of SSBs, sharing factor between each measurement, and CSSF assumption.

Proposal 1: In temporal domain case A, impacts on RAN4 requirements are at least measurement delay, interruption, scheduling availability, configurable number of cells and number of SSBs, sharing factor between each measurement, and CSSF assumption.

In temporal domain case B, the AI/ML model thins out L1-RSRP measurement to predict the L3-RSRP values. Considering this definition, the total measurement delay may be same as non-AI/ML measurement. Thus, impacts on RAN4 requirements can be interruption, scheduling availability, configurable number of cells and number of SSBs, sharing factor between each measurement, and CSSF assumption.

Proposal 2: In temporal domain case B, the total measurement delay may be same as non-AI/ML measurement. Thus, impacts on RAN4 requirements can be interruption, scheduling availability, configurable number of cells and number of SSBs, sharing factor between each measurement, and CSSF assumption.

In frequency domain, the AI/ML model uses L1-RSRP measurement of serving cell to predict the L3-RSRP values for inter-frequency measurements. This case may completely skip the inter-frequency measurements, thus all the requirements related to the inter-frequency measurements may be impacted.

Proposal 3: In frequency domain, all the requirements related to the inter-frequency measurements may be impacted because whole measurement may be skipped.

In spatial domain, the AI/ML model uses L1-RSRP measurement of limited number of beams to predict the L3-RSRP values. Since Rel-18 AI/ML for air interface intensively works for beam management, firstly the investigation how to use the outcome of Rel-18 AI/ML for air interface is needed. After this investigation, same perspective for other temporal domain studies can be started.

Proposal 4: In spatial domain, firstly the investigation how to use the outcome of Rel-18 AI/ML for air interface is needed. After this investigation, same perspective for other temporal domain studies can be started.

- Conclusion

In this contribution, we proposed our views on RAN4 requirement impacts of AI/ML for mobility.

Observation 1: Although many points are under discussions, the KPI is based on how many numbers of measurement can be replaced by predicted value for cell quality evaluation.

Proposal 1: In temporal domain case A, impacts on RAN4 requirements are at least measurement delay, interruption, scheduling availability, configurable number of cells and number of SSBs, sharing factor between each measurement, and CSSF assumption.

Proposal 2: In temporal domain case B, the total measurement delay may be same as non-AI/ML measurement. Thus, impacts on RAN4 requirements can be interruption, scheduling availability, configurable number of cells and number of SSBs, sharing factor between each measurement, and CSSF assumption.

Proposal 3: In frequency domain, all the requirements related to the inter-frequency measurements may be impacted because whole measurement may be skipped.

Proposal 4: In spatial domain, firstly the investigation how to use the outcome of Rel-18 AI/ML for air interface is needed. After this investigation, same perspective for other temporal domain studies can be started.

References

[1] 3GPP, RP-242393, “Revised SID on AIML for mobility in NR”, OPPO