3GPP TSG-RAN WG4 Meeting #112bis				                        R4-2415953

Hefei, Anhui, China, 14th – 18th October, 2024

Agenda Item:	6.8.3.4

Source:	MediaTek Inc.

Title:	Discussion on RRM requirements for less than 5MHz over NTN

WID/SID:        NR_IoT_NTN_req_test_enh

Document for:	Discussion

# 1  Introduction

In RAN#104 meeting, the “Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN” has been approved in [1], and the impact on RRM requirement for the less than 5MHz has been discussed [2].

# 2  Impact from Less than 5MHz for NR NTN on RRM core requirements

The RRM requirements specified in R18 TN NR_FR1_lessthan_5MHz_BW can be leveraged to update the requirements for FR1 NR NTN for less than 5MHz. The potential impact on RRM requirement is analyzed below, with considerations that it introduces smaller BW for <5MHz (e.g. 12/15/20 PRBs PDCCH transmission parameters), and it only impacts on SSB/PBCH-related procedures while CSI-RS has not impacted.

| The potential RRM impact from “less than 5MHz” | Agreements | Discussion & Proposals |
| --- | --- | --- |
| IDLE/INACTIVE | Agreement in 2024.08 - No core part impact. | n/a. |
| CONN Mobility (Handover) & CONN Control | Agreement in 2024.08 Define HO delay requirement for NTN less than 5MHz, and take extension of delay for TN less than 5MHz as baseline. For NTN specific HO requirement: - Time/location-based CHO delay requirement will be defined for NTN less than 5MHz. - Satellite switch time requirement will be defined for NTN less than 5MHz. - Further discuss the details of the requirements. | Leverage existing TN less than 5MHz, as in 6.1 If the target cell is an unknown intra-frequency cell and the target cell Es/Iot≥-2 dB, then Tsearch = [3] *Trs ms if the target cell SSB BW is 12PRBs, otherwise Tsearch = Trs ms. If the target cell is an unknown inter-frequency cell and the target cell Es/Iot≥-2 dB, then Tsearch = [5] *Trs ms if the target cell SSB BW is 12PRB. Proposal 1: The HO delay requirements for SSB BW of 12PRBs is extended by 2 samples, same principle as in TN. |
| Timing | WF: FFS on - Option 1: - No impacts Proposal: No impact foreseen. | No impact observed from TN less than 5MHz. Proposal 2: No impact on timing requirements due to NTN less than 5MHz. |
| Signaling characteristics (RLM) (CONN), BFD/CBD | Agreement in 2024.08 - Prioritize SSB based RLM and L1 measurement for NTN less than 5MHz WF: FFS on - Proposal 1: whether RAN4 need to define hypothetical PDCCH transmission parameter for RLF/BFD/CBD - Yes - FFS - Proposal 2: The legacy RLM OOS/IS evaluation period can be reused or be the baseline when defining the related requirements for less than 5MHz in NTN scenario | Leverage existing TN less than 5MHz, as in Table 8.1.2.1-3/4. Note CBD is not based on hypothetical PDCCH transmission parameter. Proposal 3: RAN4 to update hypothetical PDCCH transmission parameter of RLM/BFD for to NTN less than 5MHz. |
| Meas. Procedure in CONN | FFS | Leverage existing TN less than 5MHz, as in Table 9.2.6.1-2. Proposal 4: Update Time period for time index detection for to NTN less than 5MHz. |
| SSB L1-RSRP | FFS | No impact observed from TN less than 5MHz. Proposal 5: For L1-RSRP measurement based on SSB there is no RRM impacts. |

# 3  Conclusion

Proposal 1: The HO delay requirements for SSB BW of 12PRBs is extended by 2 samples, same principle as in TN.

Proposal 2: No impact on timing requirements due to NTN less than 5MHz.

Proposal 3: RAN4 to update hypothetical PDCCH transmission parameter of RLM/BFD for to NTN less than 5MHz.

Proposal 4: Update Time period for time index detection for to NTN less than 5MHz.

Proposal 5: For L1-RSRP measurement based on SSB there is no RRM impacts

# References

- RP-241281, Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN

- R4-2413974, WF for RRM of NTN with channel BW less than 5MHz, Moderator (Xiaomi), Aug. 2024.