3GPP TSG-RAN WG4 Meeting #112bis				                        R4-2415954

Hefei, Anhui, China, 14th – 18th October, 2024

Agenda Item:	6.22.2.1

Source:	MediaTek Inc.

Title:	Discussion on RRM requirements for OD-SSB for network energy saving

WID/SID:        Netw_Energy_NR_enh-Core

Document for:	Discussion

# 1  Introduction

In RAN#104 meeting, the “Revised WID: Enhancements of network energy savings for NR” has been approved in [1]. The RRM requirement for On-Demand SSB was discussed in WF [3]. Besides, one corresponding RAN1 LS is received [4] to ask the timeline for On-demand SSB operation on SCell. In this contribution, our view is further provided for the abovementioned topics.

# 2  On-Demand SSB SCell operation (Objective#1 )

RAN1 LS

Based on the RAN1 agreement in the LS [4] as below, the timeline is illustrated in Figure 1.

Agreement

The previous RAN1 agreement made in RAN1#117 is revised as follows.

- For SSB burst(s) indicated by on-demand SSB SCell operation via MAC CE, UE expects that on-demand SSB burst(s) is transmitted from time instance A which is determined as follows.

  - Alt 3-1: Time instance A is the beginning of the first slot containing [candidate SSB index 0 or the first actually transmitted SSB index] of on-demand SSB burst which is at least T slots after the slot where UE receives a signalling from gNB to indicate on-demand SSB transmission

    - The SSB time domain positions of on-demand SSB burst are configured by gNB.

  - Note: The value of T is not less than existing timeline required for UE’s MAC CE processing for SCell activation

  - (Working assumption): T is not less than T_min=  where slot n+m is a slot indicated for PUCCH transmission with HARQ-QCK information when the UE receives MAC CE signaling to indicate on-demand SSB transmission ending in slot n, and  is as defined in current specification.

    - RAN4 to confirm that T_min can be equal to

Figure 1: OD-SSB timeline

In our understanding, the T_min is provide as the MAC decoding time, which is same as the legacy MAC-CE based SCell activation (i.e. T_HARQ + 3ms), in order to let UE know the starting of SSB. So the RAN1 working assumption can be confirmed.

Proposal 1: Confirm that T_min can be equal to

RRM requirement – background

Issue 1-1-1: OD-SSB work plan

- Agreement:

  - RAN4 to first focus on the MAC CE based single SCell activation case as starting point.

  - After RAN4 concludes on the above case, RAN4 may discuss other Scell activation scenarios if needed.

Issue 1-1-3: Deactivated SCell measurement requirement

- Agreement:

  - RAN4 to discuss OD-SSB based deactivated Scell measurement and Scell activation requirement.

    - Includes both FR1 and FR2-1

In the last meeting, the WF [3] is agreed:

Also noted the following terminology during the discussion:

- Case #1: No always-on SSB on the cell

- Case #2: Always-on SSB is periodically transmitted on the cell

- Scenario #2: on-demand SSB can be triggered by gNB when SCell is configured to a UE but before the UE receives SCell activation command

- Scenario #2A: on-demand SSB can be triggered by gNB when UE receives SCell activation command

The corresponding SCell activation timeline for Scenario #2 and Scenario #2A are illustrated in Figure 2 and Figure 3, respectively. Note in Scenario #2, the OD-SSB could come later then the SCell activation comment (Subcase 2 in Figure 2), similar to Scenario #2A, although the OD-SSB activation comment comes earlier.

Figure 2: Scenario #2 - for both Case 1-no always on SSB (with the shaded block) and Case 2 (w/o the shaded block).

Figure 3: Scenario #2A - for both Case 1-no always on SSB (with the shaded block) and Case 2 (w/o the shaded block).

RRM requirement – deactivated SCell measurement

Based on Figure 2 and Figure 3, it is noted that the deactivated SCell measurement applies on the subcase 1 in Scenario #2 (not applicable for Scenario #2A).

One example for the legacy time period for deactivated SCell measurement, as shown below, the CSSF was considered in the legacy and it seems not impacted by the OD-SSB procedure.

Table 9.2.5.1-12: Time period for PSS/SSS detection, deactivated PSCell (FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | Ceil(5 x Kp) x measCyclePSCell x CSSFintra |
| DRX cycle≤ 320ms | Ceil(5 x Kp) x max(measCyclePSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | Ceil(5 x Kp) x max(measCyclePSCell, DRX cycle) x CSSFintra |

Proposal 2: The CSSF should be considered in the OD-SSB based deactivated Scell measurement.

RRM requirement –SCell activation

Based on Figure 2 and Figure 3, it can be observed that the OD-SSB may come later than the SCell activation comment in both Scenario 2/2A and case 1&2, so the T_uncertainty, as the waiting time for the OD-SSB, should be considered.

Proposal 3: For SCell activation requirement, the waiting time for OD-SSB should be considered for both scenario 2/2A in both case 1&2.

# Conclusion

Proposal 1: Confirm that T_min can be equal to

Proposal 2: The CSSF should be considered in the OD-SSB based deactivated Scell measurement.

Proposal 3: For SCell activation requirement, the waiting time for OD-SSB should be considered for both scenario 2/2A in both case 1&2.

# References

- RP-241650, Revised WID: Enhancements of network energy savings for NR

- Chair notes RAN1_117 eom1

- R4-2413885, WF on Netw_Energy_NR_enh, Ericsson, Aug. 2024.

- R1-2407565, LS on timeline for On-demand SSB operation on SCell, Apple, Aug. 2024.