3GPP TSG-RAN WG4 Meeting #112bis				                        R4-2415957

Hefei, Anhui, China, 14th – 18th October, 2024

Agenda Item:	6.27.3

Source:	MediaTek Inc.

Title:	Discussion on RRM requirements for IoT NTN phase 3

WID/SID:        IoT_NTN_Ph3-Core

Document for:	Discussion

# 1  Introduction

In RAN#104 meeting, the “Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3” has been approved in [1], and the objectives of this WID are listed below. In this contribution, we provide our view on the RRM requirement impacts.

# 2  LS on Msg3 transmission without Msg1/Msg2 (enhanced EDT)

Regarding UL synchronization for contention based Msg3 transmission without Msg1/Msg2, one following RAN1 LS [5] has been received to confirm the Note 1 and Note 2 as below.

RAN1’s response: From RAN1 perspective, assuming Note 1 and Note 2 are satisfied, an RRC Idle UE with a pre-compensated TA (i.e., the one used for Msg1 transmission during random access for IoT NTN) can satisfy requirement for Msg3 transmission/reception without Msg1/Msg2 at least in the following cases: - NB-IoT with 3.75kHz SCS. - eMTC CE mode A. Note 1: RAN1 assumes NTA=0 for Msg3 transmission without Msg1/Msg2 Note 2: RAN1 assumes that the RAN4 UL synchronization requirements specified in Table 7.20A.2-1 and Table 7.24A.2-1 of TS 36.133 apply to the Msg3 transmission without Msg1/Msg2 in NB-IoT over NTN and eMTC over NTN, respectively. Note 3: RAN1 may further discuss the cases of NB-IoT with 15kHz SCS and eMTC CE mode B … ACTION: RAN1 respectfully asks RAN4 to confirm whether RAN1’s assumptions in Note 1 and Note 2 are valid.

For the Note 1, as replied in the RAN4 LS [4], RAN4 assumed that N_TA is not configured by the network for MSG3 transmission without Msg1/Msg2, and RAN4 will follow the conclusion of the other working groups in the future, if any.

Proposal 1: Regarding Note 1, RAN4 assumed that NTA_Ref is not configured by the network for MSG3 transmission without Msg1/Msg2, and RAN4 will follow the conclusion of the other working groups in the future, if any.

RAN4’s feedback: For Msg3 transmission without Msg1/Msg2, UE can meet UE UL transmit timing requirements defined for IoT NTN in TS 36.133[attached in Annex] from UE RRM requirement definition point of view. Note 1: RAN4 assumed that NTA_Ref is not configured by the network for MSG3 transmission without Msg1/Msg2. RAN4 will follow the conclusion of the other working groups in the future, if any. Note 2: For NB-IoT with 15kHz SCS, the timing error may be larger than half CP.

For the Note 2, it can be confirmed, as R4 has replied the similar question in the RAN4 LS [4] last meeting as below.

Proposal 2: Confirm Note 2 in LS R1- 2407548.

# 3  Requirement for Msg3 transmission without Msg1/Msg2 (enhanced EDT)

One open issue is the RRM (timing) requirements for the R19 enhanced RACH-less EDT procedure, i.e. Msg3 transmission without msg1/ Random Access Response (RAR). Besides, it is noted that there was no RRM requirements defied for R15 TN RACH-based EDT procedure.

However, it is noted that the timing requirement has covered the NTN PUR, specifically in 4.6A.3 in 36.133, where PUR is also operating without Msg1/Msg2, which is analogous to the scenario the R19 enhanced (RACH-less) EDT procedure and it can be a useful reference.

## 7.20A	UE transmit timing for NB-IoT for Satellite Access

….

### 7.20A.2	Requirements

The UE initial transmission timing error shall be less than or equal to Te where the timing error limit value Te is specified in Table 7.20A.2-1. This requirement applies when it is the first transmission in a DRX cycle or the first transmission in a repetition period (R>1) for NPUSCH and NPRACH, the first transmission after an uplink transmission gap in a repetition period (R>1) for NPUSCH and NPRACH transmission, or it is the transmission on PUR.

Proposal 3: Clarify the NTN timing requirement also covers R19 enhanced RACH-less EDT procedure (Msg3 transmission without Msg1/Msg2)

# Conclusion

Proposal 1: Regarding Note 1, RAN4 assumed that NTA_Ref is not configured by the network for MSG3 transmission without Msg1/Msg2, and RAN4 will follow the conclusion of the other working groups in the future, if any.

Proposal 2: Confirm Note 2 in LS R1- 2407548.

Proposal 3: Clarify the NTN timing requirement also covers R19 enhanced RACH-less EDT procedure (Msg3 transmission without Msg1/Msg2)

# References

- RP-241624, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, MediaTek Inc. (Rapporteur)

- R2-2405769, LS on UL synchronization for contention based Msg3 transmission without Msg1/Msg2, ZTE, May 2024.

- R4-2414039, WF on RRM requirements for R19 IoT NTN Phase 3, MediaTek inc, Aug. 2024.

- R4-2414114, Reply LS to RAN2 on UL synchronization for contention based Msg3 transmission without Msg1/Msg2, ZTE, Aug. 2024.

- R1-2407548, Reply LS to RAN2 on UL synchronization for contention based Msg3 transmission without Msg1/Msg2, ZTE, Aug. 2024.