3GPP TSG-RAN WG4 Meeting #112bis                                 R4-2415962

Hefei, Anhui, China, 14th – 18th October, 2024

Source: 	MediaTek Inc.

Title: 	Discussion on IMT parameters for 14800 to 15350 MHz.

Agenda Item:	6.2.4.2

Document for:	Approval

- Introduction

In RAN#103, a new SI [1] was agreed upon to study the IMT parameters for the frequency ranges of 4400 to 4800 MHz, 7125 to 8400 MHz, and 14800 to 15350 MHz. In RAN4#112 meeting, two WFs were agreed in [2] and [3] to capture the agreements regarding the radio parameters of the UE and the BS.

In this paper, we provide our views on the radio parameters for the UE and BS for 14800 to 15350 MHz.

- IMT Parameters

Several IMT parameters for the 14800 to 15350 MHz frequency range remain to be discussed. In this paper, we present our views on these remaining parameters.

  - BS parameters

BS antenna array size for urban macro

In the last meeting, companies discussed various BS antenna array sizes based on the implementation feasibility, cost, and coverage performance. The relevant agreements captured in WF [3] are provided as follows:

Issue 2-1: BS antenna array and sub-array size - Proposals - Option 1: BS array size as 1536 and the sub-array size as 4 (CATT) - Option 2: At least 1k elements (Qualcomm) - Option 3: 1536=16x24 (Sub Array size 4) per polarization. (Nokia) - Option 4: Two options based on the simulation assumptions (dual polarization): (Ericsson) - 2048 AEs: 8x32 array and 4x1 sub-array - 1536 AEs: 8x16 and 6x1 sub-array - Recommended for further discussions - 3072 AEs dual polarization: - 16x24 array and 4x1 sub-array - 4096 AEs dual polarization: - 8x32 array and 8x1 sub-array - 16x32 array and 4x1 sub-array - 2048 AEs dual polarization: - 8x32 array and 4x1 sub-array - 1536 AEs dual polarization: - 8x16 and 6x1 sub-array - Other options are not precluded. Issue 2-2: BS antenna array size - Proposals - Option 1: Prioritize 4x4 array for Indoor scenarios (Ericsson) - Recommended WF - Further discuss the proposals

Based on our evaluation results presented in [4], at least 1024x2 (x2 for dual polarizations) BS antennas are required to achieve a comparable DL coverage to that of FR1 BS in the 15GHz frequency range. The most stringent scenario is UMa ISD450m, of which the DL throughput evaluation results are copied in Figure 1 from [4]. Note that number of UE antennas assumed to be 4 in the simulation. Other detail parameters can be found in [4].

Figure 1. DL spectrum efficiency performance of 4GHz and 15GHz with FR1-like and FR2-like UE antenna architectures.

For UL, we also conducted a system-level throughput analysis. Some simulation parameters different from DL case are provided as follows:

| Parameter | Configuration |
| --- | --- |
| UE antenna architecture | FR1-like 1/2/4-Tx for 4GHz; FR1-like 4-Tx for 15GHz |
| UE transmit power control | Follow NR TPC |
| Maximum UE transmit power | 23dBm for 4GHz; 23dBm/26dBm for 15GHz |
| MIMO mode | SU-MIMO |
| Precoding method | NR codebook-based |

Figure 2. UL spectrum efficiency performance of 4GHz and 15GHz with FR1-like UE antenna architecture.

The significant path loss at higher frequencies necessitates a greater number of antennas to maintain adequate signal strength and coverage. When the number of BS antennas is less than 1024x2, a UE operating in this frequency range may experience degraded network service compared to the existing NR network, primarily due to a limited link budget, which is particularly problematic for cell-edge users. This is especially critical in ensuring that cell-edge users receive a consistent and reliable service. To ensure non-degraded network performance, it is preferrable to deploy more than 1024x2 BS antennas, if there are no feasibility issues.

- Observation 1: At least 1024x2 (dual polarizations) BS antennas are required to achieve a comparable DL/UL coverage to that of FR1 BS in the 15GHz frequency range.

- Proposal 1: To ensure non-degraded network performance, it is preferrable to deploy at least 1024x2 (dual polarizations) BS antennas for 14800MHz to 15350MHz.

  - UE parameters

UE antenna type

In our previous paper [4], we compared the spectral efficiency (SE) performance of different UE architectures, specifically FR1-like vs FR2-like. The results demonstrated that FR1-like UE outperformed FR2-like UE in most scenarios, assuming that four antenna elements (AEs) are active simultaneously for both UE types. This superior performance of FR1-like UE can be attributed to its ability to leverage higher MIMO layer gains under favourable channel conditions. Even when high MIMO layers cannot be enabled, FR1-like UE can still perform digital BF techniques, such as MRC, to enhance link robustness.

In contrast, for FR2-like UE to achieve higher SE than FR1-like UE, it would require a larger number of AEs at the UE. However, this is impractical from an implementation standpoint due to form-factor limitations in the 15 GHz bands. The physical constraints of integrating a higher number of AEs into the UE make it challenging to realize the potential SE gains for FR2-like architectures.

Therefore, we suggest taking FR1-like UE as a reference UE architecture for 14800MHz to 15350MHz with four-antenna for both TX and RX. Regarding the modeling of FR1-like UE for coexistence simulations, we have provided an alternative in our companion paper [5], which models the digital BF capability for the UE assuming the use of four-antenna for both TX and RX.

Observation 2: FR1-like UE outperformed FR2-like UE in most scenarios in terms of SE, assuming that four antenna elements (AEs) are active simultaneously for both UE types.

Proposal 2: RAN4 to consider FR1-like UE antenna architecture as the baseline for 15GHz.

UE Rx antenna number

From above simulations, we see that 4Rx is sufficient to achieve reasonable performance compared with FR1 DL deployment. Therefore, 4Rx for DL can be assumed as the baseline for 15GHz.

Proposal 3: RAN4 to consider 4Rx UE for DL as the baseline for 15GHz.

UE ACLR/ACS

ACLR and ACS values are derived from the adjacent channel coexistence simulation. Currently, the adjacent channel coexistence simulation for the 14800 to 15350 MHz range is still ongoing. Therefore, UE ACLR and ACS values should not be concluded until the adjacent channel coexistence results are available.

Proposal 4: RAN4 to consider not concluding on the UE ACLR and ACS values until the outcomes of the adjacent channel coexistence studies are available.

UE Maximum output power

In TR 38.921, 23 dBm is used as the maximum output power for the co-existence evaluation of 6425-7125 MHz. Besides, in the last RAN4 meeting, RAN4 agree to capture 23 dBm as the typical maximum output power in the LS to the ITU for the 7125-8400 MHz frequency range and other power classed are not precluded. Uplink coverage is crucial for the rollout of the wireless communication system, and operators have shown significant interest in a higher maximum output power. Based on UL throughput simulation, we see 26 dBm UL power as a very important enabler to ensure comparable UL throughput performance at cell edge. Therefore, for the 14800 to 15350 MHz frequency range, we propose 26 dBm as the baseline for the UE's maximum output power, and higher power classes, such as 29 dBm, can also be considered.

Proposal 5: RAN4 to consider 26 dBm as the baseline for the UE's maximum output power, and higher power classes, such as 29 dBm, can also be considered.

UE Noise figure

The following was agreed at RAN4#112:

UE noise figure for both simulation and LS reply

FFS the below options:

  - Option 1: 11 dB i.e. follow simulation assumption

  - Option 2: 8 dB

  - Option 3: More than 11 dB

According to TR 38.921, a noise figure of 9 dB is used as the baseline assumption, and 13 dB is used as the optional assumption for the co-existence evaluation for the 6425 – 7125 MHz and 10 - 10.5 GHz bands. A noise figure in the [9, 13] dB interval was finally agreed upon for reporting to ITU WP5D sharing studies. Additionally, a noise figure of 13 dB for the LS reply was agreed upon in the last RAN4 meeting for the 7125-8400 MHz frequency range. Based on our analysis, considering the higher implementation loss for the frequency range of 14800 to 15350 MHz, we propose 15 dB as the UE NF value.

Proposal 6: RAN4 to consider 15dB as the UE NF value.

UE Sensitivity

According to TR38.921, sensitivity is not considered a critical parameter for sharing and compatibility studies and it was agreed not to mention any specific value for this parameter. Additionally, the sensitivity for the LS reply in the 7125-8400 MHz frequency range was addressed. Therefore, the sensitivity for the UE can be considered "to be specified."

Proposal 7: RAN4 to consider the sensitivity for the UE as “to be specified”.

- 3	Conclusion

In this contribution, we have presented our perspectives on the IMT parameters for 14800 to 15350 MHz for UEs, and have provided the following proposals:

- Observation 1: At least 1024x2 (dual polarizations) BS antennas are required to achieve a comparable DL/UL coverage to that of FR1 BS in the 15GHz frequency range.

- Proposal 1: To ensure non-degraded network performance, it is preferrable to deploy at least 1024x2 (dual polarizations) BS antennas for 14800MHz to 15350MHz.

Observation 2: FR1-like UE outperformed FR2-like UE in most scenarios in terms of SE, assuming that four antenna elements (AEs) are active simultaneously for both UE types.

Proposal 2: RAN4 to consider FR1-like UE antenna architecture as the baseline for 15GHz.

Proposal 3: RAN4 to consider 4Rx UE for DL as the baseline for 15GHz.

Proposal 4: RAN4 to consider not concluding on the UE ACLR and ACS values until the outcomes of the adjacent channel coexistence studies are available.

Proposal 5: RAN4 to consider 26 dBm as the baseline for the UE's maximum output power, and higher power classes, such as 29 dBm, can also be considered.

Proposal 6: RAN4 to consider 15dB as the UE NF value.

Proposal 7: RAN4 to consider the sensitivity for the UE as “to be specified”.

- 4	References

- RP-240787, “Study on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz”, Ericsson.

- R4-2414184, WF on 15GHz UE parameters, Qualcomm

- R4-2414283, WF on 15GHz BS parameters, ZTE

- R4-2411776, Coverage study for 14800 to 15350 MHz, MediaTek

- R4-2415435, Coexistence study for 14800 to 15350 MHz