3GPP TSG-RAN WG4 Meeting #112bis	R4-2415966

Hefei, China, Oct 14 – Oct 18, 2024

Source:	Nokia

Title:	On AI/ML CSI Compression Two-Sided Model Logistical Issues

Agenda item:	6.17.2.2

Document for:	Discussion

Introduction

At the previous RAN#105 meeting the question of AI model and dataset logistics was discussed further and the RAN Plenary agreed to formally start the infrastructure/platform for RAN4 [1]:

- RAN Plenary agrees to formally start implementing the infrastructure/platform needed by RAN4 for data sharing - MCC to work out the details of the data sharing platform with input from RAN4 experts - Discussion on the platform setup with MCC to be triggered as soon as possible - Inputs to MCC based on previous RAN4 discussions (data size, etc) - RAN4 to discuss the technical aspects (data formats, model formats etc. for purposes of studying the two-sided CSI compression) in the next RAN4 meeting

In RAN4, several offline discussions and two preliminary calls with MCC took place already since RAN4#111. Some conclusions can be found in the RAN4 reflector, and the latest post-RAN#112 discussions are captured in [2]. However, no formal agreements were made yet in RAN4 that could be used as an input to MCC for the design of the infrastructure/platform. Additionally, it is necessary to agree on the technical aspects of the model/dataset that will help the exchange of these data between the companies.

Discussion

On inputs to MCC

As discussed before, there are two different objects that need to be shared between the companies:

- ML models, specific to use cases, and

- Datasets, that for example they can be used for further aggregation, training, or verification/testing of the AIML models.

  - Some companies discussed training and testing datasets separately. However, testing datasets are usually smaller in size (10-25%) than the training ones. Therefore, it if training dataset sharing is enabled no issues is expected with testing datasets.

There is no need to identify a specific dataset type (e.g. training vs testing) when providing RAN4 input to the MCC.

In addition to the type of shared data there are three main aspects that need to be treated:

- The size of the shared data

  - This was already discussed before offline, and we just need to confirm our previous conclusions

- The retention period of shared data, i.e., for how long the models and the datasets need to be stored

- The ways to store and identify the data, i.e., file storage and naming rules.

The identified dataset and model sized are captured in [2] as follows:

Data size needs for RAN4 CSI compression testing feasibility study Training data - Training data size ~1GB up to 10GB(depends on what data will be used and the number of samples) /company (one instance) - Total space needed would depend on the number of contributing companies and meetings - Upper bound should be ~60GB per company per year, possible ~1TB per year to accommodate all companies - Data sharing might not be needed in every meeting - This data size is only for the current use case (CSI compression testing feasibility study) - Larger datasets could be needed/used in future studies depending on the use case. It would be good to maintain some flexibility AI/ML Model Storage/Sharing Model size: ~7MB –100MB - Total space needed would depend on the number of companies contributing and meetings - Upper bound should be 600MB per company per year -> ~10GB per year (assuming contributions in each meeting within a year) - These assumptions are specific to the CSI compression use case study in RAN4 Testing and inference data will be a subset of the training dataset so no need to consider the data size separately - Testing dataset should be 10-25% of the training dataset - Partitioning or any other details to be discussed separately

RAN4 still need to confirm the conclusions above on the upper bound for dataset/model sizes and storage needs per meeting:

The upper bound of dataset size for currently considered use-case (CSI compression testing feasibility study in RAN4) is up to ~10GB per company per meeting and up to ~160GB for all contributing companies per meeting.

  - Larger datasets could be used in the future studies depending on the use case so some flexibility in the dataset size should be allowed.

The upper bound of model size for currently considered use-case (CSI compression testing feasibility study in RAN4) is up to ~100MB per company per meeting and up to ~1.6GB for all contributing companies per meeting.

Regarding the retention period of the datasets and models, it will be necessary to distinguish

- whether the model/dataset is shared in between the companies during the study, i.e., only for the study purposes,

- or if it needs to be captured as a part of the TR or potentially even of the TS.

In the first case, it will be sufficient to store data only during the timeline of the SI or WI, i.e., for 18 -24 month.
However, in the second case, the same retention policy as for regular TDocs should be applied. It will be reasonable to assume that the upper bound of the storage needed for such data is not more than listed above for one meeting for all companies, i.e., 160 GB for dataset(s) and 1.6 GB for models.

The retention period of the models/datasets shared only for the study purposes can be the length of the 3GPP release, i.e., 18-24 month.

The retention policy of the models/datasets agreed to be captured in the outcome of the study or as a part of the TR can be the same as for regular TDocs/TRs.

The upper estimate of the storage needed to capture the outcomes of the study (i.e., with long retention policy) can be on the level ~160 GB for datasets and ~1.6 GB for models, i.e., as per one meeting for all companies.

Based on the preliminary discussions with MCC, it became clear that the current practice of uploading 3GPP TDocs is not very appropriate for the AIML models and datasets used for study purposes, at least because of the TDoc retention policy (i.e., all uploaded TDocs needs to be stored for backed-up for a very long time) and due to the size of the files, especially datasets. On the other hand, we understood that FTP will stay as the file exchange platform. FTP over TLS (FTPS) and HTTP interface are also considered as possible enhancements by MCC. We also would like to suggest enabling Large File Support (LFS) to support large file transfers (files larger than 2GB).

Due to the large size of the files to be exchanged (especially for the datasets) the sufficient upload/download speeds need to be ensured and Large File Support (LFS) for the FTP server would be beneficial.

The use of FTP server as a platform will a simple but practical solution, at least to provide the means to exchange and capture datasets and models in a centralized place during the RAN4 study. However, since TDoc reservation and correspondingly TDoc ID assignment approach will not be used, it will be necessary to ensure that there are no clashes in between the file names shared by the companies, that the files can be distinguished by their names and that the names are not too long.

File(s) belonging to a single model/dataset to be packed into a single zip archive.

It is necessary to agree on the rules for naming/identification of shared model/dataset (zip) files: if unique ID cannot be assigned automatically, file naming template can be agreed in RAN4, e.g., Meeting#_AI_DataType_Company_Version.zip.

RAN4-specific technical aspects

Another aspect of reference model sharing is the format of the models. Since the reference models are expected to be executed by different companies, on different SW/HW platforms, with or without specific optimization, the format of the reference models should not be proprietary or hard for conversion into any other formats. Hence, we think that the model format used for sharing should be limited to a list of the most popular formats or to the formats that can be easily converted in between each other.

The following discussions are captured in [2]:

AI/ML Models - Possible formats to be used for the AI/ML models are ONNX, PyTorch and TensorFlow - Formats will have to be discussed with MCC to see which ones can be used in 3GPP - There are pros/cons for each format, most companies seem to favor ONNX as it is open source, however, more discussion is needed to agree on a format - Further details on the format (contents of the files and what information needs to be provided) will have to discussed/agreed after deciding on the format

Additionally, in offline discussions, one of the companies have raised a question whether it will be possible to train encoder based on the fixed decoder in ONXX format coming from another company. Therefore, we performed the following experiment:

- Trained our own encoder and decoder models in PyTorch

- We saved the decoder model in ONXX format

  - following the tutorial: https://pytorch.org/tutorials/beginner/onnx/export_simple_model_to_onnx_tutorial.html

  - The latest ONNX version 1.18.0 was used for the experiment: Introduction to ONNX - ONNX 1.18.0 documentation

    - It is important to define the inputs/outputs of the model accurately.

- The decoder model in ONNX format was imported again and it was found out the training of the encoder is possible with imported ONNX model

Based on our experiment it is possible to train the encoder based on the decoder shared and imported via ONNX, at lest for the latest version of ONNX (1.18.0) and in PyTorch.

RAN4 to consider using the latest version (e.g., 1.18.0 and later) of the ONNX (Open Neural Network Exchange) to share the models in between the companies.

Even though the architecture of the models is currently agreed in the RAN4 study experiment on test encoder design, this may not be the case in the future. Additionally, the shared model file may contain only encoder or only decoder or both. Therefore, it will be beneficial to include a description of the content, as well as the inputs and outputs of the models, in the same zip file as the model. For example, when exporting and importing the ONNX model the inputs should be defined:

Export: torch_model = MyModel() torch_input = torch.randn(1, 1, 32, 32) onnx_program = torch.onnx.dynamo_export(torch_model, torch_input) Import: onnx_input = onnx_program.adapt_torch_inputs_to_onnx(torch_input)

It is necessary to include a description of the shared model file(s) (i.e., into the zip archive), specifying at least the content (e.g., encoder/decoder or both) and the inputs/outputs of the model(s) (or the reference to the TDoc with the description of those).

Regarding, the dataset sharing, similar aspects need to be discussed as for the models [2]:

- Training/Testing/Inference data: - Possible formats to be used for the dataset which brought up are: NumPy, Matlab and Parquet (numPy seems to be preferred by most companies) - Formats will have to be discussed with MCC to see which ones can be used in 3GPP. - The details of the data formatting (how data is represented, what is contained in each field, etc) will have to be discussed and agreed after the file format will be decided

MATLAB (i.e., .mat) file could be considered as a valid option for dataset files. However, the exchange of large file would require the use of HDF5 (Hierarchical Data Format version 5) file format. Using .mat files that are saved in the HDF5 format typically requires MATLAB or compatible software that can read MATLAB's .mat files. While the underlying HDF5 format is open and can be read by many tools, the .mat file format includes MATLAB-specific metadata and structures that may not be fully interpretable by generic HDF5 readers.

Regarding the parquet file format, it can be still a good open-source alternative, if for some reason, NumPy (i.e., .npy file format) cannot be used.

Since shared datasets quite probably will be used in popular ML frameworks (e.g., PyTorhc or TensorFlow), it makes sense to use Python-native file formats. As discussed in our accompanying paper on CSI compression, we already tried to use the datasets provided by the other company in NumPy format without any major issues. However, it was still necessary to perform conversion of dataset to a uniform format that could be used for training/testing. For example, one of the datasets used CSI saved in complex values, whereas in the other only real values were used.

RAN4 to consider sharing dataset in NymPy format (i.e., .npy files). If this is not acceptable to all companies, then open-source Parquet file format can be considered as well.

RAN4 to agree on shared dataset structure for CSI compression use-case based on the encoder input, i.e., specify the dimensions of the dataset (a, b, c, d, e), where, a is the total number of sample; b = 2 – real and imaginary part; c – number of sub-band (13); d – number of ports (32); e – optionally, number of layers (1 or 2).

Metadata/description of the dataset should be provided together with it, i.e., included in the zip archive.

Conclusion

In the paper, we have analyzed the potential needs of the platform for sharing AI/ML models and datasets for CSI compression use-case in RAN4. Additionally, we elaborate what else needs to be agreed in RAN4 to simplify such an exchange in between the companies, including, the file formats, metadata/description and data structure.

The following Observations and Proposals were made:

On Input to MCC:

Proposal 1: There is no need to identify a specific dataset type (e.g. training vs testing) when providing RAN4 input to the MCC.

Proposal 2: The upper bound of dataset size for currently considered use-case (CSI compression testing feasibility study in RAN4) is up to ~10GB per company per meeting and up to ~160GB for all contributing companies per meeting.

- Larger datasets could be used in the future studies depending on the use case so some flexibility in the dataset size should be allowed.

Proposal 3: The upper bound of model size for currently considered use-case (CSI compression testing feasibility study in RAN4) is up to ~100MB per company per meeting and up to ~1.6GB for all contributing companies per meeting.

Proposal 4: The retention period of the models/datasets shared only for the study purposes can be the length of the 3GPP release, i.e., 18-24 month.

Proposal 5: The retention policy of the models/datasets agreed to be captured in the outcome of the study or as a part of the TR can be the same as for regular TDocs/TRs.

Proposal 6: The upper estimate of the storage needed to capture the outcomes of the study (i.e., with long retention policy) can be on the level ~160 GB for datasets and ~1.6 GB for models, i.e., as per one meeting for all companies.

Proposal 7: Due to the large size of the files to be exchanged (especially for the datasets) the sufficient upload/download speeds need to be ensured and Large File Support (LFS) for the FTP server would be beneficial.

Proposal 8: File(s) belonging to a single model/dataset to be packed into a single zip archive.

Proposal 9: It is necessary to agree on the rules for naming/identification of shared model/dataset (zip) files: if unique ID cannot be assigned automatically, file naming template can be agreed in RAN4, e.g., Meeting#_AI_DataType_Company_Version.zip.

On RAN4-specific technical aspects:

Observation 1: Based on our experiment it is possible to train the encoder based on the decoder shared and imported via ONNX, at lest for the latest version of ONNX (1.18.0) and in PyTorch.

Proposal 10: RAN4 to consider using the latest version (e.g., 1.18.0 and later) of the ONNX (Open Neural Network Exchange) to share the models in between the companies.

Proposal 11: It is necessary to include a description of the shared model file(s) (i.e., into the zip archive), specifying at least the content (e.g., encoder/decoder or both) and the inputs/outputs of the model(s) (or the reference to the TDoc with the description of those).

Proposal 12: RAN4 to consider sharing dataset in NymPy format (i.e., .npy files). If this is not acceptable to all companies, then open-source Parquet file format can be considered as well.

Proposal 13: RAN4 to agree on shared dataset structure for CSI compression use-case based on the encoder input, i.e., specify the dimensions of the dataset (a, b, c, d, e), where, a is the total number of sample; b = 2 – real and imaginary part; c – number of sub-band (13); d – number of ports (32); e – optionally, number of layers (1 or 2).

Proposal 14: Metadata/description of the dataset should be provided together with it, i.e., included in the zip archive.

- References

- RP-242350, WF on AI/ML Logistical Issues, CIACT, Qualcomm, RAN#105, Melbourne, Australia, September 9-12, 2024.

- RP-241955, AI/ML Logistical Issues, Qualcomm, RAN#105, Melbourne, Australia, September 9-12, 2024.