TSG-RAN Working Group 4 (Radio) meeting #112-bis	R4-2415973

Hefei, China, 14th to 18th October 2024

Source:	Ericsson

Title:	On the topic BS transmitter co-existence spurious emissions requirement limits for bands in upper part of FR1

Agenda item:	3

Document for:	Discussion

# Introduction

To this meeting (RAN4#112-bis) RAN4 have received a Liaison Statement (LS) from ETSI TC MSG/TFES [1] about relevance of transmitter spurious emission requirement limits for bands defined in the upper region of FR1.

In ETSI TC MSG/TFES discussion on Rel-17 versions of Harmonized Standards for base station specifications (i.e. E-UTRA BS, MSR BS, AAS BS, NR BS) is ongoing. During the discussion it was brought to attention that coexistence requirement for transmitter spurious emissions, for 3GPP operating bands introduced in Rel-17/18 versions of 3GPP specifications may require some further checking.

In this contribution we provide some further discussion on the need to check background for co-existence limits for spurious emission requirement. The work to further check relevance of BS RF co-existence spurious emission requirements is related to other discussions on improving BS RF requirements in ongoing work item for BR RF evolution (NR_BS_RF_req_evo).

# Discussion

During the discussion in ETSI TC MSG/TFES it was brought to the attention that some requirement limits for the transmitter spurious emission co-existence requirement (e.g. as captured in TS 38.141-1, Table 6.6.5.5.1.3-1 for BS type 1-C, BS type 1-H and TS 38.141-2, Table 6.7.5.4.5-1 for BS type 1-O) may require further analysis and checking. More specifically, RAN4 need to reassess the relevance of re-using transmitter spurious emissions co-existence limits originally established for bands in the lower region of FR1 for bands in the upper region of FR1 (e.g., band n48, n77, n78, n79, n102, n104).

Analysing the current requirement levels in TS 38.141-1, Table 6.6.5.5.1.3-1, one can notice that the same basic limit of -52 dBm per MHz is used for 450 MHz frequency bands, as well as for the high frequency bands above 3500 MHz and up to 7125 MHz.

Since the co-existence requirement limits have direct impact on base station RF filter suppression, it is essential that the technical background considering assumptions and modelling are relevant. For AAS base stations the number of filters needed scale with the number of transceiver branches which drives building practice challenges and cost. Therefore, RAN4 need to check relevance of requirement limits for bands above 3500 MHz.

In the LS from ETSI TC MSG/TFES, RAN4 have been requested to look at the relevance of transmitter spurious emission requirements for bands in the upper region of FR1.

Action to RAN4:

ETSI MSG TFES respectfully asks 3GPP RAN WG4 to consider the above information in their discussions and to reevaluate if the current limits are appropriate for all bands, considering the different conditions relevant for each band. ETSI MSG TFES would also appreciate if RAN4 could share the assumptions taken for the derivation of TX spurious emissions coexistence limits in BS specifications, for both conducted and radiated requirements.

Considering the complete scope of RAN4 BS RF specifications it can noticed that also other requirement levels have been re-used from bands originally establish in the lower region of FR1 to new bands defined in the upper region of FR1. Like the requirements for co-location spurious emission and colocation receiver blocking. Even though these requirements are optional the same technical issues as for co-existence are relevant for co-location requirements. If RAN4 takes on the task to reevaluate relevant of requirement limits for bands in the upper region of FR1, a general approach including all requirement should be adopted.

To reevaluate co-existence requirement levels with the intention to find relevant requirement levels a study will be required. For a successful coexistence evaluation in RAN4 relevant simulation parameters for base station (including radio and antenna characteristics) and deployment scenario should be established. Due to the complexity of such work, the work should take place over a number of meetings within the context of a RAN4 project. The best way forward would be to include it in the ongoing Rel-19 BS RF evolution (NR_BS_RF_req_evo-Core) Work Item (WI) rather than being handled as regular maintenance or a separate new SI/WI.

To include co-existence into the scope of the BS RF evolution WI, a revision of the Work Item Description (WID) at the RAN plenary meeting in December (RAN#107 in Madrid) would be required.

If work is initiated to reevaluate relevance of co-existence spurious emission requirements for bands in the upper region of FR1 also other requirements should be considered, e.g. BS co-location requirements. For co-location requirements the requirement levels are also re-used from technical background established for lower frequencies within FR1.

Observation 1: There are other requirements than co-existence limits that have been reused for bands in the upper region of FR1, e.g. co-location spurious emission limits.

Observation 2: RAN4 could extend the scope of the ongoing BS RF evolution WI to include work related to check a potentially update co-existence requirement level for bands above 3500 MHz according to request from ETSI TC MSG/TFES. With such extension the of the WI the WID needs to be updated at RAN#107.

Based on the discussion initiated in this meeting RAN4 can capture more details of the technical issues and prepare a response LS to ETSI TC MSG/TFES with proposed workplan.

# Conclusion

In this contribution we elaborate around the need to further check the relevance of requirements defined for operating bands in the upper region of FR1 (e.g., band n48, n77, n78, n79, n102, n104).

We have made the following observations:

Observation 1: There are other requirements than co-existence limits that have been reused for bands in the upper region of FR1, e.g. co-location spurious emission limits.

Observation 2: RAN4 could extend the scope of the ongoing BS RF evolution WI to include work related to check a potentially update co-existence requirement level for bands above 3500 MHz according to request from ETSI TC MSG/TFES. With such extension of the WI the WID needs to be updated at RAN#107.

It would be appropriate for RAN4 to start a discussion on the technical concern raised by ETSI TC MSG/TFES and prepare a response LS to acknowledge the technical concern and indicate a workplan.

# References

[1]	R4-2414917, “Transmitter spurious emissions: co-existence requirements limits for high frequency bands”, ETSI TC MSG/TFES