3GPP TSG-RAN WG4 Meeting #112-bis	R4-2415974

Hefei, China, Oct 14 –18, 2024

Source:	Nokia

Title:	Discussion on less than 5 MHz NTN UE aspects

Agenda item:	6.8.3.2

Document for:	Approval

Introduction

In this paper, we provide our views on UE aspects for the support of less than 5 MHz channel bandwidth for NR NTN. Core parts of the objectives mentioned in the WID are found below [1].

- Specify the system parameters for channel bandwidth of less than 5 MHz for NR-NTN (NR based Non-Terrestrial Networks) in FR1-NTN bands - Consider the NR-NTN bands n255, n256, and n254 - Add less than 5MHz channel bandwidth - 3MHz with the RAN1 design in Rel-18 - Investigate and if necessary, specify the additional synchronization (sync) raster(s) for the additional channel bandwidth - Leverage the work on less than 5MHz for TN - Note: no RAN1 impact is expected - Specify the necessary SAN RF core requirements for NR-NTN in FR1-NTN bands - Specify the necessary NR-NTN UE RF core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Focus on power class 3 (PC3) for Tx RF requirements - Specify the necessary RRM core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands Specify the necessary signaling to support the above objectives

The first RAN4 meeting for this work item was RAN4#112 and the Wayforward for System parameters and UE RF aspects was agreed in [2]. We discuss the remaining open issues listed in the WF.

Issue 1-1-6: Whether 3MHz channel bandwidth for NR-NTN in FR1-NTN bands is mandatory or optional?

- FFS

Issue 1-2-6: Signalling of SS raster

- FFS

Discussion

SSB design for 3 MHz channel bandwidth is different from other existing channel bandwidths. It cannot be supported by legacy UE design. Therefore, 3 MHz channel bandwidth is an optional feature in UE for TN specifications. There is dedicated UE capability signalling, e.g., support3MHz-ChannelBW-Symmetric-r18 and support3MHz-ChannelBW-Asymmetric-r18 in TN specifications. This signalling is not mandatory present.

We can consider reusing the same signalling or define similar signalling for NTN. We can leave this discussion depending on operator demands whether the signalling is mandatory present and whether the support of the feature is mandatory with signalling.

3 MHz channel bandwidth is optional feature in TN and dedicated UE capability signalling is defined.

Issue 1-3-1: Whether 3MHz channel bandwidth applies to Redcap/eRedcap NTN

- FFS

The same or similar UE capability signalling can be introduced for NTN. Further input from NTN operators is requested before deciding whether signalling is made mandatory present and whether the feature is mandatory with signalling.

Currently 3 MHz channel bandwidth is not applied to RedCap/eRedcap in TN specifications. 3 MHz cannot be simply included in RAN4 specifications since impacts to RAN1 specifications are expected to include Redcap/eRedcap. As RAN1 is not involved in this WI, we cannot apply 3 MHz to RedCap/eRecap.

Including 3 MHz channel bandwidth to Redcap/eRecap NTN is not aligned with the scope of the WID since RAN1 impacts are expected.

Issue 2-1-1: UE maximum output power

- FFS on how to define the additional requirements for transmit power density for 3MHz channel bandwidth in n254

3 MHz channel bandwidth is not included to Redcap/eRedcap NTN.

The additional requirements for transmit power density for band n254 NS_04N and NS_05N in Table 6.2.1-2 of TS 38.101-5 is from EIRP density requirement in 4.2.3 of ETSI EN 301 441 [3]. This requirement is from ITU-R RR 5.364 [4] and two different limits (i.e., peak and mean) apply depending on whether another service in use or not. For 3 MHz, the same regulatory requirement is applied, therefore, the table shall be updated to include 3 MHz.

The power density within the operating band is applicable to 3 MHz channel bandwidth.

The current TS 38.101-5 is not clear enough regarding how these two different limits apply. We may further consider clarifying these two limits in TS 38.101-5.

We further discuss how to clarify two different limits apply in TS 38.101-5.

Issue 2-1-2: MPR

- FFS

Issue 2-1-4: Minimum output power/ Transmit OFF power

- Further check

  - Whether reuse the 3MHz channel bandwidth requirements of NR TN FR1 UE

    - Minimum output power: -40dBm with the measurement bandwidth 2.715 MHz

    - Transmit OFF power: -50dBm with the measurement bandwidth 2.715 MHz

Issue 2-1-5: Occupied bandwidth

- Further check

  - Addition of 3 MHz into Table 6.5.1-1 in TS 38.101-5

Issue 2-1-6: Spectrum emissions mask

- Further check

  - Whether reuse the 3MHz channel bandwidth requirements of NR TN FR1 UE

Issue 2-1-8: NR ACLR

- Further check

  - Addition of 3 MHz into Table 6.5.2.4.1-1 in TS 38.101-5

Issue 2-1-9: General spurious emission

- Further check

  - Whether reuse the TN UE approach for 3 MHz channel bandwidth correspondingly FOOB (6 MHz)

Issue 2-1-12: Other Tx requirements

- Further check

  - Whether there are no specification impact for below Tx requirements.

6.2.4 Configured transmitted power

6.3.3 Transmit ON/OFF time mask

6.3.4 Power control

6.4.1 Frequency error

6.4.2 Transmit modulation quality

6.5.2.4.2 UTRA ACLR

6.5.4 Transmit intermodulation

NTN UE transmitter architecture is in principle the same as TN UE, therefore, MPR, Minimum output power, Transmit OFF power, Occupied bandwidth, Spectrum emission mask, ACLR, general spurious emission, Configured transmitted power, Transmit ON/OFF time mask, Power control, Frequency error, Transmitter modulation quality, UTRA ACLR and Transit intermodulation for 3 MHz channel bandwidth shall follow the requirement in TS.38.101-1.

MPR, Minimum output power, Transmit OFF power, Occupied bandwidth, Spectrum emission mask, ACLR, general spurious emission, Configured transmitted power, Transmit ON/OFF time mask, Power control, Frequency error, Transmitter modulation quality, UTRA ACLR and Transit intermodulation for NTN 3 MHz channel bandwidth shall follow the requirement in TS.38.101-1.

Issue 2-1-11: Spurious emissions for UE co-existence

- FFS

Spurious emissions requirement for UE co-existence is specified in 6.5.3.2 of TS 38.101-5. They are common for all the channel bandwidths.

Spurious emissions requirement for UE co-existence is the same as the existing channel bandwidths.

Issue 2-2-1: Reference sensitivity

- FFS

In TS 38.101-1, as far as UL/DL separation is enough (without significant desense), the Rx sensitivity is 2.2 dB better than 5 MHz. As the NTN spectrum utilization for 3 MHz is the same as TN, the same scaling can be applied to NTN bands n256, n255 and n254.

REFSENS for 3 MHz can be scaled from 5 MHz by 2.2 dB.

Issue 2-2-2: Maximum input level

- FFS

In the current TS 38.101-1 and 38.101-5, the maximum input level for 20 MHz channel bandwidth or less is common (including 3 MHz for TN). We propose to keep the maximum input level for NTN UE 3 MHz is the same as for 5 – 20 MHz.

The maximum input level for 3 MHz is the same as 5, 10, 15 and 20 MHz, i.e., -40 dBm.

Issue 2-2-3: ACS

- FFS

Issue 2-2-4: In-band blocking

- FFS

Issue 2-2-5: Out-of-band blocking

- Further check

  - Define out of band blocking requirement similar to TN for 3MHz  (REFSENS + 6.0 dB)

Issue 2-2-6: Narrow band blocking

- Further check

  - Define narrow band blocking requirement similar to TN for 3MHz  (-18dB Pw)

Issue 2-2-7: Spurious response

- Further check

  - 3 MHz shall be added in table 7.7-1, same column as for 5 and 10 MHz channel bandwidth (REFSENS + 6.0 dB)

Issue 2-2-7: Spurious response

- Further check

  - 3 MHz shall be added in table 7.7-1, same column as for 5 and 10 MHz channel bandwidth (REFSENS + 6.0 dB)

Issue 2-2-8: Intermodulation characteristics

- Further check

  - There is no specification impact.

Issue 2-2-9: Spurious emissions

- Further check

NTN UE receiver architecture is in principle the same as TN UE. ACS, In-band blocking, Out-of-band blocking and Narrow band blocking for 3 MHz channel bandwidth can be reused from TN requirement in TS 38.101-1.

ACS, In-band-blocking, Out-of-band blocking, Narrow band blocking, Spurious response, Intermodulation and Receiver spurious emissions for NTN 3 MHz channel bandwidth shall follow the requirement in TS.38.101-1.

Conclusion

The following Observations and Proposals were made:

Observation 1: 3 MHz channel bandwidth is optional feature in TN and dedicated UE capability signalling is defined.

Observation 2: The same or similar UE capability signalling can be introduced for NTN. Further input from NTN operators is requested before deciding whether signalling is made mandatory present and whether the feature is mandatory with signalling.

Observation 3: Including 3 MHz channel bandwidth to Redcap/eRecap NTN is not aligned with the scope of the WID since RAN1 impacts are expected.

Proposal 1: 3 MHz channel bandwidth is not included to Redcap/eRedcap NTN.

Proposal 2: The power density within the operating band is applicable to 3 MHz channel bandwidth.

Proposal 3: We further discuss how to clarify two different limits apply in TS 38.101-5.

Proposal 4: MPR, Minimum output power, Transmit OFF power, Occupied bandwidth, Spectrum emission mask, ACLR, general spurious emission, Configured transmitted power, Transmit ON/OFF time mask, Power control, Frequency error, Transmitter modulation quality, UTRA ACLRand Transit intermodulation for NTN 3 MHz channel bandwidth shall follow the requirement in TS.38.101-1.

Proposal 5: Spurious emissions requirement for UE co-existence is the same as the existing channel bandwidths.

Proposal 6: REFSENS for 3 MHz can be scaled from 5 MHz by 2.2 dB.

Proposal 7: The maximum input level for 3 MHz is the same as 5, 10, 15 and 20 MHz, i.e., -40 dBm.

Proposal 8: ACS, In-band-blocking, Out-of-band blocking, Narrow band blocking, Spurious response, Intermodulation and Receiver spurious emissions for NTN 3 MHz channel bandwidth shall follow the requirement in TS.38.101-1.

- References

- RP-240857 New WID: Enhanced requirements and test methodology for NR and IoT NTN, RAN#103

- R4-2413528 WF on for [112][312] NR_IoT_NTN_less_than_5MHz_UERF, Xiaomi, RAN4#112

- ETSI EN 301 441 Satellite Earth Stations and Systems (SES); Harmonised Standard for Mobile Earth Stations (MES), including handheld earth stations, for Satellite Personal Communications Networks (S-PCN) operating in the 1,6 GHz/2,4 GHz frequency band under the Mobile Satellite Service (MSS) covering the essential requirements of article 3.2 of the Directive 2014/53/EU

- Radio Regulations, Edition of 2024, ITU-R