3GPP TSG-RAN WG4 Meeting #112-bis	R4-2415976

Hefei, China, Oct 14 –18, 2024

Source:	Nokia

Title:	Discussion on NR NTN system parameters for Ku band

Agenda item:	6.9.3

Document for:	Approval

Introduction

WI on the introduction of Ku Band for NR NTN has been approved in [1]. RAN4 has started the work since RAN4#112 meeting [3][4]. For frequency range definition and numerology for Ku band(s), the following WF has been approved in [5].

Issue 3-1: Frequency range definition Following the scope of the WID, definition of a new frequency range e.g. FR3, is not in scope. Issue 3-2: Numerology - Decision criteria The following technical criteria will be used to determine the feasibility of FR1 and FR2 numerology - UL timing synchronization - Phase noise - Efficient channel utilization - Beam hopping granularity - Link budget UL/DL – as a function of SCS and channel bandwidth Issue 3-3: Numerology – timing aspects The impact of SCS on timing is critical to reliable performance therefore decisions on FR1 and FR2 numerology feasibility should not be made without a thorough analysis of the issues. Issue 3-4: Numerology – phase noise The impact of SCS on susceptibility to phase noise is critical to reliable performance therefore decisions on FR1 and FR2 numerology feasibility should not be made without a thorough analysis of the issues. Issue 3-5: Numerology – efficient channel utilization The ability to efficiently support in future the required aggregate bandwidths [list] of NTN services for different SCS choices based on existing FR1 and FR2 channel bandwidths needs to be fully studied. Issue 3-6: Numerology– beam hopping granularity An analysis of the impact of different SCS on performance, although not critical, can influence the decision on FR1 vs FR2 numerology.

In this contribution, we discuss the numerology for Ku band according to the WF above. Furthermore, we discuss other remaining aspects of system parameters for Ku band.

Discussion

Frequency range

In TS 38.101-5, FR1-NTN frequency range is defined for 410-7125 MHz and FR2-NTN frequency range is defined for 17300-30000 MHz. Furthermore, in TS 38.101-1/2, FR1 is defined for 410-7125 MHz, FR2-1 for 24250 – 52600 MHz and FR2-2 for 52600 – 71000 MHz. There is no existing frequency range for Ku band in both the NTN and TN specifications.

The Ku Band is not within any of the existing frequency ranges.

Agreed in Issue 3-1, defining a new frequency range is out of scope of the WI. Thus, we reuse either FR1-NTN or FR2-NTN for Ku band by extending one of the existing NTN frequency ranges.

From the way forward for issue 3-2, the decision criteria shall be based on the analysis of UL timing synchronization, phase noise, efficient channel utilization, beam hopping granularity and link budgets. These aspects will shortly be discussed in the following:.

The timing synchronization is affected by the choice of SCS. There is the tight coupling between the SCS and the resulting amount of CP we will have available for protecting against timing inaccuracies from outside sources (for instance due to UE inaccuracies in the position determination (in the x,y,z plane) as well as the systematic errors that will arise from the inaccurate modelling of the feeder link (and from the potential inaccuracies in the prediction of the satellite’s position as a function of time). Some of the systematic errors may be possible to address by having the UE reading the GNSS information frequently, but errors in the actual UE location will be difficult to compensate by simple means. Therefore, it would be crucial that we target SCS in the lower end.

On the other hand, larger SCS is more robust against phase noise and Doppler effects as both of them increase for higher carrier frequency. For Ka band, FR2 numerology with 60 and 120 kHz SCS has been adopted. It is expected that lower SCS compared to Ka band is optimum for Ku band as the carrier frequency is approximately half. Although we have not evaluated system or link level performance, 30 and 60 kHz SCS for Ku band would be on par with 60 and 120 kHz SCS for Ka band w.r.t. phase noise and doppler compensation.

For efficient channel utilization, the transmission bandwidth configuration(s) should be maximized within the allocated bandwidth. Priority 1 scenario in WID [1] is to support 125 MHz (UL) and 250 MHz (DL). For FR1, as 4K FFT is the fundamental limitation of physical layer specifications, 250 MHz can never be supported by a single carrier. Aggregating multiple carriers must be considered. Furthermore, the assumption (though not limited to) is to reuse the existing channel bandwidths in FR1 or FR2. Unless we introduce any new channel bandwidth, the granularity of FR2 channel bandwidth is at most 50 MHz, thus, FR1 channel bandwidths would better fit to the allocated channel bandwidths. For example, UL 125 MHz can be supported by 100+20 MHz by aggregating two carriers. It can also be supported by 100+25 MHz, if we introduce 25 MHz to FR1-NTN. As 25 MHz is available in FR1 TN in TS 38.101-1, it should be straightforward to introduce to NTN. For FR2, only option would be to use 100 MHz channel bandwidth, unless we introduce a new channel bandwidth which is not even in FR2 TS in TS 38.101-2. Therefore, FR1 numerology seem to suffice for the objectives of the WI.

Beam hopping granularity is listed as one of the criteria, however, the definition of beam hopping is not clear enough to us and is not found in 3GPP terminology. If the beam corresponds to SSB, higher SCS may be more capable to transmit SSB more frequently than lower SCS. If there is such a need, the proponent may need to clarify the definition of beam hopping itself, beam hopping granularity and scenarios that may affect the numerology. For now, we do not see a specific need to use FR2 numerology for the purpose of supporting more frequent SSBs.

In summary, we think FR1-NTN numerology is better than FR2-NTN for Ku band(s). Therefore, we propose to extend FR1-NTN to cover Ku band(s).

It is proposed to extend FR1-NTN to cover Ku band(s).

It is noted that this FR1 extension should be applicable to NTN only in TS 38.101-5 and TS 38.108. The frequency range 7-24 GHz was studied for TN in TS 38.820. Further, 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz are being studied for IMT-2030. These frequency ranges for TN will be separately discussed from NTN.

FR1-NTN extension covering Ku band(s) can only be reflected in TS 38.101-5 and TS 38.108.

Operating band

In RAN4#112, it was agreed to define a Region 1 band in the following and further discuss additional bands [5].

Issue 1-2: Band definitions Define as a first Ku band for Region 1 (CEPT countries): 10.7 – 12.75 GHz (S-to-E) and 14.0 - 14.5 GHz (E-to-S). Additional band definitions are expected once the applicable regulations have been identified and studied.

The operating band n256 and below is reserved for FR1-NTN bands. So far down to n254 is reserved. We can assign new band numbers starting from n253 for Ku band(s). For the agreed Region 1 band above, the following definition applies.

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n253 | 14000 MHz - 14500 MHz | 10700 MHz - 12750 MHz | FDD |

NTN band n253 is assigned to the Ku band (14000-14500 MHz/10700-12750 MHz).

Once we agree another NTN-FR1 band, a new band number can be assigned in descending order from n252 and downwards.

Channel bandwidth and Subcarrier spacing

Priority 1 in the WI objective is to support 125 MHz (UL) and 250 MHz (DL) aggregated bandwidths based on existing channel bandwidths. As mentioned in the WI objective, for FR1-NTN, 20 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing can be introduced, though it is not restricted to. For FR2-NTN, 50 MHz, 100 MHz, 200 MHz, 400 MHz (120kHz SCS only) channel bandwidths with 120 kHz subcarrier spacing can be introduced.

FR1 option in the WID suggests only 30 kHz SCS, thought we could also introduce 15 and 60 kHz. It is to be further discussed we can only focus on 30 kHz SCS for FR1 option.

Support of NTN CA is not in the scope of this WID. Single carrier operation should be first completed in this WI. Thus operation of aggregated bandwidth 125 MHz UL and 250 MHz DL should be discussed after this WI is completed.

It is proposed to focus on single carrier channel bandwidths in this WI.

As we discussed in 2.1, adding 25 MH channel bandwidth can improve the spectrum efficiency in UL. Since it is not difficult to add the existing channel bandwidth of TN specifications to NTN specficiations, we may consider adding it to Ku band once FR1 numerology is agreed.

It is proposed to consider adding 25 MHz channel bandwidth in FR1-NTN.

Channel raster

NR-ARFCN defined in TS 38.101-5 can be reused to Ku band. For the frequency range of Ku band, the step size of NR-ARFCN is 15 kHz. We propose to assume SCS based channel raster, i.e., 30 kHz for the FR1 option (if 15 and 60 kHz SCS are excluded).

SCS based channel raster is introduced to Ku band(s).

Tx-Rx Separation

There is no clear guideline (or regulations) about their channelization schemes and channel pairing of UL and DL like existing FR2-NTN bands. At this moment, it is not possible to define the fixed channelization and fixed duplex separation. Therefore, flexible duplex may be an only option for now.

Flexible Tx-Rx frequency separation is used for Ku band NTN FDD operation.

Sync raster

Current sync raster design in TS 38.101-5 above 3 GHz can support minimum channel bandwidth at any SCS-based channel raster frequency. The existing sync raster design can be reused for Ku bands.

The existing sync raster design is reused for Ku band(s).

Conclusion

The following Observations and Proposals were made:

Observation 1: The Ku Band is not within any of the existing frequency ranges.

Proposal 1: It is proposed to extend FR1-NTN to cover Ku band(s).

Proposal 2: FR1-NTN extension covering Ku band(s) can only be reflected in TS 38.101-5 and TS 38.108.

Proposal 3: NTN band n253 is assigned to the Ku band (14000-14500 MHz/10700-12750 MHz).

Proposal 4: It is proposed to focus on single carrier channel bandwidths in this WI.

Proposal 5: It is proposed to consider adding 25 MHz channel bandwidth in FR1-NTN.

Proposal 6: SCS based channel raster is introduced to Ku band(s).

Proposal 7: Flexible Tx-Rx frequency separation is used for Ku band NTN FDD operation.

Proposal 8: The existing sync raster design is reused for Ku band(s).

- References

- RP-241690, New WID on Introduction of Ku Band for NR NTN, RAN#104, Shanghai, China, 17 – 20 June 2024.

- TR 38.863, Solutions for NR to support non-terrestrial networks (NTN): Non-terrestrial networks (NTN) related RF and co-existence aspects.

- R4-2413414 Topic summary for [112][314] NR_NTN_Ku_Band_General, RAN4#112

- R4-2413520 Ad-hoc meeting minutes for [112][314] NR_NTN_Ku_Band_General, RAN4#112

- R4-2413519 Way Forward for [112][314] NR_NTN_Ku_Band_General, RAN4#112

- R4-2411950 Discussion on NR NTN system parameters for Ku band, Nokia, RAN4#112