3GPP TSG-RAN WG4 Meeting #112-bis	R4-2415981

Hefei, China, Oct 14 –18, 2024

Source:	Nokia

Title:	Channel locations for UE capable of 40 MHz in n28

Agenda item:	5.20.1

Document for:	Approval

Introduction

RAN#104 has agreed the WI to introduce Power Class 2 and UE 40MHz Channel Bandwidth in NR band n28 [1]. The objectives of the WI are quoted in the following.

The objectives of this WI are to introduce requirements for UE Power Class 2 and 40MHz Channel Bandwidth for NR band n28 as optional features, including the following aspects: - Introduce requirements for PC2 and those associated with following bullets - Both 1Tx and 2Tx PC2 are supported - UL-MIMO is supported. - Specify A-MPR requirements for PC2 - Introduce 40MHz Channel bandwidth located in UL: 703 - 743MHz and DL: 758 - 798MHz for band n28 supporting PC3 and PC2. - Allow flexible channel allocations within the frequency range (UL: 703 - 743MHz and DL: 758 - 798MHz) for BW<40MHz. - Release independence of UE 40MHz Channel bandwidth to Rel-15 is anticipated. - No changes to the existing requirements (38.101-1 v18.5.0) (including A-MPR values and RB allocation regions) for power class 3 with existing channel bandwidths up to and including 30MHz to avoid impacts on existing power class 3 networks. In the event of new regulatory requirements, this assumption can be revisited.

The first RAN4 meeting for this work item was RAN4#112 and the Wayforward on introduction of PC2 and 40MHz CBW in NR band n28 was agreed in [5]. Regarding the channel location and channel raster restriction, as well as the release indepence, the following issues are for further study.

Issue 2-3-8 channel location Agreement: - FFS on how to capture the UL channel location for UE supporting maximum bandwidth of 40MHz in n28. Issue 2-3-9 channel raster Agreement: - The following options can be considered for n28 40MHz - Add the exceptional channel raster point of n28 (UL: 723.04MHz, DL: 778.04MHz) to UE RF specification TS 38.101-1 for UE CBW 40MHz. - UE supporting 40 MHz channel bandwidth in band n28 shall support Enhanced channel raster. Issue 2-3-10 Release independence Agreement: - FFS on the release independence of 40MHz for n28 - Option 1: Rel-15 - Option 2: Rel-16

In this contribution we discuss the channel raster limitations for supporting 40 MHz channel bandwidth.

Discussion

The WI objective specifies the channel bandwidth location in UL 703-743 MHz and DL 758-798MHz. It is not required to support any arbitrary 40 MHz channel location within NR band n28.

BS 40 MHz channel bandwidth has been already introduced as TEI16 feature [2,3]. It is dedicated to the channel raster entry at 723.04 MHz for UL and 778.04 MHz for DL.

BS 40 MHz channel bandwidth at 723.04 MHz for UL and 778.04 MHz for DL has been already introduced as TEI16 feature.

This channel raster entry for BS 40 MHz channel bandwidth (signaled in SIB1) supports 30 MHz UE specific channel bandwidth centered at 718 MHz for UL and 763 MHz for DL. This UE channel bandwidth location is aligned with the lower band edge of n28 with the minimum guard band 592.5 kHz. At the same time, 30 MHz UE specific channel bandwidth is PRB-grid aligned with BS 40 MHz channel bandwidth centered at 723.04 MHz for UL and 778.04 MHz for DL. There is a 40 kHz frequency shift for BS from the legacy channel raster of 100 kHz.

For the legacy UEs that can support only 100 kHz raster and up to 30 MHz channel bandwidth, 40 MHz BS channel bandwidth (signaled in SIB1) should be configured at this raster entry (UL 723.04MHz and DL 778.04MHz). This is because the legacy 30 MHz UEs cannot camp on or operate in this cell if 40 MHz is configured at 100 kHz raster and UE channel bandwidth is off the 100 kHz channel raster. No use case so far exists to place 40 MHz channel other than the BS channel raster at UL 723.04MHz and DL 778.04MHz.

40 MHz channel bandwidth for both BS and UE is only at the raster entry, 723.04 MHz for UL and DL 778.04 MHz for DL.

The new UEs that support 40 MHz must support the 40 kHz shifted channel raster for 40 MHz channel bandwidth. Therefore, the support of Enhanced channel raster 10 kHz is required. Enhanced channel raster is early implementable from Rel-16 [4]. Although there was a proposal to add this raster in the UE spec in the issue 2-3-9 of the WF, it is not necessary to specify the specific UE channel raster entry in UE specifications.

It is not necessary to specify the specific UE channel raster entry in UE specifications.

UE supporting 40 MHz channel bandwidth in band n28 shall support Enhanced channel raster.

The was also a proposal to restrict the channel location for 20, 25, 30 and 35 MHz in RAN4#112 [6]. They were proposed to be either in upper duplexer part of 30 MHz passband (UL 718-748 MHz and DL 773-803MHz) or the lower part 40 MHz passband (UL 703-743 MHz and DL 758-798MHz). This will enable the dual duplexer architecture of 2x40MHz and 2x30MHz and minimize the new test configurations. However, the original motivation to revisit A-MPR requirement is to use a single full duplexer of passband 2x45 MHz to reduce the UE component cost. Therefore, this channel location limitation looks too restrictive. If we are assuming the full duplexer in place, it is better not to consider channel location restriction for channel bandwidths less than 40 MHz in RAN4 core specifications. However, it may be further discussed to limit the test points in RAN5.

The channel location limitation for channel bandwidths less than 40 MHz is better not considered in RAN4 core requirement.

As mentioned in Observation 1: 40 MHz channel bandwidth is specified in Rel-16 BS specifications as a TEI16 feature. UE release independence should be at earliest back to the first release when 40 MHz is introduced to BS specifications; therefore, we propose that the UE release independence of 40 MHz for n28 is from Rel-16.

The release independence of 40 MHz for n28 is from Rel-16.

Conclusion

The following Observations and Proposals were made:

Observation 1: BS 40 MHz channel bandwidth at 723.04 MHz for UL and 778.04 MHz for DL has been already introduced as TEI16 feature.

Proposal 1: 40 MHz channel bandwidth for both BS and UE is only at the raster entry, 723.04 MHz for UL and DL 778.04 MHz for DL.

Observation 2: It is not necessary to specify the specific UE channel raster entry in UE specifications.

Proposal 2: UE supporting 40 MHz channel bandwidth in band n28 shall support Enhanced channel raster.

Proposal 3: The channel location limitation for channel bandwidths less than 40 MHz is better not considered in RAN4 core requirement.

Proposal 4: The release independence of 40 MHz for n28 is from Rel-16.

- References

- RP-241660 New WID: Introduction of Power Class 2 and UE 40MHz Channel Bandwidth in NR band n28, CMCC, CBN, RAN#104

- RP-223523 CR on R16 TS 38.104 to add channel raster exception for band n28, CMCC, RAN#98e

- RP-223524 CR on R17 TS 38.104 to add channel raster exception for band n28, CMCC, RAN#98e

- R4-2410712 Rel-18 RAN4 UE feature list for NR (version 5), CMCC, RAN4#111

- R4-2414274 WF on introduction of PC2 and 40MHz CBW in NR band n28, CMCC, RAN4#112

- R4-2412817 Topic summary for [112][115] NR_n28_PC2_40MHz, CMCC, RAN4#112