3GPP TSG-RAN WG4 Meeting #112bis            	R4-2415983

HeFei, China, 14th-18th, Oct, 2024

Agenda item:	6.16.3

Source:	Samsung

Title:	View and initial simulation results for BS demodulation requirement with MMSE-IRC receiver

Document for:	Discussion

- Introduction

In the last RAN4 meeting, the performance part of Rel-19 WI on demodulation performance requirements enhancement was under discussed. The general assumption for MMSE-IRC and interference profile was discussed, the related agreements are captured in the WF [1] as following

- General - MMSE IRC Receiver Assumption - Reuse the same receiver model for MMSE-IRC in TR 36.884 as below: - Baseline receiver assumption (for performance gain study only) - Test scenario: - Option 1: Cover HomNet and HetNet scenario, i.e., DIP set 1 and set 2 in TS 36.104 - Option 2: Consider the scenario with different TDD patterns between BS and discuss how to model the corresponding interference profile - Interference modelling criteria - Candidate options on interference modelling - Option 1: Use INR-based modelling + SNR-based requirement definition criteria and transfer the DIP value into INR value - Option 2: DIP-based moldeing - Candidate options on interference modelling - Option 1: Reuse the definition of DIP in sub-clause B.6.1 of TS 36.104 - Candidate options on SINR definition - Option 1 For BS type 1-C and BS type 1-H: For enhanced performance requirements type A, the SINR used in this clause is specified based on a single carrier and defined as: Where: is the total signal energy of one intra-cell UE in the subframe on a single antenna connector (for BS type 1-C) or on a single TAB connector (for BS type 1-H). is the summation of the received energy of the strongest inter-cell interferers explicitly defined in a test procedure plus the white noise energy N, in a bandwidth corresponding to the transmission bandwidth over the duration of a subframe on a single antenna connector (for BS type 1-C) or on a single TAB connector (for BS type 1-H). The respective energy of each inter-cell interferer relative to is defined by its associated DIP value. For BS type 1-O: For enhanced performance requirements type A, the SINR used in this clause is specified based on a single carrier and defined as: Where: is the total signal energy of one intra-cell UE in the subframe on a RIB. is the summation of the received energy of the strongest inter-cell interferers explicitly defined in a test procedure plus the white noise energy N, in a bandwidth corresponding to the transmission bandwidth over the duration of a subframe on a RIB. The respective energy of each inter-cell interferer relative to is defined by its associated DIP value.. - Interference modeling - Option 1: Reuse the same interference modeling as captured in B.6.2 in TS 36.104, to transmit random 16QAM symbol for the neighbor PUSCH - Other options not precluded - On interference power level - Reuse the LTE interference power level as a start point, and check performance gain of MMSE-IRC over baseline MMSE - It is not precluded for companies to bring system level simulation on other profiles - FFS how to account for HPUE in existing LTE profiles - Number of interferers - Option1: Consider 1 interferer case for 2 Rx antenna and 2 interferers case for 4/8 Rx antenna - Other options are not precluded - Network type - Only synchronous scenario - Test Parameters and simulation assumptions - Test scope - Option 1: Cover the following SCS and TDD pattern - 15KHz SCS FDD and TDD with pattern 3D1S1U, S=10D:2G:2U - 30KHz SCS TDD with pattern 7D1S2U, S=6D:4G:4U - Other options are not precluded - Channel bandwidth - Candidate options for CP-OFDM - Option 1 - For 15kHz SCS, cover 5MHz, 10MHz and 20MHz - For 30kHz SCS, cover 10MHz, 20MHz 40MHz, 100MHz - Option 2 - Option 2: 10MHz/15KHz and 40MHz/30MHz for initial simulation purpose - Option 3 - Minimum channel bandwidth for each SCS, i.e., 5MHz for 15KHz SCS and 10MHz for 30KHz - Option 4 - Minimum/Maximum channel bandwidth for each SCS, i.e., 5/50MHz for 15KHz SCS and 10/10MHz for 30KHz - Candidate options for DFT-s-OFDM - Option 1 - For 15KHz SCS, 5MHz - For 30KHz SCS, 10MHz - Antenna configuration - Option 1: Cover 2/4/8 Rx with 1Tx - Option 2: Consider only 4Rx and 8Rx gNB - Option 3: Cover 2/4/8 Rx with 1/2/4 Tx - Option 4: Prioritize 4Rx case especially in the case of 2 explicit interferers - Option 5: Prioritize 1Tx - Propagation condition - Option 1 - For the serving PUSCH, use TDLA30-10 and TDLC300-100 propagation channel model for HetNet and HomNet, respectively - Introduce mew propagation channel for testing with desired spread 1000ns for the interference PUSCH, i.e., TDLC1000 - Option 2: Select among legacy TDLA30, TDLB100 - Option 2A - Cover TDLA30-10, TDLB100-400 and TDLC300-100 - Same propagation condition for serving and interference PUSCH - Option 2B - For the serving PUSCH, use TDLA30-10 for HetNet and TDLB100-100 for HomNet - For the interference PUSCH, use TDLC300-100 - Option 2C - Cover TDLB100-400, TDLC300-100 and TDLA30-10 low for study purpose, select one for requirement definition - For the interference PUSCH, use TDLC300-100 - MCS - Cover QPSK/16QAM/64QAM - Option 1A: use MCS 5 (QPSK, R) - Option 1B: MCS index 2, 12, 20 in MCS table 1 - Option 1C: Cover MCS 2, 16 and 20 - Waveform - Option 1: Cover both CP-OPDM and DFT-s-OFDM - Option 2: Cover CP-OFDM only - DMRS configuration - Option 1 DM-RS; DM-RS configuration type; 1. DM-RS duration; single-symbol DM-RS. Additional DM-RS position; pos1. Number of DM-RS CDM group(s) without data; 2. Ratio of PUSCH EPRE to DM-RS EPRE; -3 dB. DM-RS port(s); 0. DM-RS sequence generation; NID0=0, group hopping and sequence hopping are disabled. - Other options are not precluded - PUSCH mapping type - Option 1: Cover both PUSCH mapping type A and type B - Other options are not precluded - Test Metric - Option 1: Use 70% of maximum throughput as metric for NR BS MMSE-IRC demodulation - Other options are not precluded

In this contribution, the view on the remaining issue of test setup for BS demodulation with MMSE-IRC are provides, meanwhile, the initial simulation results are provided for initial analysis

# 2	General

## 2.1	Test Scenario

According to the WID, similar as LTE, both homogeneous and heterogenous deployment are considered [2]. In addition, the widely use of HPUE makes UL co-channel interference from the neighboring cell UE severer than that in LTE system. Therefore, it was necessary to cover HomNet and HetNet scenarios, i.e., DIP set 1 and DIP set 2 in TS 36.104.

For the scenario with different TDD patterns among cells, according the WID, it was assumed that TDD synchronous deployments with aligned UL: DL configuration among cells, therefore, the scenario with different TDD patterns between BS is out of current scope. Meanwhile, the interference profile will be different due to the DL transmission., the system level simulation efforts for deriving the proper interference profile is needed. Therefore, considering the work scope for scenario of MMSE-IRC receiver and schedule for this WI, only cover HomNet and HetNet scenarios, and do not consider the scenario with different TDD patterns.

Proposal 1:  Following the WID, only cover HomNet and HetNet scenarios for requirement with MMSE-IRC receiver, and do not consider the scenario with different TDD patterns

## 2.2	Interference Modeling

Number of Interferers

Regarding number of interferer UE modeling, up to two interferers UE was explicitly modeling. In the LTE stage, based on evaluation, the additional MMSE-IRC gain over MMSE is not significant under 2Rx condition in case the number of interferer number is increased from 1 to 2, considering the limited Rx number to handle the interference. Therefore, the feasible of number interferer UE modeling for MMSE-IRC should be further evaluated.  Based on our initial simulation, there is a significant gain achieved by MMSE-IRC compared with MMSE receiver, it was feasible to model 1 interferer case for 2Rx antenna and 2 interferers case for 4/8 Rx antenna. Therefore, for number of interferers, 1 interferer case for 2 Rx antenna and 2 interferers case for 4/8 Rx antenna can be considered for specifying PUSCH requirement with MMSE-IRC receiver.

Proposal 2:  1 interferer case for 2Rx antenna and 2 interferers case for 4/8Rx can be considered for specifying PUSCH requirement with MMSE-IRC receiver

Interference profile

Regarding the interference Profile, it was agreed in the last meeting that to reuse the LTE interference power level as a start point, and to check performance gain of MMSE-IRC over baseline MMSE receiver.  In general, the deployment of NR will be different compared to LTE, especially large bandwidth is available in FR1 30KHz SCS, and meanwhile the HPUEs are widely deployed, which may result in large interference. Therefore, new interference profile conduced by system-level simulation work may be needed. While from baseband processing with MMSE-IRC receiver perspective, there is no difference foreseen. As shown in our simulation results, the significant gain over MMSE receiver can be achieved based on LTE interference profile. Therefore, it is feasible to reuse LTE interference power for specifying PUSCH requirement with MMSE-IRC receiver.

Different LTE, HPUEs including PC1(7dB higher than PC3 UE) or PC2 (3dB higher than PC3 UE) HPUE are widely deployed for NR system, therefore, the expected interference power will be increased. In that case, how to account for HPUE in existing LTE profiles should be further discussion. An artificial interference was proposed by company in the last meeting, 7 or 3 dB can be added into the INR1 directly, it can be also used to check the performance with MMSE-IRC under high interference condition and try to match the real deployment with avoiding the system level simulation effort. In general, we are fine to this proposal with adding 7dB to check the worst scenario.

Proposal 3: It is feasible to reuse LTE interference power profile for specifying PUSCH requirement with MMSE-IRC receiver.

Proposal 4:  The proposal with adding an artificial interference power into the existing LTE interference profile can be considered to account the impact of HPUE. 7dB can be added directly into the 1st strong interference to check the performance with MMSE-IRC receiver in the worst scenario.

## 2.3	Interference modeling criteria

Regarding the interference modeling criteria, there are two kinds of method, one is DIP-based modeling, another is INR-based modeling.

For DIP-based modeling, the following is the definition of DIP captured in the TR 36.884, where the DIP s defined as the ratio of the power of a given dominant interferer over the total power of all interferers along with the white noise. The DIP is expressed as follows:

DIPi=Ior (i)Ioc

Ioc=j=1Ncell-1Ior (j)+σ2

Where Ior (j) is the average received power from the i-th strongest interferer (Ior (j) implies the average received power from the UE scheduled by the serving cell),  σ2 is the thermal noise power over the considered bandwidth, and  Ncell is the total number of cells including the serving cell. Note that Ior (0) is never included in any DIP calculation.

Then, the uplink wideband SINR γ is defined as:

γ=Ior (0)Ioc

For INR-based, the INR for each interferer can be defined as

INR(i)=Ior (i)σ2

Then,  
Ioc=j=1Ncell-1INR(i)σ2+σ2= INR(i)σ2DIPi

the INR for each interferer can be transferred from DIP as

    - For 1 interferer only scenario:  INR1= DIPi1-DIPi

    - For 2 interferers scenario, INR1= DIP11-DIP1-DIP2,   INR2= DIP21-DIP1-DIP2

Based on above transformation, two kinds of interference modeling can be expressed to each other. Since the interference power will be added into target cell separately in link level simulation, and also can easily to control the interference power. Therefore, it is more proper to use the INR-based modeling for interference modeling with transferring the DIP value into INR value for each interferer.

As for requirement definition, in our view, both SNR and SINR-based requirement definition criteria are feasible. Based on our initial simulation results, the performance gain over MMSE receiver is similar for both SNR and SINR-based requirement definition. Similar as LTE MMSE-IRC, we can still apply the SINR based requirement definition to derive the targeting value based on 70% of TP

Observation 1:  Both INR-based and DIP-based interference modeling can be transferred to each other.

Proposal 5:  Use INR-based modelling + SINR-based requirement definition criteria and transfer the DIP value into INR value

# 3	Test parameters and simulation assumption

In this section, the initial simulation assumptions are provided for evaluation.

Test Scope

According to the WID, both FDD and TDD scenarios are included for specifying FR1 PUSCH requirement with MMSE-IRC. And TDD pattern with 3D1S1U for 15kHz SCS and 7D1S2U for 30kHz SCS should be the typical deployment in the real field. Therefore, it was necessary to cover SCS and TDD patterns as following

  - 15kHz SCS FDD and TDD with pattern 3D1S1U, S=10D:2G:2U

  - 30kHz SCS TDD with pattern 7D1S2U, S=6D:4G:4U

Proposal 6: Cover the following SCS and TDD patterns for specifying PUSCH requirement with MMSE-IRC

  - 15kHz SCS FDD and TDD with pattern 3D1S1U, S=10D:2G:2U

  - 30kHz SCS TDD with pattern 7D1S2U, S=6D:4G:4U

Channel Bandwidth

Regarding the SCS and channel bandwidth, since the Rel-15 network has already deployed in the commercial market, it is more proper to choose the typical SCS and BW applied in current network. Therefore, 15KHz SCS with 10MHz, and 30KHz SCS with 40MHz can be chosen for initial performance evaluation. While considering the test coverage and manufacture declaration, additional channel bandwidth with minimum or maximum for each SCS can be considered for requirement definition for CP-OFDM waveform.

For DFT-s-OFDM waveform, to minimize the test effort, the minimum channel bandwidth can be considered

Proposal 7:

For CP-OFDM waveform, consider 15KHz SCS, 10MHz CBW and 30KHz SCS, 40MHz CBW for initial performance evaluation with MMSE-IRC receiver. Additional channel bandwidth with minimum or maximum for each SCS can be considered for requirement definition, such as 15kHz SCS, 5MHz, 20MHz and 30kHz SCS, 10MHz, 100MHz

  - 15kHz SCS, 10MHz for initial performance evaluation

  - 30KHz SCS, 40MHz for initial performance evaluation

For DFT-s-OFDM waveform, the minimum channel bandwidth can be considered as

  - 15KHz SCS, 5MHz

  - 30KHz SCS, 10MHz

Antenna Configuration for Both Target UE and interference UE

In Rel-15, PUSCH requirement with 2Rx/4Rx/8Rx were specified. The gain of MMSE-IRC compared with MMSE is more significantly due to more Rx diversity achieved for interference mitigation. Therefore, it was necessary to evaluate the performance with different Rx number under different interfered UE modeling scenario.

As for Tx, in Rel-18, 4Tx with legacy DMRS and enhanced DMRS were considered, the related PUSCH requirements with 4Tx was specified. 4Tx is mainly targeting for FWA UE scenario. With large number of Tx with more layers, excepting for inter-cell interference, inter-layer has also impacted on the performance with MMSE-IRC receiver, the gain of MMSE-IRC may be limited compared with MMSE receiver.

During the SI phase of LTE, 1Tx UE scenario is prioritized and 2Tx UE scenario is not covered. In the real-deployment, most of commercial UEs is 1Tx UE. Similarly, we prefer to prioritize the 1Tx UE scenario, not to cover 2Tx UE and not to cover 4Tx UE for both target PUSCH and interference UE.

Proposal 8:  For target PUSCH and interference PUSCH, prioritize the 1Tx UE scenario. Not to cover 2Tx UE scenario, and 4Tx UE scenario.

Propagation condition

To check the per-PRB interference covariance estimation, it is better to choose the channel with high delay spread for the interferers. In Rel-15, several channel models are specified to cover different delay spread and doppler spread, including TDLA30-10, TDLB400-100 and TDLC300-100, where TDLC300-100 can be considered as a starting point. In LTE BS performance with MMSE-IRC receiver, different channel models were applied for target UE and interfered UE. For NR PDSCH with MMSE-IRC, the same channel was applied for both target UE and interfered UE.  Therefore, whether to differentiate the channel model used for target UE and interfered UE should be further discussed and evaluated.

Meanwhile, from the baseband processing perspective, there should be no difference for different channels. There is no necessary to introduce the new channel model, such as TDLC1000.  Therefore, we propose to reuse existing channel models to specify PUSCH requirement with MMSE-IRC receiver.

Proposal 9: FFS to differentiate the channel model used for target UE and interfered UE for performance evaluation with MMSE-IRC receiver. Reusing the existing channel models to specify the PUSCH requirement with MMSE-IRC receiver

The following candidate propagation channel can be considered as initial evaluation

  - Targeting cell:  TDLA30-10, TDLC300-100 and TDLB100-400

  - Interference cell: TDLC300-100

MCS

Regarding the MCS, MCS 16 for interfered UE can be considered for initial evaluation, which is more typical scenario, similar as LTE MMSE-IRC receiver evaluation

For target UE, MCS 2, MCS 16 and MCS 20 can be considered as candidate for initial evaluation. For requirement definition, FFS to down selection, only one MCS for requirement.

Proposal 10: Consider MCS 16 for interfered UE, MCS 2, MCS 16 and MCS 20 as candidate for initial performance evaluation with MMSE-IRC receiver.

  - Targeting cell:  MCS 2, MCS 16 and MCS 20

  - Interference cell: MCS 16

For requirement definition, FFS to down selection one MCS for requirement.

Waveform

Since our preference is to prioritize 1Tx UE scenario, both CP-OFDM and DFT-s-OFDM are feasible. As for starting point to performance evaluation, we can prioritize CP-OFDM firstly. For requirement definition, both CP-OFDM and DFT-s-OFDM waveforms can be considered to guarantee the test coverage

Proposal 11: As for starting point to performance evaluation with MMSE-IRC receiver, CP-OFDM waveform can be considered. For requirement definition, both CP-OFDM and DFT-s-OFDM waveforms could be considered to guarantee the test coverage

PUSCH mapping type

Both PUSCH mapping type A and type B are considered for specifying PUSCH requirement in NR Rel-15. For testing with different mapping types, it was conducted based on the manufacturer declaration with test applicability rule.  Similar as Rel-15, both PUSCH mapping type A and type B can be considered for specifying PUSCH requirement with MMSE-IRC receiver, reusing the existing Rel-15 test applicability rule to test.

Proposal 12: Considering both PUSCH mapping type A and mapping type B for specifying PUSCH requirement with MMSE-IRC receiver, reusing the existing Rel-15 test applicability rule to test.

Test Metric

For test metric, it is straightforward to use the70% of maximum throughput as metric for NR BS MMSE-IRC demodulation to derive the SINR value.

Proposal 13: Use the 70% of maximum throughput as metric for NR BS MMSE-IRC receiver to derive the SINR value

DMRS configuration and cell ID

For DMRS generation with different cell, for simplicity, the cell ID for targeting cell, and interference cells can be set as 0, 1, 2, respectively. And the scrambling ID can be set as cell ID, nSCIDλ=0  and quantity nSCID =1. And the group hopping and sequence hopping are disabled.

|  | Targeting cell | Interferer 1 | Interferer 2 |
| --- | --- | --- | --- |
| DM-RS port | 0 | 0 | 0 |
| Cell ID | 0 | 1 | 2 |
| DM-RS sequence generation | NIDnSCIDλ=NIDcell nSCIDλ=0 λ=0 Group hopping and sequence hopping are disabled |  |  |

Proposal 14: Considering the following for DMRS configuration and cell ID to specify PUSCH requirement with MMSE-IRC receiver

|  | Targeting cell | Interferer 1 | Interferer 2 |
| --- | --- | --- | --- |
| DM-RS port | 0 | 0 | 0 |
| Cell ID | 0 | 1 | 2 |
| DM-RS sequence generation | NIDnSCIDλ=NIDcell nSCIDλ=0 λ=0 Group hopping and sequence hopping are disabled |  |  |

# 4	Initial Simulation Results

In this section, the initial simulation results are provided to check the performance gain of MMSE-IRC over MMSE receiver.

As starting point, only 15KHz SCS ,10MHz, CP-OFDM waveform for both HomNet and HetNet are under evaluation.

Table 1.  PUSCH performance with HomNet scenario, channel model for target cell is TDLA30-10

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (DIP1, DIP2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SINR@70% (MMSE) | SINR@70% (MMSE-IRC) | Gain (SINR) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 2 | TDLA30-10 | TDLC300-100 | (-1.11. -10.91) | 1 | 1+1 (2,11) | MCS 2 | MCS16 | -4.68 | -7.63 | 2.95 | 1.79 | -1.16 | 2.95 |
| 2 | 1 | 2 | TDLA30-10 | TDLC300-100 | (-1.11. -10.91) | 1 | 1+1 (2,11) | MCS 16 | MCS16 | 7.22 | 4.45 | 2.77 | 13.69 | 10.92 | 2.77 |
| 3 | 1 | 2 | TDLA30-10 | TDLC300-100 | (-1.11. -10.91) | 1 | 1+1 (2,11) | MCS 20 | MCS16 | 10.29 | 7.49 | 2.8 | 16.76 | 13.96 | 2.8 |
| 4 | 1 | 4 | TDLA30-10 | TDLC300-100 | (-1.11. -10.91) | 1 | 1+1 (2,11) | MCS 2 | MCS16 | -7.79 | -12.67 | 4.88 | -1.32 | -6.2 | 4.88 |
| 5 | 1 | 4 | TDLA30-10 | TDLC300-100 | (-1.11. -10.91) | 1 | 1+1 (2,11) | MCS 16 | MCS16 | 3.74 | -0.73 | 4.47 | 10.21 | 5.73 | 4.48 |
| 6 | 1 | 4 | TDLA30-10 | TDLC300-100 | (-1.11. -10.91) | 1 | 1+1 (2,11) | MCS 20 | MCS16 | 6.93 | 2.45 | 4.48 | 13.4 | 8.92 | 4.48 |
| 7 | 1 | 4 | TDLA30-10 | TDLC300-100 | (-1.11. -10.91) | 2 | 1+1 (2,11) | MCS 2 | MCS16 | -7.8 | -13.38 | 5.58 | 0.61 | -4.98 | 5.59 |
| 8 | 1 | 4 | TDLA30-10 | TDLC300-100 | (-1.11. -10.91) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 3.73 | -1.36 | 5.09 | 12 | 7.04 | 4.96 |
| 9 | 1 | 4 | TDLA30-10 | TDLC300-100 | (-1.11. -10.91) | 2 | 1+1 (2,11) | MCS 20 | MCS16 | 6.92 | 1.78 | 5.14 | 15 | 10.18 | 4.82 |
| 10 | 1 | 8 | TDLA30-10 | TDLC300-100 | (-1.11. -10.91) | 1 | 1+1 (2,11) | MCS 2 | MCS16 | -9.95 | -15.83 | 5.88 | -4 | -9.37 | 5.37 |
| 11 | 1 | 8 | TDLA30-10 | TDLC300-100 | (-1.11. -10.91) | 1 | 1+1 (2,11) | MCS 16 | MCS16 | 0.83 | -3.88 | 4.71 | 7.3 | 2.59 | 4.71 |
| 12 | 1 | 8 | TDLA30-10 | TDLC300-100 | (-1.11. -10.91) | 1 | 1+1 (2,11) | MCS 20 | MCS16 | 4.03 | -0.77 | 4.8 | 10.5 | 5.7 | 4.8 |
| 13 | 1 | 8 | TDLA30-10 | TDLC300-100 | (-1.11. -10.91) | 2 | 1+1 (2,11) | MCS 2 | MCS16 | -9.94 | -16.92 | 6.98 | -1.54 | -8.52 | 6.98 |
| 14 | 1 | 8 | TDLA30-10 | TDLC300-100 | (-1.11. -10.91) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 0.83 | -4.98 | 5.81 | 9.23 | 3.42 | 5.81 |
| 15 | 1 | 8 | TDLA30-10 | TDLC300-100 | (-1.11. -10.91) | 2 | 1+1 (2,11) | MCS 20 | MCS16 | 4.03 | -1.89 | 5.92 | 12.43 | 6.51 | 5.92 |

Table 2.  PUSCH performance with HomNet scenario, channel model for target cell is TDLC300-100

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (DIP1, DIP2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SINR@70% (MMSE) | SINR@70% (MMSE-IRC) | Gain (SINR) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 2 | TDLC300-100 | TDLC300-100 | (-1.11. -10.91) | 1 | 1+1 (2,11) | MCS 2 | MCS16 | -4.46 | -7.58 | 3.12 | 2.00 | -1.11 | 3.11 |
| 2 | 1 | 2 | TDLA30-10 | TDLC300-100 | (-1.11. -10.91) | 1 | 1+1 (2,11) | MCS 16 | MCS16 | 8.50 | 5.67 | 2.83 | 14.97 | 12.13 | 2.84 |
| 3 | 1 | 2 | TDLA30-10 | TDLC300-100 | (-1.11. -10.91) | 1 | 1+1 (2,11) | MCS 20 | MCS16 | 11.71 | 9.12 | 2.59 | 18.17 | 15.59 | 2.58 |
| 4 | 1 | 4 | TDLA30-10 | TDLC300-100 | (-1.11. -10.91) | 1 | 1+1 (2,11) | MCS 2 | MCS16 | -6.36 | -11.79 | 5.43 | 0.10 | -5.32 | 5.42 |
| 5 | 1 | 4 | TDLA30-10 | TDLC300-100 | (-1.11. -10.91) | 1 | 1+1 (2,11) | MCS 16 | MCS16 | 5.18 | 0.71 | 4.47 | 11.65 | 7.17 | 4.48 |
| 6 | 1 | 4 | TDLA30-10 | TDLC300-100 | (-1.11. -10.91) | 1 | 1+1 (2,11) | MCS 20 | MCS16 | 8.12 | 3.66 | 4.46 | 14.59 | 9.85 | 4.74 |
| 7 | 1 | 4 | TDLA30-10 | TDLC300-100 | (-1.11. -10.91) | 2 | 1+1 (2,11) | MCS 2 | MCS16 | -6.34 | -12.40 | 6.06 | 2.06 | -4.00 | 6.06 |
| 8 | 1 | 4 | TDLA30-10 | TDLC300-100 | (-1.11. -10.91) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 5.21 | 0.17 | 5.04 | 13.00 | 8.57 | 4.43 |
| 9 | 1 | 4 | TDLA30-10 | TDLC300-100 | (-1.11. -10.91) | 2 | 1+1 (2,11) | MCS 20 | MCS16 | 8.14 | 3.07 | 5.07 | 16.54 | 11.47 | 5.07 |
| 10 | 1 | 8 | TDLA30-10 | TDLC300-100 | (-1.11. -10.91) | 1 | 1+1 (2,11) | MCS 2 | MCS16 | -7.55 | -14.58 | 7.03 | -1.09 | -8.12 | 7.03 |
| 11 | 1 | 8 | TDLA30-10 | TDLC300-100 | (-1.11. -10.91) | 1 | 1+1 (2,11) | MCS 16 | MCS16 | 2.68 | -2.77 | 5.45 | 9.14 | 3.70 | 5.44 |
| 12 | 1 | 8 | TDLA30-10 | TDLC300-100 | (-1.11. -10.91) | 1 | 1+1 (2,11) | MCS 20 | MCS16 | 5.38 | 0.38 | 5.00 | 11.85 | 6.85 | 5.00 |
| 13 | 1 | 8 | TDLA30-10 | TDLC300-100 | (-1.11. -10.91) | 2 | 1+1 (2,11) | MCS 2 | MCS16 | -7.56 | -15.59 | 8.03 | 0.85 | -7.19 | 8.04 |
| 14 | 1 | 8 | TDLA30-10 | TDLC300-100 | (-1.11. -10.91) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 2.73 | -3.89 | 6.62 | 11.13 | 4.52 | 6.61 |
| 15 | 1 | 8 | TDLA30-10 | TDLC300-100 | (-1.11. -10.91) | 2 | 1+1 (2,11) | MCS 20 | MCS16 | 5.39 | -0.60 | 5.99 | 13.79 | 7.81 | 5.98 |

Table 3.  PUSCH performance with HomNet scenario, channel model for target cell is TDLB100-400

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (DIP1, DIP2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SINR@70% (MMSE) | SINR@70% (MMSE-IRC) | Gain (SINR) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 2 | TDLB100-400 | TDLC300-100 | (-1.11. -10.91) | 1 | 1+1 (2,11) | MCS 2 | MCS16 | -4.10 | -7.23 | 3.13 | 2.37 | -0.77 | 3.14 |
| 2 | 1 | 2 | TDLB100-400 | TDLC300-100 | (-1.11. -10.91) | 1 | 1+1 (2,11) | MCS 16 | MCS16 | 10.44 | 7.87 | 2.57 | 16.91 | 14.33 | 2.58 |
| 3 | 1 | 2 | TDLB100-400 | TDLC300-100 | (-1.11. -10.91) | 1 | 1+1 (2,11) | MCS 20 | MCS16 | 15.84 | 13.87 | 1.97 | 22.30 | 20.34 | 1.96 |
| 4 | 1 | 4 | TDLB100-400 | TDLC300-100 | (-1.11. -10.91) | 1 | 1+1 (2,11) | MCS 2 | MCS16 | -6.23 | -11.61 | 5.38 | 0.24 | -5.15 | 5.39 |
| 5 | 1 | 4 | TDLB100-400 | TDLC300-100 | (-1.11. -10.91) | 1 | 1+1 (2,11) | MCS 16 | MCS16 | 6.23 | 1.86 | 4.37 | 12.70 | 8.33 | 4.37 |
| 6 | 1 | 4 | TDLB100-400 | TDLC300-100 | (-1.11. -10.91) | 1 | 1+1 (2,11) | MCS 20 | MCS16 | 10.16 | 5.81 | 4.35 | 16.63 | 12.28 | 4.35 |
| 7 | 1 | 4 | TDLB100-400 | TDLC300-100 | (-1.11. -10.91) | 2 | 1+1 (2,11) | MCS 2 | MCS16 | -6.22 | -12.26 | 6.04 | 2.19 | -3.85 | 6.04 |
| 8 | 1 | 4 | TDLB100-400 | TDLC300-100 | (-1.11. -10.91) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 6.24 | 1.37 | 4.87 | 14.65 | 9.77 | 4.88 |
| 9 | 1 | 4 | TDLB100-400 | TDLC300-100 | (-1.11. -10.91) | 2 | 1+1 (2,11) | MCS 20 | MCS16 | 10.20 | 5.28 | 4.92 | 18.60 | 13.68 | 4.92 |
| 10 | 1 | 8 | TDLB100-400 | TDLC300-100 | (-1.11. -10.91) | 1 | 1+1 (2,11) | MCS 2 | MCS16 | -7.42 | -14.42 | 7.00 | -0.95 | -7.95 | 7.00 |
| 11 | 1 | 8 | TDLB100-400 | TDLC300-100 | (-1.11. -10.91) | 1 | 1+1 (2,11) | MCS 16 | MCS16 | 3.42 | -1.83 | 5.25 | 9.88 | 4.64 | 5.24 |
| 12 | 1 | 8 | TDLB100-400 | TDLC300-100 | (-1.11. -10.91) | 1 | 1+1 (2,11) | MCS 20 | MCS16 | 6.74 | 2.02 | 4.72 | 13.20 | 8.49 | 4.71 |
| 13 | 1 | 8 | TDLB100-400 | TDLC300-100 | (-1.11. -10.91) | 2 | 1+1 (2,11) | MCS 2 | MCS16 | -7.41 | -15.40 | 7.99 | 0.99 | -6.99 | 7.98 |
| 14 | 1 | 8 | TDLB100-400 | TDLC300-100 | (-1.11. -10.91) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 3.41 | -2.91 | 6.32 | 11.82 | 5.49 | 6.33 |
| 15 | 1 | 8 | TDLB100-400 | TDLC300-100 | (-1.11. -10.91) | 2 | 1+1 (2,11) | MCS 20 | MCS16 | 6.77 | 1.07 | 5.70 | 15.17 | 9.47 | 5.70 |

Table 4.  PUSCH performance with HetNet scenario, channel model for target cell is TDLA30-10

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (DIP1, DIP2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SINR@70% (MMSE) | SINR@70% (MMSE-IRC) | Gain (SINR) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 2 | TDLA30-10 | TDLC300-100 | (-0.43. -13.78) | 1 | 1+1 (2,11) | MCS 2 | MCS16 | -4.9 | -10.19 | 5.29 | 5.36 | 0.07 | 5.29 |
| 2 | 1 | 2 | TDLA30-10 | TDLC300-100 | (-0.43. -13.78) | 1 | 1+1 (2,11) | MCS 16 | MCS16 | 7.19 | 2.14 | 5.05 | 17.44 | 12.4 | 5.04 |
| 3 | 1 | 2 | TDLA30-10 | TDLC300-100 | (-0.43. -13.78) | 1 | 1+1 (2,11) | MCS 20 | MCS16 | 10.2 | 5.12 | 5.08 | 20.46 | 15.38 | 5.08 |
| 4 | 1 | 4 | TDLA30-10 | TDLC300-100 | (-0.43. -13.78) | 1 | 1+1 (2,11) | MCS 2 | MCS16 | -7.78 | -15.79 | 8.01 | 2.48 | -5.54 | 8.02 |
| 5 | 1 | 4 | TDLA30-10 | TDLC300-100 | (-0.43. -13.78) | 1 | 1+1 (2,11) | MCS 16 | MCS16 | 3.75 | -3.87 | 7.62 | 14.01 | 6.39 | 7.62 |
| 6 | 1 | 4 | TDLA30-10 | TDLC300-100 | (-0.43. -13.78) | 1 | 1+1 (2,11) | MCS 20 | MCS16 | 6.92 | -0.69 | 7.61 | 17.18 | 9.56 | 7.62 |
| 7 | 1 | 4 | TDLA30-10 | TDLC300-100 | (-0.43. -13.78) | 2 | 1+1 (2,11) | MCS 2 | MCS16 | -7.77 | -16.66 | 8.89 | 5.04 | -3.85 | 8.89 |
| 8 | 1 | 4 | TDLA30-10 | TDLC300-100 | (-0.43. -13.78) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 3.73 | -4.65 | 8.38 | 16.54 | 8.16 | 8.38 |
| 9 | 1 | 4 | TDLA30-10 | TDLC300-100 | (-0.43. -13.78) | 2 | 1+1 (2,11) | MCS 20 | MCS16 | 6.91 | -1.49 | 8.4 | 19.71 | 11.32 | 8.39 |
| 10 | 1 | 8 | TDLA30-10 | TDLC300-100 | (-0.43. -13.78) | 1 | 1+1 (2,11) | MCS 2 | MCS16 | -9.65 | -19.13 | 9.48 | 0.6 | -8.87 | 9.47 |
| 11 | 1 | 8 | TDLA30-10 | TDLC300-100 | (-0.43. -13.78) | 1 | 1+1 (2,11) | MCS 16 | MCS16 | 0.92 | -7.27 | 8.19 | 11.17 | 2.98 | 8.19 |
| 12 | 1 | 8 | TDLA30-10 | TDLC300-100 | (-0.43. -13.78) | 1 | 1+1 (2,11) | MCS 20 | MCS16 | 4.07 | -4.18 | 8.25 | 14.33 | 6.08 | 8.25 |
| 13 | 1 | 8 | TDLA30-10 | TDLC300-100 | (-0.43. -13.78) | 2 | 1+1 (2,11) | MCS 2 | MCS16 | -9.65 | -20.53 | 10.88 | 3.15 | -7.72 | 10.87 |
| 14 | 1 | 8 | TDLA30-10 | TDLC300-100 | (-0.43. -13.78) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 0.89 | -8.76 | 9.65 | 13.7 | 4.05 | 9.65 |
| 15 | 1 | 8 | TDLA30-10 | TDLC300-100 | (-0.43. -13.78) | 2 | 1+1 (2,11) | MCS 20 | MCS16 | 4.08 | -5.68 | 9.76 | 16.88 | 7.13 | 9.75 |

Table 5.  PUSCH performance with HetNet scenario, channel model for target cell is TDLC300-100

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (DIP1, DIP2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SINR@70% (MMSE) | SINR@70% (MMSE-IRC) | Gain (SINR) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 2 | TDLA30-10 | TDLC300-100 | (-0.43. -13.78) | 1 | 1+1 (2,11) | MCS 2 | MCS16 | -4.61 | -10.08 | 5.47 | 5.65 | 0.17 | 5.48 |
| 2 | 1 | 2 | TDLA30-10 | TDLC300-100 | (-0.43. -13.78) | 1 | 1+1 (2,11) | MCS 16 | MCS16 | 8.47 | 3.54 | 4.93 | 18.72 | 13.79 | 4.93 |
| 3 | 1 | 2 | TDLA30-10 | TDLC300-100 | (-0.43. -13.78) | 1 | 1+1 (2,11) | MCS 20 | MCS16 | 11.71 | 6.43 | 5.28 | 21.97 | 16.69 | 5.28 |
| 4 | 1 | 4 | TDLA30-10 | TDLC300-100 | (-0.43. -13.78) | 1 | 1+1 (2,11) | MCS 2 | MCS16 | -6.14 | -14.83 | 8.69 | 4.11 | -4.58 | 8.69 |
| 5 | 1 | 4 | TDLA30-10 | TDLC300-100 | (-0.43. -13.78) | 1 | 1+1 (2,11) | MCS 16 | MCS16 | 5.33 | -2.32 | 7.65 | 15.58 | 7.94 | 7.64 |
| 6 | 1 | 4 | TDLA30-10 | TDLC300-100 | (-0.43. -13.78) | 1 | 1+1 (2,11) | MCS 20 | MCS16 | 8.20 | 0.91 | 7.29 | 18.45 | 11.17 | 7.28 |
| 7 | 1 | 4 | TDLA30-10 | TDLC300-100 | (-0.43. -13.78) | 2 | 1+1 (2,11) | MCS 2 | MCS16 | -6.15 | -15.62 | 9.47 | 6.66 | -2.81 | 9.47 |
| 8 | 1 | 4 | TDLA30-10 | TDLC300-100 | (-0.43. -13.78) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 5.34 | -3.06 | 8.40 | 18.15 | 9.75 | 8.40 |
| 9 | 1 | 4 | TDLA30-10 | TDLC300-100 | (-0.43. -13.78) | 2 | 1+1 (2,11) | MCS 20 | MCS16 | 8.23 | 0.35 | 7.88 | 21.04 | 13.16 | 7.88 |
| 10 | 1 | 8 | TDLA30-10 | TDLC300-100 | (-0.43. -13.78) | 1 | 1+1 (2,11) | MCS 2 | MCS16 | -7.08 | -17.81 | 10.73 | 3.18 | -7.56 | 10.74 |
| 11 | 1 | 8 | TDLA30-10 | TDLC300-100 | (-0.43. -13.78) | 1 | 1+1 (2,11) | MCS 16 | MCS16 | 3.01 | -6.16 | 9.17 | 13.26 | 4.09 | 9.17 |
| 12 | 1 | 8 | TDLA30-10 | TDLC300-100 | (-0.43. -13.78) | 1 | 1+1 (2,11) | MCS 20 | MCS16 | 5.62 | -3.00 | 8.62 | 15.88 | 7.26 | 8.62 |
| 13 | 1 | 8 | TDLA30-10 | TDLC300-100 | (-0.43. -13.78) | 2 | 1+1 (2,11) | MCS 2 | MCS16 | -7.10 | -19.15 | 12.05 | 5.71 | -6.34 | 12.05 |
| 14 | 1 | 8 | TDLA30-10 | TDLC300-100 | (-0.43. -13.78) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 3.03 | -7.48 | 10.51 | 15.84 | 5.33 | 10.51 |
| 15 | 1 | 8 | TDLA30-10 | TDLC300-100 | (-0.43. -13.78) | 2 | 1+1 (2,11) | MCS 20 | MCS16 | 5.63 | -4.46 | 10.09 | 18.44 | 8.35 | 10.09 |

Table 6.  PUSCH performance with HetNet scenario, channel model for target cell is TDLB100-400

| Case 1 | Tx | Rx | Channel model (target cell) | Channel model (interferer cells) | (DIP1, DIP2) | Num of Interferers | DMRS port | MCS (Target cell) | MCS (Interferer cell) | SINR@70% (MMSE) | SINR@70% (MMSE-IRC) | Gain (SINR) | SNR@70% (MMSE) | SNR@70% (MMSE) | Gain (SNR) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 2 | TDLB100-400 | TDLC300-100 | (-0.43. -13.78) | 1 | 1+1 (2,11) | MCS 2 | MCS16 | -4.28 | -9.77 | 5.49 | 5.98 | 0.49 | 5.49 |
| 2 | 1 | 2 | TDLB100-400 | TDLC300-100 | (-0.43. -13.78) | 1 | 1+1 (2,11) | MCS 16 | MCS16 | 10.41 | 5.76 | 4.65 | 20.67 | 16.01 | 4.66 |
| 3 | 1 | 2 | TDLB100-400 | TDLC300-100 | (-0.43. -13.78) | 1 | 1+1 (2,11) | MCS 20 | MCS16 | 16.69 | 12.58 | 4.11 | 26.95 | 22.83 | 4.12 |
| 4 | 1 | 4 | TDLB100-400 | TDLC300-100 | (-0.43. -13.78) | 1 | 1+1 (2,11) | MCS 2 | MCS16 | -6.14 | -14.66 | 8.52 | 4.11 | -4.40 | 8.51 |
| 5 | 1 | 4 | TDLB100-400 | TDLC300-100 | (-0.43. -13.78) | 1 | 1+1 (2,11) | MCS 16 | MCS16 | 6.36 | -1.12 | 7.48 | 16.62 | 9.14 | 7.48 |
| 6 | 1 | 4 | TDLB100-400 | TDLC300-100 | (-0.43. -13.78) | 1 | 1+1 (2,11) | MCS 20 | MCS16 | 10.31 | 2.90 | 7.41 | 20.57 | 13.15 | 7.42 |
| 7 | 1 | 4 | TDLB100-400 | TDLC300-100 | (-0.43. -13.78) | 2 | 1+1 (2,11) | MCS 2 | MCS16 | -6.01 | -15.46 | 9.45 | 6.80 | -2.65 | 9.45 |
| 8 | 1 | 4 | TDLB100-400 | TDLC300-100 | (-0.43. -13.78) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 6.41 | -1.77 | 8.18 | 19.22 | 11.04 | 8.18 |
| 9 | 1 | 4 | TDLB100-400 | TDLC300-100 | (-0.43. -13.78) | 2 | 1+1 (2,11) | MCS 20 | MCS16 | 10.37 | 2.42 | 7.95 | 23.18 | 15.23 | 7.95 |
| 10 | 1 | 8 | TDLB100-400 | TDLC300-100 | (-0.43. -13.78) | 1 | 1+1 (2,11) | MCS 2 | MCS16 | -6.95 | -17.70 | 10.75 | 3.31 | -7.44 | 10.75 |
| 11 | 1 | 8 | TDLB100-400 | TDLC300-100 | (-0.43. -13.78) | 1 | 1+1 (2,11) | MCS 16 | MCS16 | 3.71 | -5.26 | 8.97 | 13.97 | 4.99 | 8.98 |
| 12 | 1 | 8 | TDLB100-400 | TDLC300-100 | (-0.43. -13.78) | 1 | 1+1 (2,11) | MCS 20 | MCS16 | 6.98 | -1.22 | 8.20 | 17.24 | 9.03 | 8.21 |
| 13 | 1 | 8 | TDLB100-400 | TDLC300-100 | (-0.43. -13.78) | 2 | 1+1 (2,11) | MCS 2 | MCS16 | -6.97 | -18.99 | 12.02 | 5.84 | -6.19 | 12.03 |
| 14 | 1 | 8 | TDLB100-400 | TDLC300-100 | (-0.43. -13.78) | 2 | 1+1 (2,11) | MCS 16 | MCS16 | 3.70 | -6.58 | 10.28 | 16.51 | 6.23 | 10.28 |
| 15 | 1 | 8 | TDLB100-400 | TDLC300-100 | (-0.43. -13.78) | 2 | 1+1 (2,11) | MCS 20 | MCS16 | 6.99 | -2.60 | 9.59 | 19.79 | 10.20 | 9.59 |

Based on above initial simulation results, the following observations can be achieved

Observation 2: As number of interferers increasing from 1 to 2, the achieved SNR@70% Throughput is increasing, while the achieved SINR @ Throughput is decreasing

Observation 3:  For the high interference scenario in HetNet DIP set 2, the performance gain over MMSE is more compared with HomNet DIP set 1 scenario.

Observation 4: The performance gain over MMSE receiver is similar in terms of both SNR-based requirement definition and SINR-based requirement definition

Observation 5:  For targeting cell with MCS 20 and TDLB100-400 channel condition, the achieved SNR is higher than 20dB, especially for high interference scenario HetNet DIP set 2 with 2Rx.

Observation 6:  For targeting cell with 4/8Rx antenna configuration, it is feasible to achieve performance gain over MMSE under 2 interferers modeling.

# 5	Conclusion

In this contribution, the view on the BS demodulation requirement with MMSE-IRC receiver. And the initial simulation results are provided to check the performance gain over MMSE based on LTE interference profile.

Proposal 1:  Following the WID, only cover HomNet and HetNet scenarios for requirement with MMSE-IRC receiver, and do not consider the scenario with different TDD patterns

Proposal 2:  1 interferer case for 2Rx antenna and 2 interferers case for 4/8Rx can be considered for specifying PUSCH requirement with MMSE-IRC receiver

Proposal 3: It is feasible to reuse LTE interference power profile for specifying PUSCH requirement with MMSE-IRC receiver.

Proposal 4:  The proposal with adding an artificial interference power into the existing LTE interference profile can be considered to account the impact of HPUE. 7dB can be added directly into the 1st strong interference to check the performance with MMSE-IRC receiver in the worst scenario.

Observation 1:  Both INR-based and DIP-based interference modeling can be transferred to each other.

Proposal 5:  Use INR-based modelling + SINR-based requirement definition criteria and transfer the DIP value into INR value

Proposal 6: Cover the following SCS and TDD patterns for specifying PUSCH requirement with MMSE-IRC

  - 15kHz SCS FDD and TDD with pattern 3D1S1U, S=10D:2G:2U

  - 30kHz SCS TDD with pattern 7D1S2U, S=6D:4G:4U

Proposal 7:

For CP-OFDM waveform, consider 15KHz SCS, 10MHz CBW and 30KHz SCS, 40MHz CBW for initial performance evaluation with MMSE-IRC receiver. Additional channel bandwidth with minimum or maximum for each SCS can be considered for requirement definition, such as 15kHz SCS, 5MHz, 20MHz and 30kHz SCS, 10MHz, 100MHz

  - 15kHz SCS, 10MHz for initial performance evaluation

  - 30KHz SCS, 40MHz for initial performance evaluation

For DFT-s-OFDM waveform, the minimum channel bandwidth can be considered as

  - 15KHz SCS, 5MHz

  - 30KHz SCS, 10MHz

Proposal 8:  For target PUSCH and interference PUSCH, prioritize the 1Tx UE scenario. Not to cover 2Tx UE scenario, and 4Tx UE scenario.

Proposal 9: FFS to differentiate the channel model used for target UE and interfered UE for performance evaluation with MMSE-IRC receiver. Reusing the existing channel models to specify the PUSCH requirement with MMSE-IRC receiver

The following candidate propagation channel can be considered as initial evaluation

  - Targeting cell:  TDLA30-10, TDLC300-100 and TDLB100-400

  - Interference cell: TDLC300-100

Proposal 10: Consider MCS 16 for interfered UE, MCS 2, MCS 16 and MCS 20 as candidate for initial performance evaluation with MMSE-IRC receiver.

  - Targeting cell:  MCS 2, MCS 16 and MCS 20

  - Interference cell: MCS 16

For requirement definition, FFS to down selection one MCS for requirement.

Proposal 11: As for starting point to performance evaluation with MMSE-IRC receiver, CP-OFDM waveform can be considered. For requirement definition, both CP-OFDM and DFT-s-OFDM waveforms could be considered to guarantee the test coverage

Proposal 12: Considering both PUSCH mapping type A and mapping type B for specifying PUSCH requirement with MMSE-IRC receiver, reusing the existing Rel-15 test applicability rule to test.

Proposal 13: Use the 70% of maximum throughput as metric for NR BS MMSE-IRC receiver to derive the SINR value

Proposal 14: Considering the following for DMRS configuration and cell ID to specify PUSCH requirement with MMSE-IRC receiver

|  | Targeting cell | Interferer 1 | Interferer 2 |
| --- | --- | --- | --- |
| DM-RS port | 0 | 0 | 0 |
| Cell ID | 0 | 1 | 2 |
| DM-RS sequence generation | NIDnSCIDλ=NIDcell nSCIDλ=0 λ=0 Group hopping and sequence hopping are disabled |  |  |

Observation 2: As number of interferers increasing from 1 to 2, the achieved SNR@70% Throughput is increasing, while the achieved SINR @ Throughput is decreasing

Observation 3:  For the high interference scenario in HetNet DIP set 2, the performance gain over MMSE is more compared with HomNet DIP set 1 scenario.

Observation 4: The performance gain over MMSE receiver is similar in terms of both SNR-based requirement definition and SINR-based requirement definition

Observation 5:  For targeting cell with MCS 20 and TDLB100-400 channel condition, the achieved SNR is higher than 20dB, especially for high interference scenario HetNet DIP set 2 with 2Rx.

Observation 6:  For targeting cell with 4/8Rx antenna configuration, it is feasible to achieve performance gain over MMSE under 2 interferers modeling.

# References

R4-2413570, Way Forward for [112][328] NR_demod_Ph5: Phase 5