3GPP TSG-RAN WG4 Meeting #112bis            	R4-2415984

HeFei, China, 14th-18th, Oct, 2024

Agenda item:	6.8.4

Source:	Samsung

Title:	Discussion on NTN testing NGSO channel model

Document for:	Discussion

- Introduction

In the last RAN4 meeting, the scope and workplan of NTN testing for NGSO channel model was under discussion, the related agreements are captured in the WF [1] as following

- General - Work plan on channel model and requirements - Focus on TE-emulated channel model in initial stage for NGSO testing - Once sufficient progress made for channel model introduction, RAN4 can start the discussion on corresponding requirements and tests according to WI objective - Relation-ship between TE-emulated channel model for satellite motion and fading channel model - Decouple TE-emulated time-varying Doppler (frequency) and delay shifts due to satellite mobility from traditional fading channel - TE emulated channel model for satellite mobility - Methodology for Time varying Doppler and Delay shifts modelling - RAN4 aims to introduce TE-emulated time varying doppler shift and delay shift model for satellite motion with following candidate options - Option 1: Eckstein-Hechler model - Option 2: Keplerian model - Option 3: Existing model from TR 38.811 5.3.4.3 - Option 4: Proposal 2 from R4-2412552 dht=modvNGSO⋅t, 2dh,max-dh,max Tt=dtc=dh(t)2+hNGSO2c Ft=-fddhtd(t)=-fddh(t)dht2+hNGSO2 where - Tt: Propagation delay of the signal from the satellite at time t, - F(t): Doppler (frequency) shift of the signal from the satellite at time t. - dh(t): the horizontal distance between the satellite and UE at time t, - d(t): the distance between the satellite and UE at time t, - dh,max: the maximum horizontal distance between the satellite and UE, i.e., hNGSO/tan⁡(E), - c: the speed of light, - fd: Maximum Doppler shift (Hz). - Option 5: Proposal 2 from R4-2412783 - The instant satellite speed v is given as: v=μR+h - The initial angle β of vector geocentric to UE relative to the x axis is given as: β=π-α-arcsin⁡R∙cosαR+h - The satellite position s is given as: s=j∙R+h - The UE position u is given as: u=R∙ej∙β+v∙tR+h - The Doppler fd is given as: fd=vc∙fc∙realu-su-s - The delay T is given as: T=¦u-s¦c where μ=398600.4418×109m3s2 is standard gravitational parameter, R=6371008.7714m is earth average radius, h=600000m is satellite altitude for LEO-600, α is initial angle of vector UE to satellite relative to vector UE to clockwise side horizon, t is the time, c=299792458ms is the speed of light, fc=2×109Hz is the carrier frequency.. - Other options are not precluded - Note: Companies are encouraged to bring more information for option 1 and option 2 - Predefined files which generated by GMAT can be considered as reference for the comparison purpose of proposed channel model - FFS on the initial ephemeris information and parameters - Parameters for TE-emulated channel model - RAN4 further discuss the necessary parameters with associated channel model, following parameters list for information purpose: - Statellite altitude - Option 1: LEO-600km only - Option 2: Both LEO-600km and LEO-1200km - Elevation angle range - UE position - Whether to consider a non-zero time-varying feeder link delay - Other parameters not precluded - Test procedure/Configuration - During test, ephemeris information shall be updated periodically and aligned with TE-emulated channel model - RAN4 requirements and tests - Demodulation requirements - RAN4 further discuss how to apply the varying Doppler and delay shifts to the UE performance requirements and necessary simulation assumption on UE side pending on channel model discussion progress - UE assumption on propagator model for satellite motion for RAN4 simulation purpose - Option 1: Keplerian model - Other options not precluded - RAN4 further discuss test applicability if new PDSCH demodulation requirements introduced pending on channel model discussion progress

In this contribution, the view on the TE emulated channel model for satellite mobility is provided.

# 2	Discussion

Methodology for Time varying Doppler and Delay Shifts modeling

In the last meeting, serval candidate options are listed for TE emulated channel model discussion.

The purpose of specifying the TE-emulated channel model is tried to model satellite motion trajectory with time varied doppler shift and time shift and also to verify UE behavior of tracking/compensating frequency shift and time shift due to NGSO mobility. From RAN4 demodulation requirement perspective, it is more proper to derive the doppler and delay at every instance mathematically based on a reference model.

For option 4, the TE-emulated channel model was simplified as

dht=modvNGSO⋅t, 2dh,max-dh,max Tt=dtc=dh(t)2+hNGSO2c Ft=-fddhtd(t)=-fddh(t)dht2+hNGSO2 where - Tt: Propagation delay of the signal from the satellite at time t, - F(t): Doppler (frequency) shift of the signal from the satellite at time t. - dh(t): the horizontal distance between the satellite and UE at time t, - d(t): the distance between the satellite and UE at time t, - dh,max: the maximum horizontal distance between the satellite and UE, i.e., hNGSO/tan⁡(E), - c: the speed of light, - fd: Maximum Doppler shift (Hz)

The simplified channel model is to apply the similar methodology of HST scenario for NTN scenario, considering the assumption that satellite is moving in the horizontal direction, which may not be a realistic assumption for NTN scenario.

Observation 1: The channel model in Option 4 is to apply the similar methodology of HST scenario, considering the assumption the satellite is moving in the horizontal direction.

For option 5, the TE-emulated channel model was simplified with assuming satellite position is fixed and UE is moving relative to the satellite, where satellite orbit is circular orbit.

Observation 2: The channel model in Option 5 was simplified with assuming satellite position is fixed and UE is moving relative to the satellite, where satellite orbit is circular orbit

For option 3, it was based on existing model from TR 38.811 5.3.4.3, the doppler shift can be calculated as following procedure, with assuming satellite "S" on its circular orbit

Figure 1: System Geometry for Doppler computation

The doppler shift Fd is calculated as

Fd=Fdc×V×cosθ=Fdc×V×sinu1+γ2-2cosu

Where

- The satellite is at an altitude h, and R is the earth radius.

- α is the elevation angle to the satellite of the UT in M.

- θ is the angle between satellite velocity and SM

- u is the angle between OM and OS

  - Angle u is varying with satellite motion: ut=V*t /(R+h)

- γ can be computed as

γ=R+hR

In general, option 3 and option 5 are similar, assuming satellite orbit is circular orbit, where option 5 is assuming satellite is moving, which may be more practical to model satellite motion trajectory. Therefore, we suggest to option 3 can be considered as starting point.

Proposal 1: Existing channel model from TR 38.811 5.3.4 can be considered as starting point

Parameters for TE-emulated channel model

To model satellite motion trajectory with time varied doppler shift and time shift, the related satellite parameters should be further discussed and down selected. For FR1 NTN scenario, the satellite types with LEO-600 and LEO-1200 are more typical deployments.

In the previous Rel-17 NR NTN and Rel-18 IoT over NTN, it was agreed that there is no time-varying propagation delay modeling, and assuming UE compensation functionality will be covered by other requirements. Therefore, after UE frequency and timing pre-compensation, there is minor different between LEO-600 and LEO-1200. So, only LEO-600 scenario was considered for performance requirement definition.

While for modeling the satellite motion trajectory, it was better to include both LEO-600 and LEO-1200 satellite types, considering the different propagation delay and doppler shift.  Assuming regenerative satellite, the propagation delays for different NGSO satellites can be summarized under different evaluation angle of UE as following table

Table 1: propagation delay for different NGSO satellites under different evaluation angle

|  |  | LEO-600 |  | LEO-1200 |  |
| --- | --- | --- | --- | --- | --- |
| Elevation angle | Path | Distance D (km) | Delay (ms) | Distance D (km) | Delay (ms) |
| 10° | Satellite-UE-Satellite | 3864 | 12.89 | 6263 | 20.89 |
| 20° |  | 2785 | 9.29 | 4911 | 16.38 |
| 30° |  | 2150 | 7.17 | 3999 | 13.34 |
| 40° |  | 1766 | 5.89 | 3385 | 11.29 |
| 50° |  | 1523 | 5.08 | 2974 | 9.92 |
| 60° |  | 1367 | 4.56 | 2701 | 9.01 |
| 70° |  | 1271 | 4.24 | 2527 | 8.43 |
| 80° |  | 1217 | 4.06 | 2431 | 8.11 |
| 90° |  | 1200 | 4.00 | 2400 | 8.00 |

Observation 3: The range of propagation delay will be from 4ms (lowest value for LEO orbit 600km) to 12.89ms (the Elevation angle 10°), and the range of propagation delay will be from 8ms (lowest value for LEO orbit 1200km), with assuming the LEO on a circular orbit

As Doppler shift, based on existing channel model from TR 38.811, 5.3.4, LEO-600 has the largest doppler spread among all the LEO types as following figure

Figure 2: 2 GHz signal at 600 km on D/L and U/L: fixed UE and UE in motion

Observation 4: LEO-600 has the largest doppler shift compared with LEO-1200

Therefore, for the satellite types, both LE0-600 and LEO-1200 can be considered for TE-emulated time varying doppler shift and delay shift modeling, considering testing coverage from propagation delay and doppler shift, prospectively.

Meanwhile, in current test condition for NTN, for range of selected constant delay shift, both NGSO with an altitude of 600km and 1200km on a circular orbit are considered.

Proposal 2: Consider satellite types both LEO-600 and LEO-1200 for TE-emulated time varying doppler shift and delay shift modeling for satellite motion trajectory

Elevation angle range

Regarding the evaluation angle, it will impact on the delay trajectory and doppler trajectory visible range.  With small evaluation angle, the worst-case scenario can be covered, while it requires long testing time to cover the visible satellite coverage range. The testing time should be considered. With larger evaluation angle, the test coverage time can be reduced, where the doppler shift and delay shift are relatively small.

Therefore, the proper evaluation angle should be selected considering the test time and satellite motion trajectory.

In Rel-17 NR NTN channel model discussion, the evaluation angle with 30o was selected for NTN-TDLA channel case, and the evaluation angle with 30o was selected for NTN-TDLC channel case. To align with Rel-17 NTN channel model discussion, evaluation angle with 30o and 90o can be considered, respectively

Regarding the testing time, since the channel model will also be applied for IoT NTN, due to the larger number of repetition transmission, the large number of testing time is required. In current IoT NTN test, the maximum number of repetition transmission is configured as 128. Therefore, at least 128s is needed considering to derive the converged requirement for IoT NTN testing.

Proposal 3: To align with Rel-17 NTN channel model discussion, the elevation angle with 30o and 90o can be considered for testing. Large testing period is required for IoT NTN testing.

UE position

To model satellite motion trajectory with time varied doppler shift and time shift, we can assume UE is in a fixed position without mobility. In RAN5 conformance test, the position of UE is assumed in a fixed position on Earth as following

## 4.13 UE position requirements for NTN testing Clauses 4.14 and 8.1.5B define the common test environment which applies to all test cases executed for eMTC and NB-IoT NTN UEs, unless otherwise specified. - UE's positioning engine (e.g., standalone GNSS receiver) shall be enabled to allow it to acquire the position. UE position during the test is stationary. Below position shall be used to pre-configure UE position: - Latitude: 25.08439333 - Longitude: 121.56076999 - Altitude: 0

Then, based on satellite motion trajectory and UE position, the doppler shift and delay shift can be derived.

Therefore, to simplify the test, the existing UE position specified in RAN5 conformance test can be considered as a starting point.

Proposal 4: To simplify the test, the existing UE position specified in RAN5 conformance test can be considered as a starting point

FFS on Non-zero time-varying feeder link delay

To accommodate the propagation delay in NTNs, serval timing relationships are enhanced by a Common Timing advance (Common TA) and two scheduling offsets Koffset and kmac. Common Timing is a configured offset that corresponds to the Round-Trip Time (RTT) between the Reference Point (RP) and the NTN payload. And  Koffset   is a configured scheduling offset that approximately corresponds to the sum of the service link RTT and the common TA.  In current demodulation requirement,  Koffset was set to closer the service link RTT, in FR1, 8 slots with 15kHz SCS was set, and the common TA was assuming as zero. And the time and doppler condition under testing is only related to the satellite motion for DL in service Link. For feeder link, there is no related the time-varying and frequency shift model. Considering the management of delay or Doppler shift experienced over the feeder link is left to the network implementation, we do not think it was necessary to model non-zero time-varying feeder link delay.  Therefore, we propose to not consider a non-zero time-varying feeder link delay as part of the new channel model.

Proposal 5: Not to consider a non-zero time-varying feeder link delay as part of the new channel model.

UE assumption on propagator model for satellite motion for RAN4 simulation purpose

As agreed in the last meeting, for NTN testing, during test, ephemeris information shall be updated periodically and aligned with TE-emulated channel model. Based on ephemeris information and UE location, UE will perform the time-varying time/frequency shift pre-compensation. In order to facilitate future position and velocity of a satellite prediction performed by UE, the reference model may be needed for the purpose of performance requirement definition and alignment of the performance assessment across companies. In the last meeting, two kinds of reference models were proposed, one is the Eckstein-Hechler model, another is the Keplerian model.

For Keplerian model, it is based on Keplerian-only motion, and depends only on µ. It is more simple model, accounting only for central attraction of the main body, and the main asset of this model is that it can be used for elliptical and hyperbolic orbits. For Eckstein-Hechler model takes into account the effects of the zonal harmonic up to J6 (no tesseral term is considered), it is adapted to nearly circular orbits (eccentricity less than 0.05 and at most 0.1) and does not work for inclinations close to the critical inclinations. The mean elements include secular and long-period effort.

Although both of them are widely used analytical propagator models with lower implementation complexity compared with generic numerical propagators, it was not clear whether they can be transferred into a mathematical model, which are used for TE and UE vendors implementation. The proponents of these two models are encouraged to provide more details about how these two models are generated.

The purpose of channel model is tried to model satellite motion trajectory with time varied doppler shift and time shift and also to verify UE behavior of tracking/compensating frequency shift and time shift due to NGSO mobility. From RAN4 demodulation requirement perspective, it is more proper to derive the doppler and delay at every instance mathematically based on a reference model

Observation 5: Both Eckstein-Hechler model and Keplerian model are widely used analytical propagator models with lower implementation complexity compared with generic numerical propagators, it was not clear whether they can be transferred into a mathematical model, which are used for TE and UE vendors implementation

# 3	Conclusion

In this contribution, the view on the NTN testing NGSO channel model is provided

Observation 1: The channel model in Option 4 is to apply the similar methodology of HST scenario, considering the assumption the satellite is moving in the horizontal direction.

Observation 2: The channel model in Option 5 was simplified with assuming satellite position is fixed and UE is moving relative to the satellite, where satellite orbit is circular orbit

Proposal 1: Existing channel model from TR 38.811 5.3.4 can be considered as starting point

Observation 3: The range of propagation delay will be from 4ms (lowest value for LEO orbit 600km) to 12.89ms (the Elevation angle 10°), and the range of propagation delay will be from 8ms (lowest value for LEO orbit 1200km), with assuming the LEO on a circular orbit

Observation 4: LEO-600 has the largest doppler shift compared with LEO-1200

Proposal 2: Consider satellite types both LEO-600 and LEO-1200 for TE-emulated time varying doppler shift and delay shift modeling for satellite motion trajectory

Proposal 3: To align with Rel-17 NTN channel model discussion, the elevation angle with 30o and 90o can be considered for testing. Large testing period is required for IoT NTN testing.

Proposal 4: To simplify the test, the existing UE position specified in RAN5 conformance test can be considered as a starting point

Proposal 5: Not to consider a non-zero time-varying feeder link delay as part of the new channel model.

Observation 5: Both Eckstein-Hechler model and Keplerian model are widely used analytical propagator models with lower implementation complexity compared with generic numerical propagators, it was not clear whether they can be transferred into a mathematical model, which are used for TE and UE vendors implementation

# References

R4-2413516, Way Forward for [112][329] NTN_testing_NGSO_channel_model