3GPP TSG-RAN WG4 Meeting #112bis									                     R4- 2415985

Hefei, China, Oct. 14 – 18, 2024

Agenda item:			6.10.2.1

Source:	Ericsson

Title:	Discussion on RF requirement of ATG UE intra-band contiguous CA

Document for:	Approval

# Introduction

In 3GPP release 18, RAN4 has completed the standardization work for the air-to-ground (ATG) network WI, the main work focused on the FR1 single carrier scenario. To enhance the throughput to meet growing ATG capacity demand and fully utilize operators’ spectrum, the RAN plenary meeting #103 has approved a new WI on enhancements for the air-to-ground network for NR [1].

# Discussion

In the last meeting, some agreements were made addressing the general concerns and intra-band contiguous CA [2]. Part of the agreements regarding the intra-band contiguous CA are shown below:

Issue 2-1-2: whether/how to reflect operation band information in spec

Agreement:

- BCS 4 and 5 also needs to be specified besides BCS0.

Table 1: BCS for CA_n79C

| NR CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configurations or single uplink carrier5 | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Channel bandwidths for carrier (MHz) | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
| CA_n79C | CA_n79C | 50 | 60, 80, 100 |  |  |  | 200 | 0 |
|  |  | 60 | 60, 80, 100 |  |  |  |  |  |
|  |  | 80 | 80, 100 |  |  |  |  |  |
|  |  | 100 | 100 |  |  |  |  |  |
|  |  | See n79 channel bandwidths in Table 5.3.5-1 for each carrier2 |  |  |  |  | 200 | 4 and 5 |

Issue 2-2: ACS testing parameters for case 2

Agreement:

- Further check the following table.

- Table 3: Test parameters for intra-band contiguous CA with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz, case 2

| Rx Parameter | Units | NR CA bandwidth class |
| --- | --- | --- |
|  |  | C |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | -73.54 -61.55 |
| PInterferer | dBm | -424 -305 |
| BWInterferer | MHz | BWchannel CA |
| FInterferer (offset) | MHz | BWchannel CA / -BWchannel CA |
| NOTE 1: The transmitter shall be set to 24 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the carrier closest to the interferer in MHz. The interferer is an NR signal with an SCS equal to that of the closest carrier. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. NOTE 4: Pinterferer shall be set to -42dBm for omni-directional antenna. NOTE 5: Pinterferer shall be set to -30dBm for antenna array. |  |  |

#### Adjacent channel selectivity

ACS test parameters for case 1 need to be specified, it could reuse the tables in 7.5A.1

ACS case 2 needs to be updated. Particularly Power in transmission bandwidth configuration, per CC needs to be scaled according to the ATG UE maximum input level. e.g.

- Table 3: Test parameters for intra-band contiguous CA with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz, case 2

| Rx Parameter | Units | NR CA bandwidth class |
| --- | --- | --- |
|  |  | C |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | -73.54 -61.55 |
| PInterferer | dBm | -424 -305 |
| BWInterferer | MHz | BWchannel CA |
| FInterferer (offset) | MHz | BWchannel CA / -BWchannel CA |
| NOTE 1: The transmitter shall be set to 24 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the carrier closest to the interferer in MHz. The interferer is an NR signal with an SCS equal to that of the closest carrier. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. NOTE 4: Pinterferer shall be set to -42dBm for omni-directional antenna. NOTE 5: Pinterferer shall be set to -30dBm for antenna array. |  |  |

- ACS case 2 needs to be updated. Particularly, Power in the transmission bandwidth configuration per CC needs to be scaled according to the ATG UE maximum input level.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	ACS case 2 needs to be updated. Particularly, Power in the transmission bandwidth configuration per CC needs to be scaled according to the ATG UE maximum input level.

# References

- RP-240804, New WID on Enhancements for Air-to-ground network for NR.

- R4-2414322, WF on Rel-19 ATG UE RF requirements