3GPP TSG-RAN WG4 Meeting #112bis									                     R4- 2415987

Hefei, China, Oct. 14 – 18, 2024

Agenda item:			6.10.2.3

Source:	Ericsson

Title:	Discussion on RF requirement of ATG UE UL MIMO

Document for:	Approval

# Introduction

In 3GPP release 18, RAN4 has completed the standardization work for the air-to-ground (ATG) network WI, the main work focused on the FR1 single carrier scenario. To enhance the throughput to meet growing ATG capacity demand and fully utilize operators’ spectrum, the RAN plenary meeting #103 has approved a new WI on enhancements for the air-to-ground network for NR [1].

# Discussion

In the last meeting, some agreements were made addressing the general concerns [2]. Part of the agreements regarding the UL MIMO are shown below:

## UL MIMO

Issue 4-1-1: ULFPTx mode for UL-MIMO

Agreement:

- FFS on whether ULFPTx mode for UL-MIMO is supported for ATG UE

Issue 4-1-2: whether preclude the single antenna port related requirement

Agreement:

- FFS on whether to preclude the single antenna port related requirement.

Issue 4-2-1: how to modify the NR UL MIMO requirement with ATG capability antennaArrayType-r18

Agreement:

- If the legacy requirement for UL MIMO is defined at each antenna connector,

  - For ATG UE with antenna array, the requirement should be updated [at each TAB connector].

- If the legacy requirement for UL MIMO is defined per layer, FFS

  - Option 1: for ATG UE with capability antennaArrayType-r18, the requirement should be updated as sum of all TAB connectors per layer.

  - Option 2: for ATG UE with capability antennaArrayType-r18, the requirement should be updated per layer without explicitly emphasize the TAB connector.

Issue 4-2-2: whether to apply coherent UL MIMO requirement for ATG UE

Agreement:

- FFS on whether to apply coherent UL MIMO requirement for ATG UE

Issue 4-3: receiver requirements

Agreement:

- Reuse ATG single carrier requirements for following Rx requirements (ZTE, Ericsson)

  - REFSENSE

  - Max input level

  - ACS

  - Blocking

  - Spurious emission

  - Intermodulation

- FFS whether to test the Rx requirements under the UL MIMO scenario since Rx requirements is the same for R18 ATG UE and R19 ATG UE supporting UL-MIMO.

There are still many open issues that need to be addressed and the following session will discuss the RF requirement for ATG UE UL MIMO.

## RF requirements for ATG UE UL MIMO

### ULFPTx mode for UL-MIMO

For ATG UE UL-MIMO, in the case of one antenna port being configured (single layer UL MIMO operation) but with two Tx antenna connectors, the uplink full power transmission (ULFPTx) mode is applicable. From Rel-16 onwards, with the support of ULFPTx mode, one can transmit at full power either by combining two connectors using a coherent precoder or by supporting a full-power PA in full power mode. This is to ensure that UE can transmit at full power, i.e., if the UE is configured with only a single antenna port, it should always transmit at full power.

- ULFPTx mode should be supported for ATG UL-MIMO

As stated in the previous session, The ATG UE UL-MIMO is possible to be scheduled for a single antenna-port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 for codebook-based transmission with precoding matrix W=1. Therefore, from the completeness perspective, the single antenna requirement still needs to be specified for ATG UE UL MIMO.

### Coherent UL MIMO requirement for ATG UE

FFS on whether to apply coherent UL MIMO requirement for ATG UE

UE capabilities have been defined for full coherence, partial coherence, and non-coherence transmission since Rel-15 NR. These modes correspond to where all transmit chains, partial transmit chains or none of the transmit chains have sufficiently well controlled relative phase for the codebook-based operation. A UE supporting fully coherent transmission can also support partial and non-coherent transmission.

As coherent UL MIMO is an optional feature for the UE, it is generally desirable to allow as much implementation freedom as possible. Therefore, coherent UL MIMO requirement should be specified for ATG UE UL MIMO as well.

- Apply the coherent UL MIMO requirement for ATG UE.

Issue 4-2-1: how to modify the NR UL MIMO requirement with ATG capability antennaArrayType-r18

Agreement:

- If the legacy requirement for UL MIMO is defined at each antenna connector,

  - For ATG UE with antenna array, the requirement should be updated [at each TAB connector].

- If the legacy requirement for UL MIMO is defined per layer, FFS

  - Option 1: for ATG UE with capability antennaArrayType-r18, the requirement should be updated as sum of all TAB connectors per layer.

  - Option 2: for ATG UE with capability antennaArrayType-r18, the requirement should be updated per layer without explicitly emphasize the TAB connector.

# Conclusion

In the previous sections, we made the following observations:

Based on the discussion in the previous sections we propose the following:

Proposal 1	ULFPTx mode should be supported for ATG UL-MIMO

Proposal 2	Apply the coherent UL MIMO requirement for ATG UE.

# References

- RP-240804, New WID on Enhancements for Air-to-ground network for NR.

- R4-2414322, WF on Rel-19 ATG UE RF requirements

- TS. 38.822, NR UE feature list