3GPP TSG-RAN WG4 Meeting #112bis	R4-2415996

# Hefei, China, 14th – 18th October, 2024

Agenda item:	6.1.1.3.1

Source:            	Google

Title:                	Discussion on REFSENS requirement for 6Rx UE

Document for:	Approval

# Introduction

In RAN#103, a new WID on RF enhancements for NR FR1/FR2 and EN-DC was approved as one of Release 19 work items [1]. One of the objectives in the WID is to discuss 6Rx requirements for handheld and FWA UEs, which is shown as follows. In this contribution, we will focus on the REFSENS requirements for 6Rx handheld and FWA UE.

6Rx for handheld and FWA UE - Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario - Example bands: n41, n77/n78, n79, n104 - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers - Specify the Rx requirements including reference sensitivity requirements for support 6Rx - Note: the specified requirements can be applicable to both handheld UE and FWA devices - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, t4r6 depending on UE capability - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution.

In RAN4#112, the 6Rx REFSENS requirement for handheld and FWA UEs has been discussed and some agreements and open issue are captured in the approved WF [2]. In WF, the remaining open issue is about how to verify handheld and FWA UEs on single carrier RF requirement for ΔRIB,6R and other 6Rx receiver requirements. In this paper, we would like to share our view in the following.

# Topic 1: REFSENS (delta RIB,6R) ## Sub-topic 1-1: General considerations for specifying ΔRIB,6R value Issue 1-1-3: Verification of 6Rx receiver requirements Way Forward: Companies encouraged to provide their views on the ZTE proposal in Proposal 5 in R4-2411883 and any further positions on verification of 6Rx receiver requirements at future meetings given that ΔRIB,6R values are now defined.

# Discussion

In 3GPP technology, multiple antennas technique is a key feature to enhance system data DL throughput. For the FR1 RF requirement with multiple Rx antennas, the REFSENS is the major requirement which is defined in TS 38.101-1 sub-clause 7.3. In sub-clause 7.3, the REFSENS requirements with 2Rx antenna ports for each applicable operating band are defined as the baseline, and the additional multiple antenna ΔRIB requirements are introduced on the top of the 2Rx REFSENS requirements. However, in the current specification, only 4Rx requirement has been introduced for handheld UE and FWA UE from Rel-15 and 8Rx was introduced only for FWA UE in Rel-18. Hence, the ΔRIB,6R requirements for 6Rx handheld UE and FWA UE is approved for further study in Rel-19 work item.

In RAN4#112 WF [2], the ΔRIB,6R requirement are agreed to be introduced with different values for the handheld UE and FWA UE. Besides, it is also agreed to adopt the average value of ΔRIB,6R for handheld UE and FWA UE from all company proposals. However, one issue is raised by the company that how to verify the RF testing requirement for 6Rx UE needs to be further discussed.

Table 2: diversity characteristics for 2Rx/4Rx/8Rx [4]

|  | Single carrier REFSEN requirement | Other single carrier Rx requirements |
| --- | --- | --- |
| UE is equipped with 2Rx | 2Rx | 2Rx |
| UE is equipped with 4Rx | 2Rx, 4Rx | 4Rx, skip 2Rx |
| UE is equipped with 8Rx | 2Rx, 4Rx, 8Rx | 8Rx, skip both 2Rx and 4Rx |

In Table 2 within [4], it summarizes the verification applicability rule for single carrier RF receiver requirement when the UE is equipped with multiple antennas, which is captured in TS 38.101-1 sub-clause 7.2. If a UE is equipped with 2Rx antenna ports, all single carrier RF requirements should be verified for 2Rx antenna ports. However, if a UE is equipped with 4Rx antenna ports, both 2Rx and 4Rx single carrier REFSENS requirement should be verified but other single carrier RF requirement can skip the verification for 4Rx antenna ports, where the skip reason is to save more testing time without impact the overall testing result. For 8Rx, it is similar that 2Rx/4Rx/8Rx single carrier REFSENS requirement should be verified and the verification for other single carrier 2Rx and 4Rx RF requirement can be skipped.

In our view, in order to save testing time for RF receiver requirements, the similar rule can be applied to the UE equipped with 6Rx antenna ports. Hence, it is proposed to adopt the verification rule that if a UE is equipped with 6Rx antenna ports,

- Single carrier REFSENS requirement should be verified on 2Rx/4Rx and 6Rx

- Other single carrier Rx requirements should be verified on 6Rx and 2Rx/4Rx can be skipped

Proposal 1: Considering to save testing time for 6Rx RF receiver requirements, it is proposed to adopt the verification rule that if a UE is equipped with 6Rx antenna ports,

- Single carrier REFSENS requirement should be verified on 2Rx/4Rx/6Rx

- Other single carrier Rx requirements should be verified on 6Rx and 2Rx/4Rx can be skipped

# Conclusion

The proposals in this contribution are summarized in the following.

Proposal 1: Considering to save testing time for 6Rx RF receiver requirements, it is proposed to adopt the verification rule that if a UE is equipped with 6Rx antenna ports,

- Single carrier REFSENS requirement should be verified on 2Rx/4Rx/6Rx

- Other single carrier Rx requirements should be verified on 6Rx and 2Rx/4Rx can be skipped

# Reference

- RP-240828, “New WID: UE RF enhancements for FR1/FR2 and EN-DC, Phase 4”, RAN4 chair (Huawei), RAN#103

- R4-2414285, “WF on 6Rx UE requirements”, AT&T, RAN4#112

- R4-2412925, “Discussion on REFSENS requirement for 6Rx UE”, Google, RAN4#112

- R4-2411883, “Further discussion on 6Rx Reference sensitivity requirements”, ZTE, RAN4#112