3GPP TSG-RAN WG4 Meeting #112-bis														R4-2415999

Hefei, China, 14th – 18th October, 2024

Agenda item:			6.2.4

Source:	Samsung

Title:	TP on UE parameters for 14800 to 15350 MHz frequency range

Document for:	Approval

1	Introduction

Based on the work split discussion, this document is a text proposal to provide a structure with draft UE RF parameters for 14800 – 15350 MHz frequency range that possibly updated in this meeting.

2	Text Proposal

-------------------------------------------- TP BEGIN --------------------------------------------

## 6.4	UE parameters

### 6.4.1	Transmitter characteristics

#### 6.4.1.1	Power dynamic range

The minimum controlled output power of the UE is defined as the power in the channel bandwidth for all transmit bandwidth configurations (resource blocks), when the power is set to a minimum value. Based on the study in sub-clause 6.5.2, the UE antenna type for 14800 – 15350 MHz can reuse existing [FR1/2] characteristics so that both nominal maximum and minimum output power refer to existing [FR1/2] bands, i.e., power dynamic range is [56] dB for [100] MHz channel bandwidth.

#### 6.4.1.2	Spectral mask

#### 6.4.1.3	ACLR

According to the previous studies and simulation results in sub-clause 6.1.3, it was concluded that [xx] dB ACLR would be sufficient for 14800 – 15350 MHz. Thus, ACLR of [xx] dB can be considered for the frequency range 14800 – 15350 MHz.

The actual ACLR for this frequency range or parts there-of should be further studied in the WI phase.

#### 6.4.1.4	Spurious emissions

The general spurious emissions defined in [TS 38.101-1/2] [x] clause 6.5.3.1 can apply to the frequency range 14800 -15350 MHz.

#### 6.4.1.5	Maximum output power

Based on the study in sub-clause 6.5.2, the UE maximum output power for the considered frequency ranges could be [23] dBm. Other UE power classes, [e.g., 20dBm, 26dBm and 29dBm], are not precluded (corresponding ACLR limit will be adapted accordingly to avoid additional interference).

#### 6.4.1.6	Average output power

NOTE:	This parameter was not mentioned in the previous response to ITU-R WP5D.

### 6.4.2	Receiver characteristics

#### 6.4.2.1	Noise figure

A noise figure of 11 dB was assumed for coexistence simulation in sub-clause 6.1.2. For the frequency range 14800 – 15350 MHz noise figure of [11] dB can be reused.

The actual noise figure for this frequency range or parts there-of to define RF requirements should be further studied in the WI phase.

#### 6.4.2.2	Sensitivity

The sensitivity is not a critical parameter for sharing and compatibility studies. It was agreed to not mention any value for this parameter.

#### 6.4.2.3	Blocking response

The blocking characteristic specified in clause 7.6 of [TS 38.101-1/2] [x] could be applied for the frequency range 14800 – 15350 MHz.

The actual requirements for this frequency range or parts there-of may differ depending on the band plan and possible re-use of RF hardware components.

#### 6.4.2.4	ACS

According to the previous studies and simulation results in sub-clause 6.1.3, adjacent channel selectivity (ACS) is agreed as [xx] dBc for 14800 – 15350 MHz.  Thus, ACS of [xx] dB can be considered for the frequency range 14800 – 15350 MHz.

The actual ACS for this frequency range or parts there-of should be further studied in the WI phase.

-------------------------------------------- TP END --------------------------------------------