3GPP TSG-RAN WG4 Meeting #112-bis														R4-2416002

Hefei, China, 14th – 18th October, 2024

Agenda item:			6.2.4.1

Source:	Samsung

Title:	Co-existence simulation results for 14800 – 15350 MHz

Document for:	Discussion

1 Introduction

Under the Study Item to study the IMT parameters relevant for sharing and compatibility for the frequency ranges based on the LS from ITU-R WP 5D [1, 2], for 15 GHz range, RAN4 agreed to conduct coexistence studies to derive interference levels on adjacent channels.

Although it is a new frequency range to study and still has multiple options so far, a set of coexistence study assumptions is captured encouraging companies to provide simulation results in TR 38.922 [3]. In this contribution, we would like to provide some simulation results in the view of ITU-R sharing study impact and our preference among the options focusing on the first priority.

2 Discussion

As mentioned earlier, simulation assumptions for coexistence study have been further refined and captured in [3]. Table 1 shows a summary of initial simulation scenarios agreed for 14800 - 15350 MHz, which are basic assumptions to be considered for the coexistence study.

Table 1: Summary of initial simulation scenarios for 14800 - 15350 MHz [3]

| No. | Usage scenario | Aggressor | Victim | Direction | Simulation frequency | Deployment Scenario | Priority |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | eMBB | NR, 100/200/400 MHz | Same as aggressor | DL to DL | 15 GHz | Indoor hotspot | Second |
| 2 | eMBB | NR, 100/200/400 MHz | Same as aggressor | DL to DL | 15 GHz | Urban macro | First |
| 3 | eMBB | NR, 100/200/400 MHz | Same as aggressor | DL to DL | 15 GHz | Dense urban | Third |
| 4 | eMBB | NR, 100/200/400 MHz | Same as aggressor | UL to UL | 15 GHz | Indoor hotspot | Second |
| 5 | eMBB | NR, 100/200/400 MHz | Same as aggressor | UL to UL | 15 GHz | Urban macro | First |
| 6 | eMBB | NR, 100/200/400 MHz | Same as aggressor | UL to UL | 15 GHz | Dense urban | Third |

On top of the simulation scenarios, other assumptions are also captured in TR 38.922 but some of important parameters are still having multiple options. For example, although BS antenna array and sub-array options have been extensively discussed for both LS [4] and simulation [3], RAN4 could not take a single number even for the simulation work. Table 2 below tries to summarize different options in different documents for BS antenna size showing the difference in terms of the number of antenna elements for the range of 14800 - 15350 MHz.

Table 2: BS antenna array size options

|  | For LS [4] | For simulation [3] |
| --- | --- | --- |
| Option 1 | 3072 AEs dual polarization (16x24 array and 4x1 sub-array) | 3072 AEs dual polarization (16x24 array and 4x1 sub-array) |
| Option 2 | 4096 AEs dual polarization: (8x32 array and 8x1 sub-array) (16x32 array and 4x1 sub-array) | 4096 AEs dual polarization: (8x32 array and 8x1 sub-array) |
| Option 3 | 2048 AEs dual polarization: (8x32 array and 4x1 sub-array) |  |
| Option 4 | 1536 AEs dual polarization: (8x16 and 6x1 sub-array) |  |

Given that the simulation parameters in the TR have one or more options besides BS antenna element size, in this contribution, we would like to look into options having priority for our initial coexistence simulation, then to move on with our preference. For example, overall system impact to legacy services on the frequency range have been considered among the simulation scenarios in Table 1. In addition, as proposed in another contribution [5], the trade-off between performance benefits and coverage/energy costs are taken into account in terms of the feasibility of antenna related parameter options. in Table 2.

Observation 1:	Overall system impact to legacy services on the frequency range should be considered for coexistence study.

Observation 2:	Trade-off between performance benefits and coverage/energy costs are taken into account in terms of the feasibility of antenna related parameter options.

In accordance with observations above, in this contribution, we have conducted the initial simulation work focusing on parameters highlighted in yellow in Table 1 and 2, and with following assumptions in Table 3 and 4.

Table 3: Simulated cases for Urban macro scenario (DL-to-DL)

| Urban Macro (0% and 100% shift) | UE Ant | UE # per cell | UL power | Indoor ratio | BS Ant | ISD | Bandwidth |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | FR1 like | 1 | 23 | 20 | 3072 8x32x6 | 450 | 100MHz |
| 2 | FR1 like | 1 | 23 | 20 | 1536 8x16x6 | 450 | 100MHz |
| 3 | FR1 like | 1 | 23 | 20 | 3072 8x32x6 | 450 | 200MHz |
| 4 | FR1 like | 1 | 23 | 20 | 1536 8x16x6 | 450 | 200MHz |

Table 4: Simulated cases for Indoor hotspot scenario (DL-to-DL)

| Indoor (0% shift) | UE Ant | UE # per cell | UL power | Indoor ratio | BS Ant | ISD | Bandwidth |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | FR1 like | 1 | 23 | N/A | 8*8 | N/A | 100MHz |
| 2 | FR1 like | 1 | 23 | N/A | 4*4 | N/A | 100MHz |
| 3 | FR1 like | 1 | 23 | N/A | 8*8 | N/A | 200MHz |
| 4 | FR1 like | 1 | 23 | N/A | 4*4 | N/A | 200MHz |

2.1	Urban macro

2.1.1	3072 AEs, 100MHz

Table 5: Simulation results for DL throughput loss (%)

| ACIR [dB] | Coordinated (0% grid shift) |  |  |  |  |  | Un-coordinated (100% grid shift) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 | 10 | 15 | 20 | 25 | 30 | 5 | 10 | 15 | 20 | 25 | 30 |
| Average | 4% | 2% | 1% | 0% | 0% | 0% | 4% | 3% | 2% | 1% | 0% | 0% |
| 5%-tile | 30% | 17% | 10% | 5% | 4% | 2% | 34% | 21% | 12% | 8% | 4% | 3% |

| < Coordinated (0% grid shift)> | <Non-collocated> |
| --- | --- |

Figure 1: Simulation results for DL throughput loss (%)

2.1.2	1536 AEs, 100MHz

Table 6: Simulation results for DL throughput loss (%)

| ACIR [dB] | Coordinated (0% grid shift) |  |  |  |  |  | Un-coordinated (100% grid shift) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 | 10 | 15 | 20 | 25 | 30 | 5 | 10 | 15 | 20 | 25 | 30 |
| Average | 6% | 4% | 2% | 1% | 1% | 0% | 7% | 5% | 3% | 2% | 1% | 1% |
| 5%-tile | 37% | 22% | 13% | 7% | 5% | 3% | 40% | 23% | 13% | 6% | 4% | 2% |

| < Coordinated (0% grid shift)> | <Non-collocated> |
| --- | --- |

Figure 2: Simulation results for DL throughput loss (%)

2.1.3	3072 AEs, 200MHz

Table 7: Simulation results for DL throughput loss (%)

| ACIR [dB] | Coordinated (0% grid shift) |  |  |  |  |  | Un-coordinated (100% grid shift) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 | 10 | 15 | 20 | 25 | 30 | 5 | 10 | 15 | 20 | 25 | 30 |
| Average | 4% | 2% | 1% | 1% | 0% | 0% | 4% | 3% | 2% | 1% | 0% | 0% |
| 5%-tile | 28% | 16% | 9% | 5% | 3% | 2% | 34% | 20% | 12% | 7% | 3% | 2% |

| < Coordinated (0% grid shift)> | <Non-collocated> |
| --- | --- |

Figure 3: Simulation results for DL throughput loss (%)

2.1.4	1536 AEs, 200MHz

Table 8: Simulation results for DL throughput loss (%)

| ACIR [dB] | Coordinated (0% grid shift) |  |  |  |  |  | Un-coordinated (100% grid shift) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 | 10 | 15 | 20 | 25 | 30 | 5 | 10 | 15 | 20 | 25 | 30 |
| Average | 6% | 4% | 2% | 1% | 0% | 0% | 7% | 5% | 3% | 2% | 1% | 0% |
| 5%-tile | 37% | 22% | 12% | 7% | 4% | 3% | 40% | 23% | 13% | 6% | 4% | 2% |

| < Coordinated (0% grid shift)> | <Non-collocated> |
| --- | --- |

Figure 4: Simulation results for DL throughput loss (%)

Based on the tables and figures for the simulation results of urban macro scenarios above, it is observed that the throughput loss between two antenna element sizes is comparable enough for both average and edge cases as expected. It is our understanding that such similar performance between two different AEs, e.g., 3072 vs. 1536, would come from the massive antenna pattern assumptions that should be enough at 15 GHz frequency range. Therefore, potential ACIR requirements should be determined with up to 3072 AEs assumption for urban macro scenarios.

Observation 3:	It is observed that the throughput loss between two antenna element sizes is comparable enough for both average and edge cases, e.g., 3072 vs. 1536 AEs, as expected.

Proposal 1:		Potential ACIR requirements should be determined with up to 3072 AEs assumption for urban macro scenarios.

2.2	Indoor hotspot

2.2.1	8x8, 100MHz

Table 9: Simulation results for DL throughput loss (%)

| ACIR [dB] | Coordinated (0% grid shift) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 5 | 10 | 15 | 20 | 25 | 30 |
| Average | 16% | 9% | 0% | 0% | 0% | 0% |
| 5%-tile | 20% | 2% | 0% | 0% | 0% | 0% |

Figure 5: Simulation results for throughput loss (%)

2.2.2	4x4, 100MHz

Table 10: Simulation results for DL throughput loss (%)

| ACIR [dB] | Coordinated (0% grid shift) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 5 | 10 | 15 | 20 | 25 | 30 |
| Average | 20% | 5% | 0% | 0% | 0% | 0% |
| 5%-tile | 27% | 10% | 0% | 0% | 0% | 0% |

Figure 6: Simulation results for DL throughput loss (%)

2.2.3	8x8, 200MHz

Table 11: Simulation results for DL throughput loss (%)

| ACIR [dB] | Coordinated (0% grid shift) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 5 | 10 | 15 | 20 | 25 | 30 |
| Average | 7% | 0% | 0% | 0% | 0% | 0% |
| 5%-tile | 16% | 0% | 0% | 0% | 0% | 0% |

Figure 7: Simulation results for DL throughput loss (%)

2.2.4	4x4, 200MHz

Table 12: Simulation results for DL throughput loss (%)

| ACIR [dB] | Coordinated (0% grid shift) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 5 | 10 | 15 | 20 | 25 | 30 |
| Average | 12% | 0% | 0% | 0% | 0% | 0% |
| 5%-tile | 16% | 0% | 0% | 0% | 0% | 0% |

Figure 1: Simulation results for DL throughput loss (%)

3	Conclusion

This contribution provides our initial results of DL-to-DL co-existence studies for 15 GHz frequency range based on the assumptions under discussion. Following observations and proposals should be considered by this meeting.

Observation 1:	Overall system impact to legacy services on the frequency range should be considered for coexistence study.

Observation 2:	Trade-off between performance benefits and coverage/energy costs are taken into account in terms of the feasibility of antenna related parameter options.

Observation 3:	It is observed that the throughput loss between two antenna element sizes is comparable enough for both average and edge cases, e.g., 3072 vs. 1536 AEs, as expected.

Proposal 1:		Potential ACIR requirements should be determined with up to 3072 AEs assumption for urban macro scenarios.

Reference

[1] R4-2400333	Parameters of terrestrial component of IMT for sharing and compatibility studies in the frequency
 				bands 4 400-4 800 MHz, 7 125-8 400 MHz and 14.8-15.35 GHz

[2]	RP-241326	Study on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz

[3]	3GPP TR 38.922 V0.2.0	(2024-08)

[4] R4-2414283	WF on 15GHz BS parameters

[5] R4-2416001	Views on BS and UE parameters for 14800 to 15350 MHz frequency range